Cisco XRV9000

IOS-XR Release 24.2.2

IOS-XR System Upgrade/Downgrade Procedure

Contents

Purpose, Scope and Audience	3

Obtain Required Package Files	3

Configuration Backup	4

Pre-Upgrade Tasks	4

Software Upgrade	5

Classic Method	6

Post-Upgrade Tasks	7

Classic Method	8

Post-Downgrade Tasks	9

Caveats	10

# Purpose, Scope and Audience

This document provides information on the two methods Classic and GISO methods available for system upgrade and downgrade for XRV9000 Series platforms from software version 7.X.X to 24.2.2 and vice versa.

This document covers NGXR to NGXR upgrade procedure only.

| Upgrade |  |  |
| --- | --- | --- |
| V1 | V2 | Bridge SMU |
| 73x | 2422 | N/A |
| 75x | 2422 | CSCwj08501 |
| 76x | 2422 | CSCwj08501 |
| 77x | 2422 | CSCwj08501 |
| 78x | 2422 | CSCwj08501 |
| 79x | 2422 | N/A |
| 710x | 2422 | N/A |
| 711x | 2422 | N/A |
| 241x | 2422 | N/A |
| 242x | 2422 | N/A |

Note: CSCwj08501 need it on V1 image and is applicable for vRR ESXi and Appliance setup

| Downgrade |  |  |
| --- | --- | --- |
| V1 | V2 | Bridge SMU |
| 2422 | 241x | N/A |
| 2422 | 242x | N/A |

Note :  For Appliance Mode , downgrade from  2422 to 75x /76x /77x /78x /79x /710x/711x/ is not supported.

For ESXi , downgrade from  2422 to 75x /76x /77x /78x is not supported.

Please refer the caveat Section for more details.

# Obtain Required Package Files

Mini ISO Package is mandatory to perform the System Upgrade and upgrade needs to be done from XR VM. Additional XR packages listed below are needed depending on the router configuration and required features:

| Description | Package Name |
| --- | --- |
| Boot Image | xrv9k-mini-x-24.2.2.iso |
| mpls | xrv9k-mpls-1.0.0.0-r2422.x86_64.rpm |
| mpls-rsvp-te | xrv9k-mpls-te-rsvp-1.0.0.0-r2422.x86_64.rpm |
| multicast | xrv9k-mcast-1.0.0.0-r2422.x86_64.rpm |
| ospf | xrv9k-ospf-1.0.0.0-r2422.x86_64.rpm |
| isis | xrv9k-isis-1.0.0.0-r2422.x86_64.rpm |
| Li | xrv9k-li-x-1.0.0.0-r2422.x86_64.rpm |
| Eigrp | xrv9k-eigrp-1.0.0.0-r2422.x86_64.rpm |
| k9sec | xrv9k-k9sec-1.0.0.0-r2422.x86_64.rpm |
| Mgbl | xrv9k-mgbl-1.0.0.0-r2422.x86_64.rpm |

# Configuration Backup

Copy the running-configuration to a harddisk on the router.

RP/0/RP0/CPU0:XRV9000# copy running-config harddisk:/running_config

Verify/fix configuration file system (mandatory):

RP/0/RP0/CPU0:XRV9000#cfs check

Copy the running-configuration to a remote scp server

RP/0/RP0/CPU0:XRV9000#scp harddisk:/<file name> root@1.2.3.4:/auto/config/.

# Pre-Upgrade Tasks

System Stability Check: The following commands should be executed to verify basic system stability before the upgrade. At the XR prompt:

sysadmin-vm:0_RP0# show install health

Mon Mar  11 15:34:37.297 UTC+00:00

INFO Platform is: xrv9k

INFO Collecting Cards Information

INFO Collecting Sysadmin VMs Information

INFO Collecting XR VMs Information

INFO Verifying all the required VMs are running.

INFO Pass: All required VMs are Running

INFO Verifying SCP file transfer on VMs

INFO Pass: scp file transfer succeed on all xr vms

INFO Pass: scp file transfer succeed on all cal vms

INFO Collecting HDPARM values of harddisk

INFO hdparm value on card 0/RP0 is 355.24 MB/sec

INFO Collecting sysadmin VMs data

INFO Collecting Host data

INFO Collecting XR VMs data

INFO Collecting Lead VMs data

INFO Verifying Test Plugins

INFO Verifying Plugins results

INFO Verifying Result for:cal_version

INFO Verifying Result for:host_version

INFO Verifying Result for:host_smus

INFO Verifying Result for:xr_version

INFO Verifying Result for:cal_smus

INFO Verifying Result for:smartctl

INFO Verifying Result for:xr_smus

INFO Verifying Result for:cal_local_active_swp

INFO Verifying Result for:cal_local_committed_swp

INFO Verifying Result for:xr_disk_space

INFO Verifying Result for:cal_disk_space

INFO Verifying Result for:xr_marker_files

INFO Verifying Result for:cal_marker_files

INFO Verifying Result for:xr_mount_points

INFO Verifying Result for:cal_mount_points

INFO Verifying Result for:xr_stale_symlinks

INFO Verifying Result for:cal_stale_symlinks

INFO Verifying Result for:xr_prepared_packages

INFO Verifying Result for:cal_prepared_packages

INFO Verifying Result for:xr_tmp_staging_dir

INFO Verifying Result for:cal_master_active_swp

INFO Verifying Result for:cal_master_committed_swp

INFO Verifying Result for:xr_master_active_swp

INFO Verifying Result for:xr_master_committed_swp

INFO Verifying Result for:xr_local_active_swp

INFO Verifying Result for:xr_local_committed_swp

INFO Verifying Result for:cal_image

INFO Verifying Result for:cal_tmp_staging_dir

INFO Verifying Result for:cal_install_tmp_staging_dir

INFO Verifying Result for:cal_repo_file_permission

INFO **********************************************************************

INFO System is in Consistent State. You can go ahead with next operation.

INFO **********************************************************************

INFO Total time taken: 20.4302339554 seconds.

- sysadmin-vm:0_RP0#

Cost-out IGP: To minimize traffic loss during the upgrade please follow below steps:

For OSPF use “max-metric” command.

RP/0/RP0/CPU0:XRV9000(config-ospf)# max-metric router-lsa

For ISIS use “spf-overload-bit” command.

RP/0/RP0/CPU0:XRV9000(config-isis)# set-overload-bit

Check available space in install repository and delete unnecessary files/folder if needed to make sure that sufficient memory is available

sysadmin-vm:0_RP0# show media

Example : Harddisk required minimum 2GB during image staging.

Check inactive packages and remove them before upgrading.

XR: RP/0/RP0/CPU0:XRV9000#install remove inactive all

Sysadmin: sysadmin-vm:0_RP0# show install inactive

Check and delete core files and any other files which are not required in harddisk

RP/0/RP0/CPU0:XRV9000#run

[xr-vm_node0_RP0_CPU0:~]$cd /misc/disk1

[xr-vm_node0_RP0_CPU0:/misc/disk1]$rm *core*

# Software Upgrade

Classic Method

All System Upgrade related install operations should be done in the XR VM plane.

Download 24.2.2 image from CCO.

Copy the 24.2.2 iso image / tar file along with the rpm packages to the router harddisk and verify that files are copied successfully

RP/0/RP0/CPU0:XRV9000#scp  root@1.2.3.4://auto/<image file> /misc/disk1/.

Verify the md5 checksum of the tar/individual rpms with the original MD5 values on CCO

[xr-vm_node0_RP0_CPU0:/misc/disk1]$md5sum xrv9k-mini-x-24.2.2.iso

Perform ‘install add’ of 2422 tar file / iso image and rpm packages:

RP/0/RP0/CPU0:XRV9000#install add source harddisk:/ xrv9k-mini-x-24.2.2.iso + Required rpms

OR

RP/0/RP0/CPU0:XRV9000#install add source tftp://auto/../ xrv9k-mini-x-24.2.2.iso

+ required rpm pkgs

Take a note of the install operation id generated by the add operation in previous step

Install operation 1 finished successfully

Activate all the packages

RP/0/RP0/CPU0:XRV9000#install activate id 1 noprompt synchronous

Router will reload at the end of activation operation for new packages to take effect.

Verify that all the packages are installed correctly in XR and SysAdmin

RP/0/RP0/CPU0:XRV9000#show install active

sysadmin-vm:0_RP0# show install active

Execute ‘install commit’ to commit the newly active software (install commit is required after any install activate operation else after router reload, nodes will go back to previously committed software)

RP/0/RP0/CPU0:XRV9000#install commit

Verify system stability through commands described under Check System Stability section (3.1) after router comes up with new software

Verify show version to check router is upgraded to desired image.

RP/0/RP0/CPU0:RR3-XRv-M5#show ver

Mon Sep 30 05:18:37.660 UTC

Cisco IOS XR Software, Version 24.2.2

Copyright (c) 2013-2024 by Cisco Systems, Inc.

Build Information:

Built By     : swtools

Built On     : Sun Sep 29 01:34:06 PDT 2024

Built Host   : iox-ucs-004

Workspace    : /auto/srcarchive10/prod/24.2.2/xrv9k/ws

Version      : 24.2.2

Location     : /opt/cisco/XR/packages/

Label        : 24.2.2

cisco IOS-XRv 9000 () processor

System uptime is 9 minutes

RP/0/RP0/CPU0:RR3-XRv-M5#

Check to see if there were any failed startup configurations and use CLI "clear configuration inconsistency" to clear failed configuration to proceed.

RP/0/RP0/CPU0:XRV9000#show configuration failed startup

Add recommended SMUs for 24.2.2 if not already in initial tarball (optional)

RP/0/RP0/CPU0:XRV9000#install add source harddisk: <mandatory SMU tar file for 24.2.2>

Activate the recommended SMUs (if recommended smu’s were added)

RP/0/RP0/CPU0:XRV9000#install activate id <add id of previous step> noprompt synchronous

After system comes up from reload, execute ‘install commit’

Please check “show install committed summary” to make sure your system is upgraded and committed with your desired version/24.2.2

# Post-Upgrade Tasks

Disk cleanup: Once software upgrade has been completed, disk space can be recovered by removing any inactive packages that are no longer needed (if the packages are required at a later time, they can be re-added):

RP/0/RP0/CPU0:XRV9000#install remove inactive all

Verify/fix configuration file system (mandatory):

RP/0/RP0/CPU0:XRV9000#cfs check

Restore IGP metric if changed before the upgrade (this is done from xr vm)

OSPF

RP/0/RP0/CPU0:XRV9000# (config-ospf)# no max-metric router-lsa

ISIS

RP/0/RP0/CPU0:XRV9000# (config-isis)# no set-overload-bit

# Software Downgrade:

Classic Method

All System Upgrade related install operations should be done in the XR VM plane.

Download 24.1.2 mini-ISO and packages tar and SMUs from CCO.

Copy iso image to tftp / scp / ftp server. Verify the contents of the tar file/iso image”

Copy the 24.1.2 iso image/tar file to the router harddisk and verify that file is copied successfully

RP/0/RP0/CPU0:XRV9000#scp  root@1.2.3.4://image/CCO/ xrv9k-mini-x-24.1.2.iso

RP/0/RP0/CPU0:XRV9000#scp  root@1.2.3.4://image/CCO/ rpm.tar

Verify the md5 checksum of the tar/individual rpms with the original MD5 values on CCO

- bash-4.2$ md5sum fullk9-R-XRV9000-792.tar

[xr-vm_node0_RP0_CPU0:/misc/disk1]$md5sum xrv9k-mini-x-24.1.2.iso

- [xr-vm_node0_RP0_CPU0:/misc/disk1]$

Perform ‘install add’ of 792 iso image / tar file:

RP/0/RP0/CPU0:XRV9000#install add source harddisk:/ xrv9k-mini-x-24.1.2.iso + Required Rpm tar

OR

RP/0/RP0/CPU0:XRV9000#install add source tftp://auto/../ xrv9k-mini-x-24.1.2.iso + required rpm pkgs

Take a note of the install operation id generated by the add operation in previous step

Install operation 9 finished successfully

Activate all the packages

RP/0/RP0/CPU0:XRV9000#install activate id 9 noprompt synchronous

Router will reload at the end of activation operation for new packages to take effect.

Verify that all the packages are installed correctly in XR and SysAdmin

RP/0/RP0/CPU0:XRV9000#show install active

sysadmin-vm:0_RP0# show install active

Execute ‘install commit’ to commit the newly active software (install commit is required after any install activate operation else after router reload, nodes will go back to previously committed software)

RP/0/RP0/CPU0:XRV9000#install commit

Verify system stability through commands described under Check System Stability section (3.1) after router comes up with new software

Verify show version to check router is upgraded to desired image.

RP/0/RP0/CPU0:XRV9000#show version

Check to see if there were any failed startup configurations and use CLI "clear configuration inconsistency" to clear failed configuration to proceed.

RP/0/RP0/CPU0:XRV9000#show configuration failed startup

Add recommended SMUs for 24.1.2 if not already in initial tarball (optional)

RP/0/RP0/CPU0:XRV9000#install add source harddisk: <mandatory SMU tar file for 24.1.2>

Activate the recommended SMUs (if recommended smu’s were added)

RP/0/RP0/CPU0:XRV9000#install activate id <add id of previous step> noprompt synchronous

After system comes up from reload, execute ‘install commit’

Please check “show install committed summary” to make sure your system is downgraded and committed with your desired version/24.1.2

# Post-Downgrade Tasks

Disk cleanup: Once software upgrade has been completed, disk space can be recovered by removing any inactive packages that are no longer needed (if the packages are required at a later time, they can be re-added):

RP/0/RP0/CPU0:XRV9000#install remove inactive all

Verify/fix configuration file system (mandatory):

RP/0/RP0/CPU0:XRV9000#cfs check

Restore IGP metric if changed before the upgrade (this is done from xr vm)

OSPF

RP/0/RP0/CPU0:XRV9000# (config-ospf)# no max-metric router-lsa

ISIS

RP/0/RP0/CPU0:XRV9000# (config-isis)# no set-overload-bit

# Caveats

Observed DDTS#CSCwe12104 - Xrv9k-Unable to upgrade/downgrade image from 772.10i to 781.33i.

Problem Description:

we are hitting an exception error as mentioned in DDTS# CSCwe12104 while downgrading from 2422 to 75x/76x/77x/78x releases, during admin VM reload the router is going unresponsive. We are observing this issue on XRv9K launched on ESXi hypervisor and Appliance mode. This issue is not SMuable, So Downgrade was blocked.

Recovering options:

- Fresh installation in ESXI hypervisor mode.

- USB boot and CIMC boot are workaround for Appliance mode.

Observed DDTS#CSCwj08501 - Increase the sysadmin ram allocation from 3GB to 4GB for install upgrade/downgrade failures.

Problem Description :

we are hitting an exception error as mentioned in DDTS# CSCwj08501while upgrading from 76x/77x/78x releases to 2422, during admin VM reload the router is going unresponsive.  This issue is specific to vRR ESXI and Appliance mode. SMU required for V1 image.

Observed DDTS# CSCwj52358 upgrade failed due to sysadmin vm restarted silently when upgrade XRV9K from 24.2.1.27I to 29I

Problem Description :

we are hitting an exception error as mentioned in DDTS# CSCwj52358 while downgrading from 2422 releases to 79x/710x/711x.  This issue is specific to vRR Appliance mode. This issue is not Smuable, USB boot is only option.