# FORTIGATE CLI COMMANDS FOR TROUBLESHOOTING

## Basic Configuration Commands:

FortiGate firewall is delivered with default settings of

Management IP Address: 192.168.1.99 / 255.255.255.0
Credentials: admin with no password

#### Configure Host Name:

FW# config system global
(global)# set hostname FG100D
FW# end

#### 2. Configure MGMT Interface

FW# config system interface
(interface)# edit mgmt
(mgmt)# set ip 192.168.80.254 255.255.255.0
(mgmt)# set allowaccess ping https ssh
(mgmt)# end

#### 3. Configure DNS

FW# config system dns
(dns)# set primary 192.168.80.150
(dns)# set secondary 192.168.80.151
(dns)# set domain “itadminguide.com”
(dns)# end

#### 4. Configure NTP Server

FW# config system ntp
(ntp)# set server 192.168.80.10
(ntp)# set status enable
(ntp)# end

## Basic Troubleshooting Commands:

#### 5. Ping and Traceroute host name or IP address

FW# execute ping

//ping from a specific firewall interface
FW# execute ping-options source <interface ip>
FW# execute ping <IP>

//traceroute from a specific firewall interface
FW# execute traceroute-options source <interface_ip>
FW# execute traceroute <IP>

#### 6. ARP

FW# get system arp

// clear arp table
FW# execute clear system arp table

#### 7. Routing

//similar to “show ip route” in Cisco
FW# get router info routing-table all

//route cache, current session with routing information
FW# diagnose ip rtcache list

## Automatic Backup Configuration:

#### 8. Automatic Configuration Backup after each logout

FW# config system global
FW# set revision-backup-on-logout enable
FW# end

All backup revisions can be seen in GUI > admin (top right) > Configuration > Revisions

## Troubleshooting IPSec VPN tunnel logs

When troubleshooting site-to-site IPSEC VPN tunnels in FortiGate firewalls, these commands enable debugging on the firewall console and provide detailed information to identify the problem.

- Login to CLI as admin

- Disable any debug that are currently running

diagnose debug disable

- Clear any debug filters that are previously applied

diagnose vpn ike log-filter clear

- Set filter to show debug logs of a specific VPN tunnel. This is especially helpful if you have several VPN tunnels and facing problem with only one peer.

diagnose vpn ike log-filter dst-addr4 10.10.10.1

- Enable debug mode on IKE handshaking process.

diagnose debug app ike 255

- Enable debug logging to console

diagnose debug enable

After you fix the problem, don’t forget to disable debug

diagnose debug disable