## FortiGate Sub Interfaces (VLAN Trucking)

## Problem

I was asked by a colleague at work the other day, can we replace the Cisco firewalls with FortiGate firewalls for a client? As a business we are heading towards Forti, but before I said yes I wanted to know what the firewall was actually doing before I said yes. On closer inspection the firewall in question didn’t appear to be doing anything too scary, but I did notice that the LAN interface was sub-interfaced to the various internal VLANs. I didn’t know how FortiGate handled this, so I fired it up on the test bench to test FortiGate Sub Interfaces.

So I needed to create TWO sub interfaces on the FortiGate (on port3).

## Creating FortiGate Sub Interfaces

Simply put, on a FortiGate if you want what a Cisco engineer would refer to as a ‘sub interface‘, then you simply add a VLAN interface to a physical interface. Like so, Network > Interfaces > {Physical Interface} > Create New > Interface.

Give the new interface a name (and alias if required) > Interface Type should be VLAN > Select the parent physical interface > Add the VLAN ID (Tag) and specify an IP address of the interface.

Just for testing I’ll allow PING, on the VLAN interface also > OK.

Repeat the procedure to add further sub interfaces (VLANs).

Remember this is just a ‘Router on a stick‘ configuration, to further allow traffic to the internet, (or between VLANs) you still need to add that traffic to the firewall policy to let the traffic through, (it is a firewall after all!)

### Setting up Switches for FortiGate Sub Interfaces?

I’ve probably got this covered elsewhere on the site, but the core switch is Cisco so I just created a trunk port, and allowed ALL VLANs, (because I’m lazy, in production, you might want to lock that down a little!)

!

interface Ethernet0/0

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface Ethernet0/1

switchport access vlan 150

switchport mode access

!

interface Ethernet0/2

switchport access vlan 200

switchport mode access

!

interface Ethernet0/3

switchport access vlan 150

switchport mode access

!

interface Ethernet1/0

switchport access vlan 200

switchport mode access

no cdp enable

!

My Switch isn’t Cisco it’s HP/Aruba! Then you simply TAG the VLANs required to the uplink see this article.