Reset and update Hitachi HCP node G10 “Quanta D51B-2U”

1:  Reset BMC password & update BMC

Only way To reset BMC password is to make a firmware update with same or newer version

Start server With Killdisk Usb key in  Windows PE mode or other windows PE boot

Make a Copy of the Firmware Map in root off the windows drive

Open CMD In Firmware Map and run the “Win.bat” to update BMC firmwre

2: Update Bios

Start server

Log into the bios and check BMC setup “IP address”

Start Explore with the “IP Address” in the last fan is “maintenance”! with Bios update