


























DF400 Disk Subsystem


1. SAFETY SUMMARY


















































Rev.9





�



CONTENTS


1. �EMBED Word.Picture.6��� General Safety Guidelines	SAF-0020


2. �EMBED Word.Picture.6��� Hazard Warning Statements	SAF-0070


3. �EMBED Word.Picture.6��� Locations Where Warning and Caution Labels Are Affixed	SAF-0090


3.1  Rackmount Type (RKL)	SAF-0090


3.2  Cabinet Type (CK)	SAF-0092


3.3  Rackmount Type (RK)	SAF-0100


3.4  Rackmount Type (RKY)	SAF-0120


3.5  Rack Frame (U3)	SAF-0140


3.6  Rack Frame (U4)	SAF-0155


3.7  Rack Frame (U5)	SAF-0170





�
�EMBED Word.Picture.6��� SAFETY SUMMARY


1.	�EMBED Word.Picture.6��� General Safety Guidelines


Read the following safety guidelines carefully and follow them when you conduct maintenance of the machine.


Before starting maintenance


(	Maintenance of the machine must be done only by trained and qualified engineers.


(	Read and follow the safety guidelines and procedures in this manuals.


(	In this manual and on the machine, hazard warnings are provided to aid you in preventing or reducing the risk of death, personal injury, or product damage. Understand and follow these hazard warnings fully.


(	The hazard warning which appear on the warning labels on the machine or in the manual have one of the following alert headings WARNING, or CAUTION.


� EMBED Word.Picture.6  ��� DANGER :	�
Indicates an imminently hazardous situation which, if not avoided, will result in death or serious injury.�
�
� EMBED Word.Picture.6  ��� WARNING :	�
Indicates a potentially hazardous situation which, if not avoided, can result in death or serious injury.�
�
� EMBED Word.Picture.6  ��� CAUTION :	�
Indicates a potentially hazardous situation which, if not avoided, will or can result in minor or moderate injury, or serious damage of product.�
�
� EMBED Word.Picture.6  ����
The alert symbol shown left precedes every signal word for hazard warnings, and appears in safety related descriptions in the manual.�
�
��
On some areas of the product you may find these symbols.  The mean CAUTION.  Do proceed with any action until you have read the instructions provided on the marking and in this manual.�
�
� EMBED Word.Picture.6  ����
On some areas of the product you may find this symbol.  It means CAUTION.  Take extra care to avoid personal injury, or damage to the product.�
�
��
On some areas of the product you may find this symbol.  It means that an abbreviated set of instructions are provided on the product in the general area of the symbol.�
�
The signal word ‘NOTICE’ is used to present warnings which are not directly related to personal injury hazards.


(	When warning labels become dirty or start peeling off, replace them.


(	Keep in mind that the hazard warnings in this manual or on the machine cannot cover every possible case, as it is impossible to predict and evaluate all circumstances beforehand.�Be alert and use your common sense.


(	This subsystem is a “class 1 laser system” which emits no hazardous laser beam.  Be sure to operate it according to this manual.  Do not perform an operations other than those written in this manual.  Otherwise, unexpected failures or accidents may be caused.


�
�EMBED Word.Picture.6��� SAFETY SUMMARY (Continued)


During work


(	For each procedure, follow the given sequence of steps.


(	Use the spare, consumables and materials for maintenance which are specified in the manual; otherwise, personal injury or damage of the machine, as well as deterioration of the product’s quality, may result.


(	Use the special tools and instruments specified for the work in the manual or commercially available tools and instruments which fit the purpose.


(	Use measurement instruments and powered tools which are properly calibrated or periodically inspected.


(	Keep the maintenance area neat and tidy.


(	Always put away parts, materials or tool when not in use.


(	Wear an eye protector where liquid may splash or anything may fly about.


(	When lifting anything heavy, hold it close to yourself and keep your back erect, to prevent injury to your back or spine.�When lifting anything, for the weight of which CAUTION is indicated, use a proper lifting tool or have somebody help you.


(	Keep a soldering iron and its stand away from you to prevent accidental contact and burns.


(	When using sharp objects or cutting tools, make sure that no part of your body lies in the path of the blade bid, or point.


(	Before finishing your work, make sure that all parts removed during maintenance have been in stalled back in their original positions in the machine.�Make sure that no tool or foreign material left in the machine.








�
�EMBED Word.Picture.6��� SAFETY SUMMARY (Continued)


Prevention of electric shocks


(	Before starting work, make sure that, unless otherwise specifically instructed, there is no potential electric hazard in the maintenance area such as insufficient grounding or a wet floor.


(	Before starting work, note where the emergency power-off switches are located and make sure you know how to operate them.


(	Unless otherwise specifically instructed, cut off all power sources to the machine before staring maintenance. Just switching off the machine power supplies is usually not enough.�When power is fed from a wall or floor outlet, unplug the power supply cord, or turn off the switch on the power distribution panel or board. Attach a notice on the panel or board prohibiting the use of the switch.�If the machine power has been already turned off, make sure yourself that these conditions are satisfied.


(	Do not touch any uninsured conductor or surface, where so instructed, which remains charged for a limited time after the external power supply to the machine is disconnected.


(	When working on a machine which has a grounding terminal, make sure that the terminals properly connected to the facility’s ground.


(	When working close to a hazardously energized part, do not work alone; work with another person who can immediately turn off the power in an emergency.


(	Do not wear any metallic item such as a wrist watch with a metallic surface, or metallic accessories.�If you wear eyeglasses with a metallic frame, take care not to let the frame touch an uninsured surface.


(	Make sure that your hands and arms are dry.


(	Unless otherwise specifically instructed, use only one hand when it is necessary to work near an exposed live electric circuit.�This prevents the completion of the circuit through both hands even if you accidentally touch the circuit.


(	Do not use a dental mirror near an exposed live electric circuit.�The mirror surface is conductive and can become hazardous even if it is made of plastic.


(	Unless otherwise specifically instructed, do not supply power to any subassembly such as a power supply unit or a motor while it is removed from the machine.





Preventing being caught by rotating or moving parts


(	Tuck in your tie, scarf, shirt, or any other loose clothing so that it will not be caught by a rotating or moving part.


(	Tie up long hair.


(	Unless otherwise specifically instructed, do not supply power to any device with rotating or moving parts which are not properly covered.


(	When instructed to supply power to any device with rotating or moving parts whose covers have been removed, work with another person who can immediately turn off the power in an emergency.


�
�EMBED Word.Picture.6��� SAFETY SUMMARY (Continued)


Procedure in an emergency


For electric schock


(	Do not panic. Do not become another victim through contact with the injured person.


(	First, shut off the electric current passing through the victim.�Use the emergency power-off switch, if there is one, or, otherwise, a normal power-off switch. If this cannot be done, push the victim away from the source of the electric current by using a nonconductor object such as a dry wooden stick.


(	Then, call an ambulance.


(	If the victim is unconscious, artificial respiration may be necessary.�A proper method for performing artificial respiration or resuscitation should be learned beforehand.�If the victim’s heart is not beating, card-pulmonary resuscitation should be performed by a trained and qualified person.





For outbreak of fire


(	First shut off all the power from the machine using the emergency power-off switch.


(	If the fire continues burning after the power is shut off, take suitable actions including the use of a fire extinguisher or a call for the fire department.











DF400	K6601626-





REV.9�
May.1999�
Jun.1999�
Aug.1999�
Mar.2000�
May.2000�
Sep.2000�
�
Copyright ( 1998, 2000, Hitachi, Ltd.


SAF-0000





REV.5�
May.1998�
Sep.1998�
Jan.1999�
Mar.1999�
May.1999�
Aug.1999�
�
Copyright ( 1998, 1999, Hitachi, Ltd.


SAF-0010





REV.3�
May.1998�
Jan.1999�
May.1999�
Aug.1999�
�
�
�
Copyright ( 1998, 1999, Hitachi, Ltd.


SAF-0020





REV.3�
May.1998�
Sep.1998�
Jan.1999�
Aug.1999�
�
�
�
Copyright ( 1998, 1999, Hitachi, Ltd.


SAF-0030





REV.1�
May.1998�
Aug.1999�
�
�
�
�
�
Copyright ( 1998, 1999, Hitachi, Ltd.


SAF-0040





REV.1�
May.1998�
Aug.1999�
�
�
�
�
�
Copyright ( 1998, 1999, Hitachi, Ltd.


SAF-0050














