2.4	Powering Off and On Procedures


Turn off the power correctly according to the “Powering OFF procedure (1)” shown below.


Notice :	If optional features are not installed to the system correctly, user data may be lost.


(1)	Powering OFF procedure


Notice :	When the AC/DC PS is duplicated, it is necessary to operate the Power ON/OFF switches of both AC/DC PSs.





� EMBED Word.Picture.6  ���


Notice :	If a power down occurs or the breaker is set to OFF by mistake before the POWER LED goes off, turn on the power right away according to the “(2) Powering ON procedure” below, then execute the power off procedure above.





�
*1 :	Check whether the cache memory is in the backup mode.�The cache memory installed in this subsystem is controlled with a write-after method.   When the power is turned off, the subsystem automatically writes data on the cache memory left unwritten onto the hard disk drive. (This operation is called a destaging.)   The subsystem turns off the power when this process is completed.   When the power is turned off owing to the unexpected power failure or by the operation of the circuit breaker, the powering off procedure cannot be performed.  In this case, the cache memory enters the backup mode in which the power is supplied by the battery to securing the data.


Notice :	If installation/de-installation of the mounted component (HDU ASSY, AC/DC PS, CTL ASSY, or BATTERY ASSY) is executed while keeping the subsystem in the backup mode, the battery may be powered off causing user’s data to be lost.�When handling the component with the subsystem power off, be sure to check if data in the cache memory has been backed up, and execute the maintenance in the state that the subsystem is released from the backup mode.


(	Checking the backup state of the cache memory�You can confirm whether the unit is in the backup mode by checking the indication of the CACHE PWR (green LED) on the CTL ASSY, as described below.�Set the circuit breaker on the rear of the AC/DC PS to “(” (OFF) position for “(” (ON) position in the state that the main switch (POWER SW) is set to “ � EMBED Word.Picture.6  ��� ” (OFF) position.�Determine the state of the memory through the indication of the CACHE PWR.


CACHE PWR LED on	Indicates that the cache memory is in the backup mode.�CACHE PWR LED off	Indicates that the backup mode is canceled.


Notice :	In case the data is not properly backed up, the user’s data will be lost.


(2)	Powering ON procedure


When restarting the subsystem, power it on after waiting for a minute or more from the power off.


Notice :	When the AC/DC PS is duplicated, it is necessary to operate the Power ON/OFF switches of both AC/DC PSs.


�EMBED Word.Picture.6���





�
(a)	Powering ON procedure (when the power control is set to REMOTE)


�EMBED Word.Picture.6���


(b)	Powering on procedure (when the power control is set to LOCAL)


� EMBED Word.Picture.6  ���








�
2.5	Installing the CTL ASSY and I/F ADAPTER ASSY


Perform the addition following the procedure below.�When adding the I/F ADAPTER ASSY only, do steps (1), (2), (3), (4), (6), (8), (9), and (10) only.�The I/F ADAPTER ASSY is installed on the CTL ASSY. When adding the I/F ADAPTER ASSY, the CTL ASSY must also be removed.�A quantity of the I/F ADAPTER ASSY (DLUDS, DLU2S) to be installed is one per CTL ASSY. When adding the CTL ASSY, an addition of the I/F ADAPTER ASSY is also required.


(1)	Insert a backup floppy disk in a drive and make a backup copy of the system parameter into the FD.


(2)	Turn off the unit. (See RKL-OPT-0200.)


Notice 1 :	If the power is not turned off normally, the user’s data might be lost.


Notice 2 :	For the interface type, SCSI or Fibre Channel can be selected. However, mixed use of the both types is not allowed in the same subsystem.


(3)	Remove the FAN ASSY.


(4)	Remove the DUMY(CTL)ASSY or the CTL ASSY to which the I/F ADAPTER ASSY is added.


(5)	Install a CACHE ASSY in the additional CTL ASSY. �Install the CACHE ASSYs with the same storage capacity in the CTL-00 which is already installed and the CTL-01 which is to be added.


(6)	Install the additional I/F ADAPTER ASSY.�Remove the DUMY(I/F)ASSY located at the position on the CTL ASSY where the I/F ADAPTER ASSY is to be installed.�When the subsystem is connected via the SCSI connection, remove the jumper pin cover and set the jumper pin ( JP1) in the same way as that of the I/F ADAPTER ASSY already installed. Set the jumper pin (JP2) to the automatic switching mode.


(7)	Add the CTL ASSY.�Insert the CTL ASSY in a predetermined position with the right and left levers opened. Push the right and left levers in the arrow marked erection ( � EMBED Word.Picture.6  ��� ) at the same time.








�



� EMBED Word.Picture.6  ���


Figure 2.5.1  Installing the CTL ASSY








�



� EMBED Word.Picture.6  ���


Figure 2.5.2  Installing the I/F ADAPTER ASSY


Table 2.5.1  Terminator Power Setting


No.�
Name�
Jumper setting�
Function�
�
1�
JP1�
�EMBED Word.Picture.6����
Turning on/off of the subsystem power is controlled locally.


•	Setting made at factory before shipment:�+5V terminator power is supplied from both of the DF400 and the host.


(Self supply)�
�
�
�
�EMBED Word.Picture.6����
Turning on/off of the subsystem power is controlled remotely by the turning on/off of the terminator power.


+5V terminator power is supplied from the power supply unit of the host only.	(*)


(Supply from the outside)�
�
*: When connecting the UPS, set the terminator power source to the outside one.





�
(8)	Connect SCSI cable (Fibre Channel cable), terminator, LAN (option) cable, RS232C cable.


��EMBED Word.Picture.6���


(9)	Attach the FAN ASSY.


(10)	Perform the setting operation (*1) following the flowchart below.


� EMBED Word.Picture.6  ���


(*1)	For details of the setting, see Chapter 8, “PANEL”.


(*2)	To confirm the revision number of the flash program, see Section 3.2, “Confirmation of Flash Program Revision” in Chapter 7 on page MIC-0080.








�
Procedure (


(a)	Open the panel cover.


(b)	Operate as described below.


� EMBED Word.Picture.6  ���








�



� EMBED Word.Picture.6  ���





�
(c)	When the system parameter setting menu shown below is displayed, perform the following operation referring to page PNL-0070.�Select ‘DUAL MODE’ from the ‘DUAL CONFIG’ (dual operation attribute setting menu, and set it to the ‘DUAL ACTIVE’ or ‘HOT STAND-BY’. Set a TARGET ID for each PORT by selecting it from the ‘TARGET ID’ (TARGET ID setting) menu.)�Set the items #1, 3, 4, 6, 8, 10, 12, 18, 19, 20, and 24 in system parameter set up menu (See PNL-0070) for each controller individually. (The other menus are common menus. Same settings are executed for both controllers. Modification by CTL-01 is not available.)


(	The system parameter setting menu appears on the panel in the order of CTL-00 and CTL-01.


� EMBED Word.Picture.6  ���


(d)	After the setting is ended, make a copy of the data into the backup FD again. (See RKL-OPT-0030.)


(e)	In a short while, the READY LED on the unit comes on.�Check to see “Version = XXXX/X” is displayed on the panel, and that corresponds to see system floppy disk version.


(f)	Close the panel cover.








�
Supplementary procedure to the procedure (�: When performing the setting using the DIP switch


(a)	Remove the front bezel.


(b)	Set the selector No.4 of the DIP switch 1 (locating in the CTL ASSY which is slready installed) and the selectors No.3 and No.4 of the DIP switch 2 (locating in the CTL ASSY which is to be added) on the KEYBOARD PANEL ASSY to the ON position, install the KEYBOARD PANEL ASSY, and turn the power on.


� EMBED Word.Picture.6  ���


(c)	Insert the system parameter backup FD and restore the system parameter data from the FD.


(d)	When the system parameter setting menu shown below is displayed, perform the following operation referring to page PNL-0070.�Select ‘DUAL MODE’ from the ‘DUAL CONFIG’ (dual operation attribute setting menu, and set it to the ‘DUAL ACTIVE’ or ‘HOT STAND-BY’. Set a TARGET ID for each PORT by selecting it from the ‘TARGET ID’ (TARGET ID setting) menu.)�Set the items #1, 3, 4, 6, 8, 10, 12, 18, 19, 20, and 24 in system parameter set up menu (See PNL-0070) for each controller individually. (The other menus are common menus. Same settings are executed for both controllers. Modification by CTL-01 is not available.)


(	The system parameter setting menu appears on the panel in the order of CTL-00 and CTL-01.


(	“(” appears on the panel when the CTL-01 system parameter is to be set.


“(” not displayed	: CTL-00 system parameter is set.


“(” displayed	: CTL-01 system parameter is set.


� EMBED Word.Picture.6  ���


(e)	After the setting is ended, make a copy of the data into the backup FD again. (See RKL-OPT-0030)


(f)	Check to see that READY LED is on.


(g)	Turn off the unit (by setting the main switch on the LCD PANEL ASSY to off).


(h)	Set the DIP switch 1, and 2 on KEYBOARD PANEL ASSY as shown below.


� EMBED Word.Picture.6  ���


(i)	Turn on the unit (by setting the main switch on the LCD PANEL ASSY to on).





�
(j)	In a short while, the READY LED on the unit comes on.�Check to see “Version = XXXX/X” is displayed on the panel, and that corresponds to see system floppy disk version.


(k)	Attach the front bezel.











�
2.6	Installing the CACHE ASSY


(1)	Turn off the power according to the “Powering Off and On procedures” in RKL-OPT-0200.


Notice :	If the power is not turned off normally, the user’s data might be lost.


Notice :	When the power has been already turned off, check the CACHE PWR LED to see if the cache backup is not being performed. If the cache backup is being performed, turn on and off the replacement after the power is turned off normally.


Sometimes the subsystem does not go power off after the main switch turned to “� EMBED Word.Picture.6  ���” side with panel display “TOOMNYPIN” or “SYSDTSVER” or “POFFCMUNC”. In this case refer to page TRB-0030 and turn off the power.


(2)	Remove the FAN ASSY.


(3)	Loosen the right and left screws of the CTL ASSY.


(4)	Pull the right and left lever of the CTL ASSY at the same time in the direction shown by the arrow ( �EMBED Word.Picture.6��� ) and take out the CTL ASSY.


(5)	Install the additional CACHE ASSY.


Notice :	When installing the CACHE ASSYs, install them in the order starting from the cache slot #00.


Notice :	CACHE ASSY of the largest capacity must be mounted cache slots #00 in case that CACHE ASSYs of different capacity are used.


(6)	Install the CTL ASSY.


(7)	Attach the FAN ASSY.


(8)	Turn on the subsystem following the “Powering ON procedure” explained in RKL-OPT-0210.�(The READY LED on the subsystem comes on.)








�



� EMBED Word.Picture.6  ���


Figure 2.6.1  Installing the CACHE ASSY





DF400	K6602032-





REV.1�
May.1999�
May.2000�
�
�
�
�
�
Copyright © 1999, 2000 Hitachi, Ltd.


RKL-OPT-0200





REV.2�
May.1999�
Aug.1999�
May.2000�
�
�
�
�
Copyright © 1999, 2000 Hitachi, Ltd.


RKL-OPT-0210





REV.1�
May.1999�
Jun.1999�
�
�
�
�
�
Copyright © 1999, Hitachi, Ltd.


RKL-OPT-0220





REV.2�
May.1999�
Jun.1999�
Aug.1999�
�
�
�
�
Copyright © 1999, Hitachi, Ltd.


RKL-OPT-0230





REV.0�
Aug.1999�
�
�
�
�
�
�
Copyright © 1999, Hitachi, Ltd.


RKL-OPT-0231





REV.3�
May.1999�
Jun.1999�
Aug.1999�
Mar.2000�
�
�
�
Copyright © 1999, 2000, Hitachi, Ltd.


RKL-OPT-0240





REV.3�
May.1999�
Jun.1999�
Aug.1999�
Mar.2000�
�
�
�
Copyright © 1999, 2000, Hitachi, Ltd.


RKL-OPT-0250





REV.0�
May.1999�
�
�
�
�
�
�
Copyright © 1999, Hitachi, Ltd.


RKL-OPT-0260





REV.0�
May.1999�
�
�
�
�
�
�
Copyright © 1999, Hitachi, Ltd.


RKL-OPT-0270





REV.1�
May.1999�
Mar.2000�
�
�
�
�
�
Copyright © 1999, 2000, Hitachi, Ltd.


RKL-OPT-0280





REV.2�
May.1999�
Aug.1999�
Mar.2000�
�
�
�
�
Copyright © 1999, 2000, Hitachi, Ltd.


RKL-OPT-0290





REV.0�
May.1999�
�
�
�
�
�
�
Copyright © 1999, Hitachi, Ltd.


RKL-OPT-0300





REV.1�
May.1999�
Mar.2000�
�
�
�
�
�
Copyright © 1999, 2000, Hitachi, Ltd.


RKL-OPT-0310





REV.0�
May.1999�
�
�
�
�
�
�
Copyright © 1999, Hitachi, Ltd.


RKL-OPT-0320








Notice :	When bending the Fibre Channel cable to connect it, give it a bend with a long radius (not less than 30 mm) so as not to apply the cable and the connector an excessive stress.











