2.2.5	Routing the interface cables of the mounted subsystem (RKL) 


( In the case of the U4 model, Rev.A


�Notice :	When bending the Fibre Channel cable to connect it, �give it a bend with a long radius (not less than 30 mm) �so as not to apply the cable and the connector an �excessive stress.


	Pass all the interface cables down to the floor through the opening on the bottom and fasten them with the repeat binders.


(1)	Route the I/F cable through outside of the rail (left) in the rear of the subsystem and fasten it with a Repeat Binder. 


� EMBED Word.Picture.6  ���


Figure 2.2.5-a  Routing the I/F Cable (Rev.A)








(2)	Close the rear door.�
( In the case of the U4 model, Rev.B


�Notice :	When bending the Fibre Channel cable to connect it, �give it a bend with a long radius (not less than 30 mm) �so as not to apply the cable and the connector an �excessive stress.


	Pass all the interface cables down to the floor through the opening on the bottom and fasten them with the repeat binders.


(1)	Route the I/F cable through outside of the rail (left) in the rear of the subsystem and fasten it with a Repeat Binder. 





� EMBED Word.Picture.6  ���


Figure 2.2.5-b  Routing the I/F Cable (Rev.B)





(2)	Close the rear door.


DF400	K6601945-





REV.0�
Aug.1999�
�
�
�
�
�
�
Copyright ( 1999, Hitachi, Ltd.


FR-INS-0157





REV.0�
Aug.1999�
�
�
�
�
�
�
Copyright ( 1999, Hitachi, Ltd.


FR-INS-0158








Fibre Channel cable





R





I/F ADAPTER ASSY





Fibre Channel cable





R





I/F ADAPTER ASSY











