(4)	Installing the side panel and skirt (side)


Install the side panel and skirt that were removed from the CPU frame on the terminal rack frame.


(i)	Fasten the side panel with the eight.


(ii)	Fasten the two side hooks with the screws( (two each).


(iii)	Fasten the skirt (side)with the three screws(


(iv)	Open the front panel.


(v)	Fasten the side panel and the grand cables.





� EMBED Word.Picture.6  ���





Figure 3.2.6  Installing the Side Cover and Skirt





�
(5)	Installing the skirts


(a)	Install the skirts on the front and rear sides of the rack frame with screws (. (See Figure 3.2.7)


Notice :	Make the bottom of each skirt contact the floor to eliminate static electricity.





� EMBED Word.Picture.6  ���





Figure 3.2.7  Installing the Skirt





�
3.3	Connecting cables


Notice :	Start the work after referring to notes on cable routing given on page ENT-0081 in Chapter 2, “ENTRY”.


3.3.1	Connecting power cables


Notice :	For the connection of the U5 rack frame to the DF400-RK, see in the FR-OPT manual.





Tool : Phillips screwdriver (No.2)


Connect one of the following cables to the U5 rack frame according to the place where it is used.


Place of use�
Name of Parts�
Parts No.�
Quantity�
Comment�
�
North America�
AC CABLE�
3502682-A�
2�
4 m�
�
Europe �
AC CABLE�
3502683-A�
2�
4 m�
�






� EMBED Word.Picture.6  ���


Take full care to connect the power cables correctly and tighten the screws firmly lest they should loosen.


(1)	Open the front door.


(2)	Remove the Lod of the front door.


(3)	Remove screws ( and remove the cable guide.


(4)	Loosen the screws (, and remove the cable cover.


(5)	Pass the power cable through the cutout of the bottom of the subsystem and pull it into the subsystem.


(6)	Route the power cable to connect in to the power receiving connector of the PDB.


(7)	Bind the power cable connected to the PDB1 (upper PDB) to the rack fame with the cable cover by fastening the cover using the screws (.


(8)	Fasten the cable guide with the screws (. 





�












� EMBED Word.Picture.6  ���





Figure 3.3.1  Connecting the Power Cable





�
3.3.2	Connecting the interface cables of the mounted subsystem


�Notice :	When bending the Fibre Channel cable to connect it, �give it a bend with a long radius (not less than 30 mm) �so as not to apply the cable and the connector an �excessive stress.


(1)	Pass the I/F cable through an opening for the external connection cables on the bottom of the rack frame, and pull it into the frame.


(2)	Connect the I/F cable to the specified connector of the I/F ADAPTER ASSY.


(3)	Clamp the I/F cable with a locking wire saddle, and route it.   When doing this, give the cable excess length to allow components to be replaced.   Besides, be careful not to bend the cable sharply.


� EMBED Word.Picture.6  ���


Figure 3.3.2  Routing the I/F Cables





DF400	K6601945-





REV.0�
Jan.1999�
�
�
�
�
�
�
Copyright ( 1999, Hitachi, Ltd.


FR-INS-0250





REV.0�
Jan.1999�
�
�
�
�
�
�
Copyright ( 1999, Hitachi, Ltd.


FR-INS-0260





REV.2�
Jan.1999�
May.1999�
Aug.1999�
�
�
�
�
Copyright ( 1999, Hitachi, Ltd.


FR-INS-0270





REV.0�
May.1999�
�
�
�
�
�
�
Copyright ( 1999, Hitachi, Ltd.


FR-INS-0272





REV.2�
Jan.1999�
May.1999�
Aug.1999�
�
�
�
�
Copyright ( 1999, Hitachi, Ltd.


FR-INS-0280








Fibre Channel cable





R





I/F ADAPTER ASSY











