3.6.4 Installation of Serial 8-port Adapter (DKC-F310I-8S)

Note : When installing three or more channel options, �the CL Additional Power Supply (DKC-F310I-80) must be installed. �In such a case, be sure to add the channel option(s) after the CL Additional Power Supply is added.



3.6.4.1 Parts list

Table 3.6.4-1  Parts List for Serial 8-port Adapter

No.�Model Number�Part Name�Part No.�Quantity�Remarks��1�DKC-F310I-8S�Serial 4-port Adapter PCB�3252509-A�2�����I/F Connector Panel�3250912-A�2�����Screw�SB408�8�����Label�5500245-1�4�For Serial����Screw�SB306�4�����Bracket�5471998-1�2�����Holder�2084816-1�8�����Nameplate�3252542-4�1�For DKC-F310I-8S��



3.6.4.2 Flowchart



(1)	Insert the PCBs.

(

(2)	Install the I/F connector panel.

(

(3)	Connect the optical fiber cables.

(

(4)	Attach the labels.

(

(5)	Attach the nameplate.





�3.6.4.3 Installation Procedure of Serial 8-port Adapter



Note:	Be sure to wear your wrist strap and attach to ground prior to performing the following work.  This will ensure that the IC and LSI on the PCB are protected from static electricity.



Insertion of the PCBs

a.	Remove the dummy plate installed in the installation location referring to the Fig. 3.6.4-1.�(Note) Dummy plates should be stored for future use in De-installation.

b.	Insert the PCBs to the correct locations in the Logic Box.  Refer to Table 3.6.4-2.

c.	Fasten the two screws referring to Fig. 3.6.4-2.



Table 3.6.4-2  Inserting Location (Front of the unit)

Cluster�CL1�CL2�CL1��Slot No.�N�P�Q�R�S�T�U�V�W�X�Y�Z��Function�TM�CHA�CHA�CHA�CHA�CACHE�CACHE�CHA�CHA�CHA�CHA�TM��Location No.�TM-1N�CHA

-1P�CHA

-1Q�CHA

-1R�CHA

-1S�CACHE

-1T�CACHE

-2U�CHA

-2V�CHA

-2W�CHA

-2X�CHA

-2Y�TM-1Z��Order of addition��Basic�Add.1�Add.2�Add.3���Basic�Add.1�Add.2�Add.3���	Up to 4 serial 8-port adapters can be installed in the subsystem.



� EMBED Word.Picture.6  ���



	Fig. 3.6.4-1  Removal of Dummy Plate		Fig. 3.6.4-2  Insertion of PCB



�Installation of the I/F connector panel�Install the I/F connector panel in the order of the location number (((( shown in Fig. 3.6.4-3.  When the I/F connector panel is installed to the location ( in the DKC which installs the OUTLET BOX (maintenance tool), remove the OUTLET BOX referring to Fig. 3.6.4-4 before installation of the I/F connector panel. However, when a mixture of parallel channel is installed, referring to [LOCATION04-40] in a LOCATION Section.



���� EMBED Word.Picture.6  ���

Fig. 3.6.4-3 Location of I/F Connector Panel (Front)



a.	When the I/F connector panel is installed to the location (, remove the OUTLET BOX referring to Fig. 3.6.4-4.

b.	Remove the dummy plate referring to Fig. 3.6.4-5.

c.	Install the I/F connector panel referring to Fig. 3.6.4-6.�(Note): Dummy plates should be stored for future use in de-installation.

d.	When the OUTLET BOX was removed, install the OUTLET BOX to the original position after installing the I/F connector panel.

� EMBED Word.Picture.6  ���

Fig. 3.6.4-4  Removal of OUTLET BOX

� EMBED Word.Picture.6  ���

Fig. 3.6.4-5  Removal of Dummy Plate	Fig. 3.6.4-6  Installation of I/F Connector Panel



�Connection of the optical fiber cable

a.	Pull the optical fiber cable into the DKC through the I/F connector panel.�Connect the optical fiber cable to the PCB referring to Serial Channel Interface [LOCATION04-60] in LOCATION section.

b.	Fix the cable with Holder( referring to Fig. 3.6.4-7.

c.	Fix the cable to Lower Strain Relief( and attach Upper Strain Relief( and Bracket( referring to Fig. 3.6.4-8.

� EMBED Word.Picture.6  ���	� EMBED Word.Picture.6  ���

�





Labeling

a.	Attach the labels "SERIAL PORT" to ( through ( referring to Table. 3.6.4-3.



Table 3.6.4-3  Attachment of Labels

Order of addition�Basic�Add.1�Add.2�Add.3��Labeling position�( and (�( and (�( and (�( and (��Channel No.�A,B,C,D�E,F,G,H�J,K,L,M�N,P,Q,R��















�



















Fig. 3.6.4-9  Attachment of Labels

�Attachment of the nameplate

a.	Attach the nameplate referring to Fig. 3.6.4-10.



� EMBED Word.Picture.6  ���

Fig. 3.6.4-10  Attachment of Nameplate



Return to General Flow.�New Installation: Go to INST02-11 Step (6).�Non-disruptive Installation: Go to INST02-21 Step (4)-2.�Disruptive Installation: Go to INST02-40 Step (5).





�3.6.5 Installation of SCSI 8-port Adapter (DKC-F310I-8C)

Note : When installing three or more channel options, �the CL Additional Power Supply (DKC-F310I-80) must be installed. �In such a case, be sure to add the channel option(s) after the CL Additional Power Supply is added.



3.6.5.1 Parts list

Table 3.6.5-1  Parts List for SCSI 8-port Adapter

No.�Model Number�Part Name�Part No.�Quantity�Remarks��1�DKC-F310I-8C�SCSI 4-port Adapter PCB�3252505-A�2�����I/F Connector Panel�2102035-A�1�Cluster 1 side����I/F Connector Panel�2102035-B�1�Cluster 2 side����Screw�SB408�8�����Label�5500247-1�4�SCSI����Nameplate�3252542-6�1�For DKC-F310I-8C����Connector (Terminator)�5486080-868�8���



3.6.5.2 Flowchart



(1)	Insert the PCBs.

(

(2)	Install the I/F connector panels.

(

(3)	Connect the SCSI cables.

(

(4)	Attach the labels.

(

(5)	Attach the nameplate.





�3.6.5.3 Installation Procedure of SCSI 8-port Adapter



Note:	Be sure to wear your wrist strap and attach to ground prior to performing the following work.  This will ensure that the IC and LSI on the PCB are protected from static electricity.



Insertion of the PCBs

a.	Remove the dummy plate installed in the installation location referring to the Fig. 3.6.5-1.�(Note) Dummy plates should be stored for future use in De-installation.

b.	Insert the PCBs to the correct locations in the Logic Box.  Refer to Table 3.6.5-2.

c.	Fasten the two screws referring to Fig. 3.6.5-2.



Table 3.6.5-2  Inserting Location (Front of the unit)

Cluster�CL1�CL2�CL1��Slot No.�N�P�Q�R�S�T�U�V�W�X�Y�Z��Function�TM�CHA�CHA�CHA�CHA�CACHE�CACHE�CHA�CHA�CHA�CHA�TM��Location No.�TM-1N�CHA

-1P�CHA

-1Q�CHA

-1R�CHA

-1S�CACHE

-1T�CACHE

-2U�CHA

-2V�CHA

-2W�CHA

-2X�CHA

-2Y�TM-1Z��Order of addition��Basic�Add.1�Add.2�Add.3���Basic�Add.1�Add.2�Add.3���	Up to 4 SCSI 8-port adapters can be installed in the subsystem.



� EMBED Word.Picture.6  ���



	Fig. 3.6.5-1  Removal of Dummy Plate		Fig. 3.6.5-2  Insertion of PCB



�Installation of the I/F connector panel�Install the I/F connector panel in the order of the location number (((( shown in Fig. 3.6.5-3.  When the I/F connector panel is installed to the location ( in the DKC which installs the OUTLET BOX (maintenance tool), remove the OUTLET BOX referring to Fig. 3.6.5-4 before installation of the I/F connector panel. However, when a mixture of parallel channel is installed, referring to [LOCATION04-40,70] in a LOCATION Section.



���� EMBED Word.Picture.6  ���

Fig. 3.6.5-3 Location of I/F Connector Panel (Front)



a.	When the I/F connector panel is installed to the location (, remove the OUTLET BOX referring to Fig. 3.6.5-4.

b.	Remove the dummy plate referring to Fig. 3.6.5-5.

c.	Wire the SCSI cables to the front of DKC and then install the I/F connector panel referring to Fig. 3.6.5-6.�(Note): Dummy plates should be stored for future use in de-installation.

d.	When the OUTLET BOX was removed, install the OUTLET BOX to the original position after installing the I/F connector panel.



� EMBED Word.Picture.6  ���

Fig. 3.6.5-4  Removal of OUTLET BOX

� EMBED Word.Picture.6  ���

Fig. 3.6.5-5  Removal of Dummy Plate	Fig. 3.6.5-6  Installation of I/F Connector Panel



�Connection of the SCSI cables

In case of Basic

a.	Connect the SCSI cables to the sub-edge connectors on PCB.  Refer to Fig. 3.6.5-7 and Table 3.6.5-3.



Table 3.6.5-3  SCSI Cable Connection

No.�SCSI cable�Sub-edge connector����on PCB��1�1F(SC1�1F(P1��2�1F(SC2�1F(P2��3�1F(SC3�1F(P3��4�1F(SC4�1F(P4��5�2F(SC1�2F(V1��6�2F(SC2�2F(V2��7�2F(SC3�2F(V3��8�2F(SC4�2F(V4��� EMBED Word.Picture.6  ���

Fig. 3.6.5-7  Connection of SCSI Cables (Basic)



In case of Add.1

a.	Connect the SCSI cables to the sub-edge connectors on PCB.  Refer to Fig. 3.6.5-8 and Table 3.6.5-4.



Table 3.6.5-4  SCSI Cable Connection

No.�SCSI cable�Sub-edge connector����on PCB��1�1F(SC1�1F(Q1��2�1F(SC2�1F(Q2��3�1F(SC3�1F(Q3��4�1F(SC4�1F(Q4��5�2F(SC1�2F(W1��6�2F(SC2�2F(W2��7�2F(SC3�2F(W3��8�2F(SC4�2F(W4��� EMBED Word.Picture.6  ���

Fig. 3.6.5-8  Connection of SCSI Cables (Add.1)





�In case of Add.2

a.	Connect the SCSI cables to the sub-edge connectors on PCB.  Refer to Fig. 3.6.5-9 and Table 3.6.5-5.



Table 3.6.5-5  SCSI Cable Connection

No.�SCSI cable�Sub-edge connector����on PCB��1�1F(SC1�1F(R1��2�1F(SC2�1F(R2��3�1F(SC3�1F(R3��4�1F(SC4�1F(R4��5�2F(SC1�2F(X1��6�2F(SC2�2F(X2��7�2F(SC3�2F(X3��8�2F(SC4�2F(X4��� EMBED Word.Picture.6  ���

Fig. 3.6.5-9  Connection of SCSI Cables (Add.2)



In case of Add.3

a.	Connect the SCSI cables to the sub-edge connectors on PCB.  Refer to Fig. 3.6.5-10 and Table 3.6.5-6.



Table 3.6.5-6  SCSI Cable Connection

No.�SCSI cable�Sub-edge connector����on PCB��1�1F(SC1�1F(S1��2�1F(SC2�1F(S2��3�1F(SC3�1F(S3��4�1F(SC4�1F(S4��5�2F(SC1�2F(Y1��6�2F(SC2�2F(Y2��7�2F(SC3�2F(Y3��8�2F(SC4�2F(Y4��� EMBED Word.Picture.6  ���

Fig. 3.6.5-10  Connection of SCSI Cables (Add.3)









�Labeling

a.	Attach the labels "SCSI PORT" to ( through ( referring to Table. 3.6.5-7.



Table 3.6.5-7  Attachment of Labels

Order of addition�Basic�Add.1�Add.2�Add.3��Labeling position�( and (�( and (�( and (�( and (��Channel No.�A,B,C,D�E,F,G,H�J,K,L,M�N,P,Q,R��















�























Fig. 3.6.5-11  Attachment of Labels



�Attachment of the nameplate

a.	Attach the nameplate referring to Fig. 3.6.5-12.



� EMBED Word.Picture.6  ���

Fig. 3.6.5-12  Attachment of Nameplate





Return to General Flow.�New Installation: Go to INST02-11 Step (6).�Non-disruptive Installation: Go to INST02-21 Step (4)-2.�Disruptive Installation: Go to INST02-40 Step (5).







�3.6.6 Installation of Fibre 4-port Adapter for Optical Cable (DKC-F310I-4F/4FE)

Note : When installing three or more channel options, �the CL Additional Power Supply (DKC-F310I-80) must be installed. �In such a case, be sure to add the channel option(s) after the CL Additional Power Supply is added.



3.6.6.1 Parts list

Table 3.6.6-1  Parts List for Fibre 4-port Adapter

No.�Model Number�Part Name�Part No.�Quantity�Remarks��1�DKC-F310I-4F�Fibre 2-port Adapter PCB�3252506-A�2�����I/F Connector Panel�3250912-A�2�����Screw�SB408�8�����Label�5500248-1�4�For Fibre����Screw�SB306�4�����Bracket�5471998-1�2�����Nameplate�3252542-7�1�For DKC-F310I-4F��2�DKC-F310I-4FE�Fibre 2-port Adapter PCB�3252506-C�2�����I/F Connector Panel�3250912-A�2�����Screw�SB408�8�����Label�5500248-1�4�For Fibre����Screw�SB306�4�����Bracket�5471998-1�2�����Nameplate�3252542-75�1�For DKC-F310I-4FE��



3.6.6.2 Flowchart



(1)	Insert the PCBs.

(

(2)	Install the I/F connector panel.

(

(3)	Connect the optical fiber cables.

(

(4)	Attach the labels.

(

(5)	Attach the nameplate.





�3.6.6.3 Installation Procedure of Fibre 4-port Adapter



Note:	Be sure to wear your wrist strap and attach to ground prior to performing the following work.  This will ensure that the IC and LSI on the PCB are protected from static electricity.



Insertion of the PCBs

a.	Remove the dummy plate installed in the installation location referring to the Fig. 3.6.6-1.�(Note) Dummy plates should be stored for future use in De-installation.

b.	Insert the PCBs to the correct locations in the Logic Box.  Refer to Table 3.6.6-2.

c.	Fasten the two screws referring to Fig. 3.6.6-2.



Table 3.6.6-2  Inserting Location (Front of the unit)

Cluster�CL1�CL2�CL1��Slot No.�N�P�Q�R�S�T�U�V�W�X�Y�Z��Function�TM�CHA�CHA�CHA�CHA�CACHE�CACHE�CHA�CHA�CHA�CHA�TM��Location No.�TM-1N�CHA

-1P�CHA

-1Q�CHA

-1R�CHA

-1S�CACHE

-1T�CACHE

-2U�CHA

-2V�CHA

-2W�CHA

-2X�CHA

-2Y�TM-1Z��Order of addition��Basic�Add.1�Add.2�Add.3���Basic�Add.1�Add.2�Add.3���	Up to 4 Fibre 4-port adapters can be installed in the subsystem.

� EMBED Word.Picture.6  ���

Fig. 3.6.6-1  Removal of Dummy Plate	Fig. 3.6.6-2  Insertion of PCB 



�Installation of the I/F connector panel�Install the I/F connector panel in the order of the location number (((( shown in Fig. 3.6.6-3.  When the I/F connector panel is installed to the location ( in the DKC which installs the OUTLET BOX (maintenance tool), remove the OUTLET BOX referring to Fig. 3.6.6-4 before installation of the I/F connector panel. However, when a mixture of parallel channel is installed, referring to [LOCATION04-40] in a LOCATION Section.



���� EMBED Word.Picture.6  ���

Fig. 3.6.6-3 Location of I/F Connector Panel (Front)



a.	When the I/F connector panel is installed to the location (, remove the OUTLET BOX referring to Fig. 3.6.6-4.

b.	Remove the dummy plate referring to Fig. 3.6.6-5.

c.	Install the I/F connector panel referring to Fig. 3.6.6-6.�(Note): Dummy plates should be stored for future use in de-installation.

d.	When the OUTLET BOX was removed, install the OUTLET BOX to the original location after installing the I/F connector panel.



� EMBED Word.Picture.6  ���

Fig. 3.6.6-4  Removal of OUTLET BOX

� EMBED Word.Picture.6  ���

Fig. 3.6.6-5  Removal of Dummy Plate	Fig. 3.6.6-6  Installation of I/F Connector Panel



�Connection of the optical fiber cable

a.	Pull the optical fiber cable into the DKC through the I/F connector panel referring to Fig. 3.6.6.7. Connect the optical fiber cable to the PCB. Refer to Fibre Channel Interface [LOCATION04-80] in LOCATION section.

b.	Fix the cable to Lower Strain Relief( and attach Upper Strain Relief( and Bracket( referring to Fig. 3.6.6-8.



� EMBED Word.Picture.6  ���� EMBED Word.Picture.6  ���

�







�Labeling

a.	Attach the labels "FIBRE PORT" to (,(,(, or ( referring to Table. 3.6.6-3 and Fig. 3.6.6-9.



Table 3.6.6-3  Attachment of Labels

Order of addition�Basic�Add.1�Add.2�Add.3��Labeling position�(�(�(�(��Channel No.�A,B�E,F�J,K�N,P��

�



















































Fig. 3.6.6-9  Attachment of Labels





�Attachment of the nameplate

a.	Attach the nameplate referring to Fig. 3.6.6-10.



� EMBED Word.Picture.6  ���

Fig. 3.6.6-10  Attachment of Nameplate





Return to General Flow.�New Installation: Go to INST02-11 Step (6).�Non-disruptive Installation: Go to INST02-21 Step (4)-2.�Disruptive Installation: Go to INST02-40 Step (5).





�3.6.7 Installation of Fibre 4-port Adapter for Optical Cable (DKC-F310I-4GS/4GL)

Note : When installing three or more channel options, �the CL Additional Power Supply (DKC-F310I-80) must be installed. �In such a case, be sure to add the channel option(s) after the CL Additional Power Supply is added.



3.6.7.1 Parts list

Table 3.6.7-1  Parts List for Fibre 4-port Adapter (Short Wavelength)

No.�Model Number�Part Name�Part No.�Quantity�Remarks��1�DKC-F310I-4GS�Fibre 2-port Adapter PCB�3254519-A�2����(Short Wavelength)�I/F Connector Panel�3250912-A�2�����Screw�SB408�8�����Label�5500248-1�4�For Fibre����Screw�SB306�4�����Bracket�5471998-1�2�����Nameplate�3252542-63�1���

Table 3.6.7-2  Parts List for Fibre 4-port Adapter (Long Wavelength)

No.�Model Number�Part Name�Part No.�Quantity�Remarks��2�DKC-F310I-4GL�Fibre 2-port Adapter PCB�3254519-B�2����(Long Wavelength)�I/F Connector Panel�3250912-A�2�����Screw�SB408�8�����Label�5500248-1�2�For Fibre����Screw�SB306�4�����Bracket�5471998-1�2�����Nameplate�3252542-64�1���



3.6.7.2 Flowchart



(1)	Insert the PCBs.

(

(2)	Install the I/F connector panel.

(

(3)	Connect the optical fiber cables.

(

(4)	Attach the labels.

(

(5)	Attach the nameplate.





�3.6.7.3 Installation Procedure of Fibre 4-port Adapter



Note:	Be sure to wear your wrist strap and attach to ground prior to performing the following work.  This will ensure that the IC and LSI on the PCB are protected from static electricity.



Insertion of the PCBs

a.	Remove the dummy plate installed in the installation location referring to the Fig. 3.6.7-1.�(Note) Dummy plates should be stored for future use in De-installation.

b.	Insert the PCBs to the correct locations in the Logic Box.  Refer to Table 3.6.7-3.

c.	Fasten the two screws referring to Fig. 3.6.7-2.



Table 3.6.7-3  Inserting Location (Front of the unit)

Cluster�CL1�CL2�CL1��Slot No.�N�P�Q�R�S�T�U�V�W�X�Y�Z��Function�TM�CHA�CHA�CHA�CHA�CACHE�CACHE�CHA�CHA�CHA�CHA�TM��Location No.�TM-1N�CHA

-1P�CHA

-1Q�CHA

-1R�CHA

-1S�CACHE

-1T�CACHE

-2U�CHA

-2V�CHA

-2W�CHA

-2X�CHA

-2Y�TM-1Z��Order of addition��Basic�Add.1�Add.2�Add.3���Basic�Add.1�Add.2�Add.3���	Up to 4 Fibre 4-port adapters can be installed in the subsystem.

� EMBED Word.Picture.6  ���

Fig. 3.6.7-1  Removal of Dummy Plate	Fig. 3.6.7-2  Insertion of PCB 



�Installation of the I/F connector panel�Install the I/F connector panel in the order of the location number (((( shown in Fig. 3.6.7-3.  When the I/F connector panel is installed to the location ( in the DKC which installs the OUTLET BOX (maintenance tool), remove the OUTLET BOX referring to Fig. 3.6.7-4 before installation of the I/F connector panel. However, when a mixture of parallel channel is installed, referring to [LOCATION04-40] in a LOCATION Section.



���� EMBED Word.Picture.6  ���

Fig. 3.6.7-3 Location of I/F Connector Panel (Front)



a.	When the I/F connector panel is installed to the location (, remove the OUTLET BOX referring to Fig. 3.6.7-4.

b.	Remove the dummy plate referring to Fig. 3.6.7-5.

c.	Install the I/F connector panel referring to Fig. 3.6.7-6.�(Note): Dummy plates should be stored for future use in de-installation.

d.	When the OUTLET BOX was removed, install the OUTLET BOX to the original location after installing the I/F connector panel.



� EMBED Word.Picture.6  ���

Fig. 3.6.7-4  Removal of OUTLET BOX

� EMBED Word.Picture.6  ���

Fig. 3.6.7-5  Removal of Dummy Plate	Fig. 3.6.7-6  Installation of I/F Connector Panel



�Connection of the optical fiber cable

a.	Pull the optical fiber cable into the DKC through the I/F connector panel referring to Fig. 3.6.7-7. Connect the optical fiber cable to the PCB. Refer to Fibre Channel Interface [LOCATION04-80] in LOCATION section.

b.	Fix the cable to Lower Strain Relief( and attach Upper Strain Relief( and Bracket( referring to Fig. 3.6.7-8.



� EMBED Word.Picture.6  ���� EMBED Word.Picture.6  ���

�







�Labeling

a.	Attach the labels "FIBRE PORT" to (,(,(, or ( referring to Table. 3.6.7-4 and Fig. 3.6.7-9.



Table 3.6.7-4  Attachment of Labels

Order of addition�Basic�Add.1�Add.2�Add.3��Labeling position�(�(�(�(��Channel No.�A,B�E,F�J,K�N,P��

�



















































Fig. 3.6.7-9  Attachment of Labels





�Attachment of the nameplate

a.	Attach the nameplate referring to Fig. 3.6.7-10.



� EMBED Word.Picture.6  ���

Fig. 3.6.7-10  Attachment of Nameplate





Return to General Flow.�New Installation: Go to INST02-11 Step (6).�Non-disruptive Installation: Go to INST02-21 Step (4)-2.�Disruptive Installation: Go to INST02-40 Step (5).





	K6601527-



REV.1�Nov.1997�Apr.1998������Copyright ©1997,1998, Hitachi, Ltd.

INST03-�page �92�0



REV.1�Nov.1997�Jan.1998������Copyright ©1997,1998, Hitachi, Ltd.

INST03-�page �93�0



REV.1�Nov.1997�Oct.1998������Copyright ©1997,1998, Hitachi, Ltd.

INST03-�page �94�0



REV.0�Nov.1997�������Copyright ©1997, Hitachi, Ltd.

INST03-�page �95�0



REV.1�Nov.1997�Apr.1998������Copyright ©1997,1998, Hitachi, Ltd.

INST03-�page �96�0



REV.3�Nov.1997�Apr.1998�Jun.1998�Jan.2000����Copyright ©1997,2000, Hitachi, Ltd.

INST03-�page �97�0



REV.1�Nov.1997�Jan.1998������Copyright ©1997,1998, Hitachi, Ltd.

INST03-�page �98�0



REV.2�Nov.1997�Jun.1998�Oct.1998�����Copyright ©1997,1998, Hitachi, Ltd.

INST03-�page �99�0



REV.0�Nov.1997�������Copyright ©1997, Hitachi, Ltd.

INST03-�page �101�0



REV.1�Nov.1997�Jun.1998������Copyright ©1997,1998, Hitachi, Ltd.

INST03-�page �102�0



REV.2�Nov.1997�Apr.1998�Jun.1998�����Copyright ©1997,1998, Hitachi, Ltd.

INST03-�page �103�0



REV.2�Nov.1997�Apr.1998�Oct.1998�����Copyright ©1997,1998, Hitachi, Ltd.

INST03-�page �104�0



REV.1�Nov.1997�Jan.1998������Copyright ©1997,1998, Hitachi, Ltd.

INST03-�page �105�0



REV.1�Nov.1997�Oct.1998������Copyright ©1997,1998, Hitachi, Ltd.

INST03-�page �106�0



REV.2�Nov.1997�Apr.1998�Oct.1998�����Copyright ©1997,1998, Hitachi, Ltd.

INST03-�page �107�0



REV.0�Nov.1997�������Copyright ©1997, Hitachi, Ltd.

INST03-�page �108�0



REV.2�Nov.1997�Apr.1998�Oct.1998�����Copyright ©1997,1998, Hitachi, Ltd.

INST03-�page �109�0



REV.2�Nov.1997�Apr.1998�Mar.1999�����Copyright ©1997,1999, Hitachi, Ltd.

INST03-�page �110�0



REV.3�Nov.1997�Jan.1998�Apr.1998�Mar.1999����Copyright ©1997,1999, Hitachi, Ltd.

INST03-�page �111�0



REV.2�Nov.1997�Apr.1998�Mar.1999�����Copyright ©1997,1999, Hitachi, Ltd.

INST03-�page �112�0



REV.2�Nov.1997�Apr.1998�Mar.1999�����Copyright ©1997,1999, Hitachi, Ltd.

INST03-�page �113�0



REV.2�Nov.1997�Apr.1998�Mar.1999�����Copyright ©1997,1999, Hitachi, Ltd.

INST03-�page �114�0



REV.2�Nov.1997�Apr.1998�Mar.1999�����Copyright ©1997,1999, Hitachi, Ltd.

INST03-�page �115�0







Location at CL1



Location at CL2



(: Position for Basic installation

(: Position for Add.1 installation

(: Position for Add.2 installation

(: Position for Add.3 installation



Fig. 3.6.4-7	Connection of Optical Fiber Cable



Fig. 3.6.4-8	Installation of Optical Fiber Cable



REMOTE



         (



         (



J



K



L



M



DISABLE



ENABLE



A



          (



         (



DISABLE



ENABLE



B



DISABLE



ENABLE



C



DISABLE



ENABLE



D



         (



         (



N



P



Q



R



DISABLE



ENABLE



E



          (



         (



DISABLE



ENABLE



F



DISABLE



ENABLE



G



DISABLE



ENABLE



H



STORAGE CLUSTER 1



REMOTE



REMOTE



DISABLE



ENABLE



A



          (



         (



DISABLE



ENABLE



B



DISABLE



ENABLE



C



DISABLE



ENABLE



D



DISABLE



ENABLE



E



          (



         (



DISABLE



ENABLE



F



DISABLE



ENABLE



G



DISABLE



ENABLE



H



         (



         (



J



K



L



M



          (



         (



N



P



Q



R



STORAGE CLUSTER 2



 Labeling

 position



DKC



REMOTE



CHANNEL



CHANNEL



CHANNEL



CHANNEL



(: Position for Basic installation

(: Position for Add.1 installation

(: Position for Add.2 installation

(: Position for Add.3 installation



Location at CL1



Location at CL2



REMOTE



         (



         (



J



K



L



M



DISABLE



ENABLE



A



          (



         (



DISABLE



ENABLE



B



DISABLE



ENABLE



C



DISABLE



ENABLE



D



         (



         (



N



P



Q



R



DISABLE



ENABLE



E



          (



         (



DISABLE



ENABLE



F



DISABLE



ENABLE



G



DISABLE



ENABLE



H



STORAGE CLUSTER 1



REMOTE



REMOTE



DISABLE



ENABLE



A



          (



         (



DISABLE



ENABLE



B



DISABLE



ENABLE



C



DISABLE



ENABLE



D



DISABLE



ENABLE



E



          (



         (



DISABLE



ENABLE



F



DISABLE



ENABLE



G



DISABLE



ENABLE



H



         (



         (



J



K



L



M



          (



         (



N



P



Q



R



STORAGE CLUSTER 2



 Labeling

 position



DKC



REMOTE



CHANNEL



CHANNEL



CHANNEL



CHANNEL



(: Position for Basic installation

(: Position for Add.1 installation

(: Position for Add.2 installation

(: Position for Add.3 installation



Location at CL2



Location at CL1



Fig. 3.6.6-7	Connection of Optical Fiber Cable



Fig. 3.6.6-8	Installation of Optical Fiber Cable



REMOTE



         (



         (



J



K



L



M



DISABLE



ENABLE



A



          (



         (



DISABLE



ENABLE



B



DISABLE



ENABLE



C



DISABLE



ENABLE



D



         (



         (



N



P



Q



R



DISABLE



ENABLE



E



          (



         (



DISABLE



ENABLE



F



DISABLE



ENABLE



G



DISABLE



ENABLE



H



STORAGE CLUSTER 1



REMOTE



REMOTE



DISABLE



ENABLE



A



          (



         (



DISABLE



ENABLE



B



DISABLE



ENABLE



C



DISABLE



ENABLE



D



DISABLE



ENABLE



E



          (



         (



DISABLE



ENABLE



F



DISABLE



ENABLE



G



DISABLE



ENABLE



H



         (



         (



J



K



L



M



          (



         (



N



P



Q



R



STORAGE CLUSTER 2



 Labeling

 position



DKC



REMOTE



CHANNEL



CHANNEL



CHANNEL



CHANNEL



(: Position for Basic installation

(: Position for Add.1 installation

(: Position for Add.2 installation

(: Position for Add.3 installation



Location at CL2



Location at CL1



Fig. 3.6.7-7	Connection of Optical Fiber Cable



Fig. 3.6.7-8	Installation of Optical Fiber Cable



REMOTE



         (



         (



J



K



L



M



DISABLE



ENABLE



A



          (



         (



DISABLE



ENABLE



B



DISABLE



ENABLE



C



DISABLE



ENABLE



D



         (



         (



N



P



Q



R



DISABLE



ENABLE



E



          (



         (



DISABLE



ENABLE



F



DISABLE



ENABLE



G



DISABLE



ENABLE



H



STORAGE CLUSTER 1



REMOTE



REMOTE



DISABLE



ENABLE



A



          (



         (



DISABLE



ENABLE



B



DISABLE



ENABLE



C



DISABLE



ENABLE



D



DISABLE



ENABLE



E



          (



         (



DISABLE



ENABLE



F



DISABLE



ENABLE



G



DISABLE



ENABLE



H



         (



         (



J



K



L



M



          (



         (



N



P



Q



R



STORAGE CLUSTER 2



 Labeling

 position



DKC



REMOTE



CHANNEL



CHANNEL



CHANNEL



CHANNEL








