3.8 Installation of Redundant Power Supply



3.8.1 Flowchart



3.8.1.1 Installation of the CL Additional Power Supply (DKC-F310I-80)......[INST03-1460]



	1.	Confirm the PS Enable/Disable Switch.

(

	2.	Install the Additional Power Supply (5V and 3V PS).

(

	3.	Connect the inlet cable.

(

	4.	Turn on the Additional Power Supply.

(

	5.	Attach the nameplate.

(

	6.	Check the DC voltage.





3.8.1.2 Installation of the CL Redundant Power Supply (DKC-F310I-81).....[INST03-1500]



	1.	Confirm the PS Enable/Disable Switch.

(

	2.	Install the Redundant Power Supply (5V, 3V, and Multi PS).

(

	3.	Connect the inlet cable.

(

	4.	Turn on the Redundant Power Supply.

(

	5.	Attach the nameplate.

(

	6.	Check the DC voltage.





�3.8.1.3 Installation of the CL Additional Redundant Power Supply (DKC-F310I-82)�.....[INST03-1540]



	1.	Confirm the PS Enable/Disable Switch.

(

	2.	Install the Redundant Power Supply (5V and 3V PS).

(

	3.	Connect the inlet cable.

(

	4.	Turn on the Redundant Power Supply.

(

	5.	Attach the nameplate.

(

	6.	Check the DC voltage.





�3.8.2 Installation of the CL Additional Power Supply (DKC-F310I-80)



3.8.2.1 Parts List

Table 3.8.2.1-1  Parts List

No.�Model Number�Part Name�Part No.�Quantity�Remarks��1�DKC-F310I-80�5V power supply�5500003-A�2�HS05065C-S�������TAH-325HSA�������HS05065C-W����3V power supply�5500003-B�2�HS03075C-S�������TAH-264HSA�������HS03075C-W����Nameplate�3252542-9�1���



3.8.2.2 Installation Procedure of the CL Additional Power Supply

Confirmation of the PS Enable/Disable Switch

a.	Confirm the PS Enable/Disable Switch is turned to (DOWN) Disable.  Refer to Fig. 3.8.2.2-1.



� EMBED Word.Picture.6  ���



Fig. 3.8.2.2-1  Switch



�Installation of the additional power supply

a.	Open the locking clamps and remove the inlet cables from the inserting locations.  Refer to Table 3.8.2.2-1, Fig. 3.8.2.2-2, and Fig. 3.8.2.2-3.

b.	Loosen the screws and remove the dummy plates.

c.	Install the additional power supplies in the correct locations and fasten the screws.  Refer to Table 3.8.2.2-1, Fig. 3.8.2.2-2, and Fig. 3.8.2.2-4.

Note: Dummy plates should be stored for future use in De-installation.



Table 3.8.2.2-1  Inserting Location

No.�Part Name�PS Location��Remarks����Cluster 1�Cluster 2���1�5V Power Supply�5VPS12�5VPS22���2�3V Power Supply�3VPS12�3VPS22���

� EMBED Word.Picture.6  ���



Fig. 3.8.2.2-2  Inserting Location

� EMBED Word.Picture.6  ���� EMBED Word.Picture.6  ���

	Fig. 3.8.2.2-3  Removal of Dummy Plate	Fig. 3.8.2.2-4  Installation of Power Supply



�Connection of the inlet cable

a.	Connect the inlet cables to the power supply and fix them with the lever.  Refer to Fig. 3.8.2.2-5.



� EMBED Word.Picture.6  ���

Fig. 3.8.2.2-5  Connection of Inlet Cable



Turn on the Additional Power Supply

a.	Turn the PS Enable/Disable Switch to (UP) Enable.  Refer to Fig. 3.8.2.2-6.



� EMBED Word.Picture.6  ���



Fig. 3.8.2.2-6  Switch

�Attachment of the nameplate

a.	Attach the nameplate of DKC-F310I-80 referring to Fig. 3.8.2.2-7.



� EMBED Word.Picture.6  ���



Fig. 3.8.2.2-7  Attachment of Nameplate





Check of DC voltage

a.	See CHECK OF DC VOLTAGE in PERIODIC SECTION.



Return to General Flow.

New Installation:	Go to INST02-10 step (4A).  

Non-disruptive Installation:	Go to INST02-20 Step (2)-2.  

Disruptive Installation:	Go to INST02-40 Step (3A).  





�3.8.3 Installation of the CL Redundant Power Supply (DKC-F310I-81)



3.8.3.1 Parts List

Table 3.8.3.1-1  Parts List

No.�Model Number�Part Name�Part No.�Quantity�Remarks��1�DKC-F310I-81�5V power supply�5500003-A�2�HS05065C-S�������TAH-325HSA�������HS05065C-W����3V power supply�5500003-B�2�HS03075C-S�������TAH-264HSA�������HS03075C-W����Multi power supply�5479654-C�2�HS300-04C-R, V�����5491916-2��TSG-370HS-C�����EC5000367-A��HS400-08GR����Nameplate�3252542-10�1���



3.8.3.2 Installation Procedure of the CL Redundant Power Supply

Confirmation of the PS Enable/Disable Switch

a.	Confirm the PS Enable/Disable Switch is turned to (DOWN) Disable.  Refer to Fig. 3.8.3.2-1.



� EMBED Word.Picture.6  ���



Fig. 3.8.3.2-1  Switch



�Installation of the redundant power supply

a.	Open the locking clamps and remove the inlet cables from the inserting locations.  Refer to Table 3.8.3.2-1, Fig. 3.8.3.2-2, and Fig. 3.8.3.2-3.

b.	Loosen the screws and remove the dummy plates.

c.	Install the redundant power supplies in the correct locations and fasten the screws.  Refer to Table 3.8.3.2-1, Fig. 3.8.3.2-2, and Fig. 3.8.3.2-4.

Note: Dummy plates should be stored for future use in De-installation.



Table 3.8.3.2-1  Inserting Location

No.�Part Name�PS Location��Remarks����Cluster 1�Cluster 2���1�5V power supply�5VPS11�5VPS21���2�3V power supply�3VPS11�3VPS21���3�Multi power supply�MPS11�MPS21���

� EMBED Word.Picture.6  ���      � EMBED Word.Picture.6  ���

Fig. 3.8.3.2-2 Inserting Location

� EMBED Word.Picture.6  ���        � EMBED Word.Picture.6  ���

Fig. 3.8.3.2-3  Removal of Dummy Plate	Fig. 3.8.3.2-4  Installation of Power Supply



�Connection of the inlet cable

a.	Connect the inlet cables to the power supply and fix them with the lever.  Refer to Fig. 3.8.3.2-5.



� EMBED Word.Picture.6  ���� EMBED Word.Picture.6  ���

Fig. 3.8.3.2-5  Connection of Inlet Cable





Turn on the Redundant Power Supply

a.	Turn the PS Enable/Disable Switch to (UP) Enable.  Refer to Fig. 3.8.3.2-6.



� EMBED Word.Picture.6  ���

Fig. 3.8.3.2-6  Switch



�Attachment of the nameplate

a.	Attach the nameplate of DKC-F310I-81 referring to Fig. 3.8.3.2-7.



� EMBED Word.Picture.6  ���



Fig. 3.8.3.2-7 Attachment of Nameplate





Check of DC voltage

a.	See CHECK OF DC VOLTAGE in PERIODIC SECTION.



Return to General Flow.�New Installation: Go to INST02-10 step (4A)-2.�Non-Disruptive Installation :  Go to INST02-20 step(3)-2.�Disruptive Installation : Go to INST02-40 step(3A)-2.



�3.8.4 Installation of the CL Additional Redundant Power Supply (DKC-F310I-82)



3.8.4.1 Parts List

Table 3.8.4.1-1  Parts List

No.�Model Number�Part Name�Part No.�Quantity�Remarks��1�DKC-F310I-82�5V power supply�5500003-A�2�HS05065C-S�������TAH-325HSA�������HS05065C-W����3V power supply�5500003-B�2�HS03075C-S�������TAH-264HSA�������HS03075C-W����Nameplate�3252542-11�1���



3.8.4.2 Installation Procedure of the CL Additional Redundant Power Supply

Confirmation of the PS Enable/Disable Switch

a.	Confirm the PS Enable/Disable Switch is turned to (DOWN) Disable.  Refer to Fig. 3.8.4.2-1.



� EMBED Word.Picture.6  ���



Fig. 3.8.4.2-1  Switch



�Installation of the redundant power supply

a.	Open the locking clamps and remove the inlet cables from the inserting locations.  Refer to Table 3.8.4.2-1, Fig. 3.8.4.2-2, and Fig. 3.8.4.2-3.

b.	Loosen the screws and remove the dummy plates.

c.	Install the redundant power supplies in the correct locations.  Refer to Table 3.8.4.2-1, Fig. 3.8.4.2-2, and Fig. 3.8.4.2-4.

Note: Dummy plates should be stored for future use in De-installation.



Table 3.8.4.2-1  Inserting Location

No.�Part Name�PS Location��Remarks����Cluster 1�Cluster 2���1�5V power supply�5VPSC13�5VPSC23���2�3V power supply�3VPSC13�3VPSC23���

� EMBED Word.Picture.6  ���



Fig. 3.8.4.2-2  Inserting Location



� EMBED Word.Picture.6  ���            � EMBED Word.Picture.6  ���

Fig. 3.8.4.2-3  Removal of Dummy Plate	Fig. 3.8.4.2-4  Installation of Power Supply



�Connection of the inlet cable

a.	Connect the inlet cables to the power supply and fix them with the lever.  Refer to Fig. 3.8.4.2-5.



� EMBED Word.Picture.6  ���

Fig. 3.8.4.2-5  Connection of Inlet cable



Turn on the Redundant Power Supply

a.	Turn the PS Enable/Disable Switch to (UP) Enable.  Refer to Fig. 3.8.4.2-6.



� EMBED Word.Picture.6  ���



Fig. 3.8.4.2-6  Switch



�Attachment of the nameplate

a.	Attach the nameplate of DKC-F310I-82 referring to Fig. 3.8.4.2-7.



� EMBED Word.Picture.6  ���



Fig. 3.8.4.2-7  Attachment of Nameplate





Check of DC voltage

a.	See CHECK OF DC VOLTAGE in PERIODIC SECTION.



Return to General Flow.�New Installation: Go to INST02-10 step (5).�Non-Disruptive Installation : Go to INST02-20 step(3)-3.�Disruptive Installation : Go to INST02-40 step(4).







	K6601527-



REV.0�Nov.1997�������Copyright ©1997, Hitachi, Ltd.

INST03-�page �145�0



REV.2�Nov.1997�Aug.1999�Jan.2000�����Copyright ©1997,2000, Hitachi, Ltd.

INST03-�page �146�0



REV.0�Nov.1997�������Copyright ©1997, Hitachi, Ltd.

INST03-�page �147�0



REV.2�Nov.1997�Aug.1999�Jan.2000�����Copyright ©1997,2000, Hitachi, Ltd.

INST03-�page �148�0



REV.2�Nov.1997�Apr.1998�Oct.1998�����Copyright ©1997,1998, Hitachi, Ltd.

INST03-�page �149�0



REV.3�Nov.1997�Mar.1999�Aug.1999�Jan.2000����Copyright ©1997,2000, Hitachi, Ltd.

INST03-�page �150�0



REV.0�Nov.1997�������Copyright ©1997, Hitachi, Ltd.

INST03-�page �151�0



REV.3�Nov.1997�Mar.1999�Aug.1999�Jan.2000����Copyright ©1997,2000, Hitachi, Ltd.

INST03-�page �152�0



REV.2�Nov.1997�Apr.1998�Oct.1998�����Copyright ©1997,1998, Hitachi, Ltd.

INST03-�page �153�0



REV.2�Nov.1997�Aug.1999�Jan.2000�����Copyright ©1997,2000, Hitachi, Ltd.

INST03-�page �154�0



REV.0�Nov.1997�������Copyright ©1997, Hitachi, Ltd.

INST03-�page �155�0



REV.2�Nov.1997�Aug.1999�Jan.2000�����Copyright ©1997,2000, Hitachi, Ltd.

INST03-�page �156�0



REV.2�Nov.1997�Apr.1998�Oct.1998�����Copyright ©1997,1998, Hitachi, Ltd.

INST03-�page �157�0










