3.11 Installation of Additional Disk Adapter (DKC-F310I-50/50E)





3.11.1 Parts list





Table 3.11-1  Parts List for Additional Disk Adapter


No.�
Model Number�
Part Name�
Part No.�
Quantity�
Remarks�
�
1�
DKC-F310I-50�
Disk Adapter PCB�
1505479-A�
2�
�
�
�
�
Nameplate�
3252542-2�
1�
For DKC-F310I-50�
�
2�
DKC-F310I-50E�
Disk Adapter PCB�
1505479-B/�1505871-B�
2�
Support of HMRCF�
�
�
�
Nameplate�
3253835-1�
1�
For DKC-F310I-50E�
�






3.11.2 Flowchart





(1)	Insert the PCBs.


(


(2)	Connect the SCSI cables.


(


(3)	Bind the SCSI cables


(


(4)	Attach the nameplate.





�
3.11.3 Installation Procedure of Additional Disk Adapter





Note:	Be sure to wear your wrist strap and attach to ground prior to performing the following work.  This will ensure that the IC and LSI on the PCB are protected from static electricity.





Insertion of the PCBs


a.	Remove the dummy plate installed in the installation location referring to the Fig. 3.11-1.�(Note) Dummy plates should be stored for future use in De-installation.


b.	Insert the PCBs to the correct locations in the Logic Box.  Refer to Table 3.11-2.


c.	Fasten the two screws referring to Fig. 3.11-2.





Table 3.11-2  Inserting Location (Rear of the unit)


Cluster�
CL2�
CL1�
CL2�
�
Slot No.�
M�
L�
K�
J�
H�
G�
F�
E�
D�
C�
B�
A�
�
Function�
TM�
DKA�
DKA�
DKA�
DKA�
CACHE�
CACHE�
DKA�
DKA�
DKA�
DKA�
TM�
�
Location No.�
TM


-2M�
DKA


-2L�
DKA


-2K�
DKA


-2J�
DKA


-2H�
CACHE


-2G�
CACHE


-1F�
DKA


-1E�
DKA


-1D�
DKA


-1C�
DKA


-1B�
TM


-2A�
�
Order of addition�
�
Add.3�
Add.2�
Add.1�
Basic�
�
�
Add.3�
Add.2�
Add.1�
Basic�
�
�
	Up to 4 disk adapters can be installed in the subsystem.


� EMBED Word.Picture.6  ���





Fig. 3.11-1  Removal of Dummy Plate	Fig. 3.11-2 Insertion of PCB





�
Connection of SCSI cables


In case of Add.1


a.	Connect the SCSI cable to the sub-edge of PCB. (Refer to Fig. 3.11-3.)


� EMBED Word.Picture.6  ���


Fig.3.11-3 Connection of SCSI cable (Add.1)





In case of Add.2


a.	Connect the SCSI cable to the sub-edge of PCB. (Refer to Fig. 3.11-4.)


� EMBED Word.Picture.6  ���


Fig.3.11-4 Connection of SCSI cable (Add.2)





In case of Add.3


a.	Connect the SCSI cable to the sub-edge of PCB. (Refer to Fig. 3.11-5.)


� EMBED Word.Picture.6  ���


Fig.3.11-5 Connection of SCSI cable (Add.3)





�
Binding of the SCSI cables


In case of Add.1


a.	Bind the SCSI cables referring to Fig. 3.11-6.


� EMBED Word.Picture.6  ���





Fig.3.11-6  Binding of the SCSI cables (Add.1)





In case of Add.2


a.	Bind the SCSI cables referring to Fig. 3.11-7.


� EMBED Word.Picture.6  ���


Fig. 3.11-7  Binding of the SCSI cables (Add.2)





�
In case of Add.3


a.	Bind the SCSI cables referring to Fig. 3.11-8.


� EMBED Word.Picture.6  ���


Fig. 3.11-8  Binding of the SCSI cables (Add.3)





Attachment of the nameplate


a.	Attach the nameplate referring to Fig. 3.11-9.





� EMBED Word.Picture.6  ���


Fig. 3.11-9  Attachment of Nameplate





Return to General Flow.�New Installation: Go to INST02-11 step (7).�Non-disruptive Installation: Go to INST02-22 step (9)-2.�Disruptive Installation: Go to INST02-41 step (6).











	K6601527-











REV.2�
Nov.1997�
Jun.1998�
Aug.1999�
�
�
�
�
Copyright ©1997,1999, Hitachi, Ltd.


INST03-�page �184�0

















REV.1�
Nov.1997�
Aug.1999�
�
�
�
�
�
Copyright ©1997,1999, Hitachi, Ltd.


INST03-�page �185�0





REV.0�
Nov.1997�
�
�
�
�
�
�
Copyright ©1997, Hitachi, Ltd.


INST03-�page �186�0





REV.1�
Nov.1997�
Jun.1998�
�
�
�
�
�
Copyright ©1997,1998, Hitachi, Ltd.


INST03-�page �187�0





REV.1�
Nov.1997�
Jun.1998�
�
�
�
�
�
Copyright ©1997,1998, Hitachi, Ltd.


INST03-�page �188�0

















