9 HMRCF



9.1 Installation of HMRCF



1.	New installation of HMRCF

(1)	In case of a new system

	There is no change for installation procedure.  But DKAs should be WP102-B instead of usual DKAs.

	DKA (WP102-B) is registered as a option (DKC-F310I-50E).  WP132-B is available instead of WP102-B.



� EMBED Word.Picture.6  ���



�(2)	In case of a system already using.

	These procedure have to be done before user use HMRCF function.

NO�Item�Contents��1�Micro Program exchange�Micro program have to be upgraded first.��2�Replacement of DKAs�ALL DKAs in the Subsystem have to be replaced to DKA (WP102B *a).

Use PCB Replace procedure to replace DKA.��3�PP installation�HMRCF is a option product.  So it is necessary to install PP before using HMRCF.

It is able to install PP, if all DKAs are WP102-B.

See Remote Console manual.��*a:	DKA (WP102-B) is registered as a option (DKC-F310I-50E). WP132-B is available instead of WP102-B.



� EMBED Word.Picture.6  ���



�2.	Procedure for increment of max. pair number



(1)	In case of a new system

	It is needed that DKAs should be WP102-B instead of usual DKAs.

	And check “Use HMRCF over 64 pair” box on the Shared Memory Size Configuration panel at the end of Define Configuration and Install procedure.

	Be sure to check the shared memory capacity that showed at the Shared Memory Size Configuration panel and shared memory hardware that is on the DKC are the same.



� EMBED Word.Picture.6  ���



�(2)	In case of a system already using HMRCF

	In this case SM incrementation is needed. Follow the flowchart on the next page.

	Shared memory capacity have to be increased to store HMRCF control information for 512 pairs.

	<<Caution>>	Shared Memory have to be SH100-B(64MB).

			Because capacity of SM is over 192 MB X 2 after installation.



NO�Item�Contents��1�Micro Program exchange�Micro program have to be upgraded first.��2�Installation of Shared Memory�Increment Shared Memory capacity.

If there is SH024(16MB), it have to be exchange to SH100-B (64MB) before SM installation.��3�Create additional �control table�Push the “Set over 64” button of HMRCF screen on SVP.��

�

� EMBED Word.Picture.6  ���

�

� EMBED Word.Picture.6  ���



	K6601527-



REV.0�Oct.1998�������Copyright ©1998, Hitachi, Ltd.

INST09-�page �1�0



REV.1�Oct.1998�Mar.1999������Copyright ©1998,1999, Hitachi, Ltd.

INST09-�page �2�0



REV.0�Oct.1998�������Copyright ©1998, Hitachi, Ltd.

INST09-�page �3�0



REV.0�Oct.1998�������Copyright ©1998, Hitachi, Ltd.

INST09-�page �4�0



REV.0�Oct.1998�������Copyright ©1998, Hitachi, Ltd.

INST09-�page �5�0



REV.0�Oct.1998�������Copyright ©1998, Hitachi, Ltd.

INST09-�page �6�0










