OP Panel-2





1.	Memorize the current setting of Enable/Remote/Disable Switches on the OP Panel-2.





2.	Set the switches of the spare PCB to the proper position.


a.	If Enable/Disable LED is on, set the Enable/Remote/Disable Switch to Enable position.


b.	If Enable/Disable LED is off, set the Enable/Remote/Disable Switch to Disable position.





3.	Remove the PCB.


a.	Loosen the two screws and remove the plate.


b.	Set the jumper socket(JP1) of the failed PCB referring to following figure.


c.	Disconnect the cables.





� EMBED Word.Picture.6  ���


Fig. T2-6  Removal of Plate





�
d.	Loosen the two screws and remove the cover from the Movable Rack.





� EMBED Word.Picture.6  ���


Fig. T2-7  Removal of Cover





e.	Loosen the two screws and remove the OP Panel-2.





� EMBED Word.Picture.6  ���


Fig. T2-8  Removal of OP Panel-2








�
4.	Remove the plates from the failed PCB, and then attach them to the spare PCB.


a.	Remove the four screws and the plates from the failed PCB.


b.	Attach the plates to the spare PCB and fasten the four screws.


� EMBED Word.Picture.6  ���


							Fig. T2-9  Replacement of PCB





5.	Attach the PCB.


a.	Attach the spare PCB and fasten the two screws.  Refer to Fig. T2-8.


b.	Attach the cover to the Movable Rack and fasten the two screws.  Refer to Fig. T2-7.


c.	Set the jumper socket(JP1) referring to Fig. a.


d.	Connect the cables.


e.	Set the jumper socket(JP1) referring to Fig. b.


f.	Attach the plate and fasten the two screws.





� EMBED Word.Picture.6  ���


Fig. T2-10  Attachment of Plate





6.	Go to SVP post procedure t1 [REP04-490].


	K6601527-





REV.0�
Nov.1997�
�
�
�
�
�
�
Copyright (1997, Hitachi, Ltd.


REP03-�page �109�0











