[HARDWARE T7]














Location�
Function Name of Component�
Part Name�
�
Lower left


front of DKC�
1�
RCU CONN


�
 SZ625-A�
�












� EMBED Word.Picture.6  ���








Notice: Be sure to wear your wrist strap and attach to ground prior to performing the following work.  This will ensure that the IC and LSI on the PCB are protected from static electricity.





�
1. Remove the RCU CONN.


a. Disconnect all cables from the RCU CONN.


b. Loosen the screw( and remove the plate.


c. Remove the screw( and remove the RCU CONN.





� EMBED Word.Picture.6  ���





2. Remove the plate from the failed PCB, and then attach them to spare PCB.


a. Remove the four screws and the plate from the failed PCB.


b. Attach the plate to the spare PCB and fasten the four screws.





� EMBED Word.Picture.6  ���





3. Attach the PCB.


a. Attach the RCU CONN and fasten the screw(.


b. Attach the plate and fasten the screw(.


c. Connect the cables(P70, 71).





4. Go to SVP post procedure t1 [REP04-490].





	K6601527-





REV.0�
Nov.1997�
�
�
�
�
�
�
Copyright (1997, Hitachi, Ltd.


REP03-�page �127�0











