[HARDWARE T16]














Location�
Function Name of Component�
�
Lower front 


of DKC�
1�
 Outlet Box


�
�
(Reference)


The related PCB for replacement of Outlet Box


1. Breaker Box-1 (Front PS Box in DKC)�
�












� EMBED Word.Picture.6  ���





�
1. Circuit Breaker (Off)





� EMBED Word.Picture.6  ���


Be Careful of Electric Shock


  �  The power to the device is still on after turning off the breakers shown below.


  �  The device may be powered off when turning off the breakers not shown below.








� EMBED Word.Picture.6  ���


Mind Your Head


  �  Be sure to mind your head during operation, or you might hit your head against the movable rack, which could cause injury.








a.	Turn off the circuit breakers in the DKC.





		( Circuit Breakers to be Turned Off When Replacing Outlet Box


No.�
Unit�
Location No.�
Breaker No.�
Remarks�
�
1�
DKC�
Outlet Box�
CB301�
Failure to turn off may result in an electric shock.�
�
2�
DKC�
Outlet Box�
CB302�
Failure to turn off may result in an electric shock.


Only IU Type �
�






� EMBED Word.Picture.6  ���





�
2. Outlet Box Removal





a. Remove the Front PS Box Cover 2 from the Front PS Box.


b. Detach the power cord set(s) from the plug socket(s) on the Outlet Box.





� EMBED Word.Picture.6  ���








�
c. Detach the OLB power cable (P203) from Breaker Box-1 





� EMBED Word.Picture.6  ���


Be Careful of Electric Shock


  �	Be sure to turn off the circuit breakers (CB301 and CB302) on Outlet Box before operation.


  �	The connector J203 on Breaker Box-1 is a live part, so be sure to keep tools and parts such as a screwdriver and screw from contact with the connector.


  �	The connector J203 on Breaker Box-1 is a live part, so be sure to keep tools and parts such as a screwdriver and screw from contact with the connector.





� EMBED Word.Picture.6  ���





d. Remove the cover and draw the OLB power cable(P203).


� EMBED Word.Picture.6  ���





�
e. Detach the Outlet Box frame ground wire from the DKC frame.  Remove the two screws that fasten the Outlet Box.





� EMBED Word.Picture.6  ���








f. Remove the Outlet Box.





� EMBED Word.Picture.6  ���





�
3. Outlet Box Attachment








a. Detach the jumper connectors (P311, P312) from the failed Outlet Box, then attach the jumper connectors (P311, P312) to spare Outlet Box.(Only for IU type)





b. Turn off the circuit breaker(s) (CB301, CB302) on the spare Outlet Box.





� EMBED Word.Picture.6  ���








c. Insert the Outlet Box to the stopper position located at the bottom of the DKC, then fix the Outlet Box with two screws.





� EMBED Word.Picture.6  ���    � EMBED Word.Picture.6  ���


  








�
d.	Attach the frame ground wire to the DKC frame, using the screw and lock washer.





� EMBED Word.Picture.6  ���








e.	Pass the OLB power cable(P203) through the gap in the plate and attach the cover.


� EMBED Word.Picture.6  ���





�



f.	Insert the Outlet Box connector as it was.


	Attach the OLB power cable (P203) .


	Connect the OLB power cable (P203) to Breaker Box-1.





� EMBED Word.Picture.6  ���


Be Careful of Electric Shock


  �	Be sure to turn off the circuit breakers (CB301 and CB302) on Outlet Box before operation.


  �	The connector J203 on Breaker Box-1 is a live part, so be sure to keep tools and parts such as a screwdriver and screw from contact with the connector.


  �	Be sure to connect or disconnect the power cable P203 for the Outlet Box with one hand. This can prevent an electric current from flowing between your arms if you touch the live part of the J203.





Location No.�
Cable No.�
Connector No.�
Remarks�
�
Breaker Box-1�
P203�
J203�
OLB Power Cable�
�



� EMBED Word.Picture.6  ���





�
g. Attach the power cord set(s) to the Plug Socket(s) on the Outlet Box.





� EMBED Word.Picture.6  ���








h. Attach the Front PS Box Cover 2 to Front PS Box. 





�
4. Circuit Breaker (On)





a. Turn on the circuit breakers in the DKC.





� EMBED Word.Picture.6  ���








				( Circuit Breakers to be Turned On After Replacing Outlet Box


No.�
Unit�
Location No.�
Breaker No.�
Remarks�
�
1�
Disk Controller�
Outlet Box�
CB301�
�
�
2�
Disk Controller�
Outlet Box�
CB302�
Only IU Type �
�






5. Go to SVP post-procedure t1 [REP04-490].





	K6601527-





REV.0�
Nov.1997�
�
�
�
�
�
�
Copyright (1997, Hitachi, Ltd.


REP03-�page �171�0











