





















SVP SECTION



�Contents



SVP 	SVP01-10

1 How to Operate the SVP (PC) 	SVP01-10

1.1 How to use a trackball or touchpad 	SVP01-10

1.2 Windows Screen Component Nomenclature 	SVP01-20

1.3 How to use Windows 	SVP01-30

1.4 Power On 	SVP01-50

1.5 Power Off 	SVP01-60

1.6 Mode 	SVP01-70

1.7 Run 	SVP01-80

1.8 Screen Saver, and SVP reboot function 	SVP01-90



2 Function of the SVP 	SVP02-10

2.1 TOD (Time Or Date) setting 	SVP02-10

2.2 Log indication 	SVP02-30

2.3 Log delete 	SVP02-150

2.4 Monitoring 	SVP02-170



2.6 Online read margin (ORM) 	SVP02-250

2.7 SIM Reporting Specification 	SVP02-370

2.8 Management of drive threshold values 	SVP02-390

2.9 DUMP/LOG FD Copy 	SVP02-460

2.10 SIM Log Complete 	SVP02-500

2.11 Dump 	SVP02-530

2.12 Logical Device Maintenance 	SVP02-580

2.12.1 Logical Device 	SVP02-580

2.12.2 Open VOL 	SVP02-706

2.13 Pin Data indication 	SVP02-710

2.14 Multi PCB Replace 	SVP02-730



2.16 System Option 	SVP02-760

2.17 Recovery Procedure for Bus 	SVP02-790

2.18 Blocking of Cluster 	SVP02-860

2.19 Recovering of Cluster 	SVP02-890

2.20 Parallel Configuration 	SVP02-930



2.22 PCB Revision Display 	SVP02-950

2.23 HRC&HODM Operation 	SVP02-970

2.23.1 Referring to the Path Status 	SVP02-970

2.23.2 Referring to the Pair Status 	SVP02-990

2.23.3 Suspending the Volume Pair(s) 	SVP02-1000

2.23.4 Resuming the Suspended Pair(s) 	SVP02-1020

2.23.5 Deleting the Existing HRC Pair(s) 	SVP02-1040

2.23.6 Deleting the Existing HODM Pair(s) 	SVP02-1060

2.23.7 Deleting an RCU Registered 	SVP02-1080

2.23.8 Adding the Path(s) to the RCU 	SVP02-1090



�2.23.9 Reducing the Path(s) from the RCU 	SVP02-1110

2.23.10 Changing the Port Type 	SVP02-1130

2.23.11 Displaying HRC/HODM Operation Status 	SVP02-1150

2.23.12 Adding the SSID(s) to the RCU 	SVP02-1170

2.23.13 Reducing the SSID(s) from the RCU 	SVP02-1190

2.23.14 HODM Snapshot 	SVP02-1192

2.23.15 Referring the C/T group status 	SVP02-1194

2.23.16 Releasing the using C/T group 	SVP02-1195

2.23.17 Changing the using C/T group parameters 	SVP02-1196

2.23.18 Changing the asynchronous option parameters 	SVP02-1197

2.23.19 Clear reported SIM 	SVP02-1198

2.24 Tuning Parameter 	SVP02-1200

2.25 Change SM Module group size 	SVP02-1210

2.26 Change CM Module group size 	SVP02-1280

2.27 HORC Operation (for OPEN VOL) 	SVP02-1350

2.27.1 Referring to the Path Status 	SVP02-1350

2.27.2 Referring to the Pairdisplay of OPEN VOL (open volume) 	SVP02-1370

2.27.3 Referring to the Pairdisplay of LDEV 	SVP02-1371

2.27.4 Suspending the Volume Pair(s) of OPEN VOL (open volume) 	SVP02-1380

2.27.5 Resuming the Suspended Pair(s) of OPEN VOL (open volume) 	SVP02-1400

2.27.6 Resuming the Suspended Pair(s) of LDEV 	SVP02-1411

2.27.7 Deleting the Existing HORC Pair(s) 	SVP02-1420

2.27.8 Deleting an RCU Registered 	SVP02-1440

2.27.9 Adding the Path(s) to the RCU 	SVP02-1450

2.27.10 Reducing the Path(s) from the RCU 	SVP02-1470

2.27.11 Changing the Port Type 	SVP02-1490

2.27.12 Displaying HORC Operation Status 	SVP02-1510

2.27.13 Adding the SSID(s) to the RCU 	SVP02-1530

2.27.14 Reducing the SSID(s) from the RCU 	SVP02-1550

2.27.15 Referring the C/T group status 	SVP02-1552

2.27.16 Releasing the using C/T group 	SVP02-1553

2.27.17 Changing the using C/T group parameters 	SVP02-1554

2.27.18 Changing the asynchronous option parameters 	SVP02-1555

2.28 HORC Operation (for LDEV) 	SVP02-1560

2.28.1 Referring to the Path Status 	SVP02-1560

2.28.2 Referring to the Pair Status 	SVP02-1580

2.28.3 Suspending the Volume Pair(s) 	SVP02-1590

2.28.4 Resuming the Suspended Pair(s) 	SVP02-1610

2.28.5 Deleting the Existing HORC Pair(s) 	SVP02-1630

2.28.6 Deleting an RCU Registered 	SVP02-1650

2.28.7 Adding the Path(s) to the RCU 	SVP02-1660

2.28.8 Reducing the Path(s) from the RCU 	SVP02-1680

2.28.9 Changing the Port Type 	SVP02-1700

2.28.10 Displaying HORC Operation Status 	SVP02-1720

2.28.11 Adding the SSID(s) to the RCU 	SVP02-1740

2.28.12 Reducing the SSID(s) from the RCU 	SVP02-1760



�3. Activating and Terminating STATUS 	SVP03-10

3.1 Activating STATUS 	SVP03-10

3.2 Terminating STATUS 	SVP03-30

3.3 Updating the STATUS display 	SVP03-40

3.4 Main screen 	SVP03-40

3.5 DKC 	SVP03-50

3.6 DKU 	SVP03-100

3.7 Copy status 	SVP03-120

3.8 Logical device 	SVP03-120

3.9 Micro-program version 	SVP03-130

3.10 LCP/MCP Path 	SVP03-180

3.11 Pin 	SVP03-190

3.12 SCSI Path 	SVP03-200

3.13 Error or Failure Status Action 	SVP03-240



4 References for HRC&HODM Operation Screen 	SVP04-10

4.1 Main screen 	SVP04-10

4.1.1 HRC Main screen 	SVP04-10

4.1.2 HODM Main screen 	SVP04-41

4.2 Port... screen 	SVP04-50

4.3 Add RCU... screen 	SVP04-60

4.4 Edit Path/SSID 	SVP04-100

4.4.1 Path/SSID Edit... screen 	SVP04-100

4.4.2 Edit Path... screen 	SVP04-100

4.4.3 Edit SSID... screen 	SVP04-110

4.4.4 Add SSID... screen 	SVP04-110

4.5 RCU Option... screen 	SVP04-120

4.6 RCU Status... screen 	SVP04-130

4.7 Add Pair... screen 	SVP04-150

4.8 Add Pair... screen(HODM) 	SVP04-160

4.9 Delete Pair... screen 	SVP04-170

4.10 Pair Option... screen 	SVP04-180

4.11 Pair Status... screen 	SVP04-190

4.11.1 Pair Status... screen (HRC) 	SVP04-190

4.11.2 Pair Status... screen (HODM) 	SVP04-205

4.12 Suspend Pair... screen 	SVP04-210

4.13 Usage... screen 	SVP04-220

4.14 Add Group/Group Option... screen 	SVP04-250

4.15 Group Status... screen 	SVP04-260

4.16 Async Option... screen 	SVP04-270



5 Option Install 	SVP05-10



6 Display of  Disk Drive Mode 	SVP06-10



�7 References for HORC Operation Screen (for OPEN VOL) 	SVP07-10

7.1 HORC Main screen 	SVP07-10

7.2 HORC Sub Main screen 	SVP07-41

7.3 Port...screen 	SVP07-50

7.4 Add RCU... screen 	SVP07-60

7.5 Edit Path/SSID 	SVP07-80

7.5.1 Path/SSID Edit... screen 	SVP07-80

7.5.2 Edit Path... screen 	SVP07-80

7.5.3 Edit SSID... screen 	SVP07-90

7.5.4 Add SSID... screen 	SVP07-90

7.6 RCU Option... screen 	SVP07-100

7.7 RCU Status... screen 	SVP07-110

7.8 Paircreate... screen 	SVP07-130

7.9 Pairsplit-S... screen 	SVP07-140

7.10 Pair Option... screen 	SVP07-150

7.11 Pairdisplay... screen (of OPEN VOL) 	SVP07-160

7.12 Pairdisplay... screen (of LDEV) 	SVP07-172

7.13 Pairsplit-r... screen 	SVP07-180

7.14 Usage... screen 	SVP07-190

7.15 Add Group/Group Option... screen 	SVP07-220

7.16 Group status... screen 	SVP07-230

7.17 Async Option... screen 	SVP07-240



8 Details of the HMRCF Operation Screen 	SVP08-10

8.1 Details for Screen 	SVP08-90



9 References for HORC Operation Screen (for LDEV) 	SVP09-10

9.1 HORC Main screen 	SVP09-10

9.2 Port...screen 	SVP09-50

9.3 Add RCU... screen 	SVP09-60

9.4 Edit Path/SSID	SVP09-80

9.4.1 Path/SSID Edit... screen 	SVP09-80

9.4.2 Edit Path... screen 	SVP09-80

9.4.3 Edit SSID... screen 	SVP09-90

9.4.4 Add SSID... screen 	SVP09-90

9.5 RCU Option... screen 	SVP09-100

9.6 RCU Status... screen 	SVP09-110

9.7 Add Pair... screen 	SVP09-130

9.8 Delete Pair... screen 	SVP09-140

9.9 Pair Option... screen 	SVP09-150

9.10 Pair Status... screen 	SVP09-160

9.11 Suspend Pair... screen 	SVP09-180

9.12 Usage... screen 	SVP09-190



10 Details of the HOMRCF Operation Screen 	SVP10-10

10.1 Operation of HOMRCF 	SVP10-10

10.2 Procedure for confirming the volume status 	SVP10-220

10.3 Notice about HOMRCF operation 	SVP10-350



�11 HIHSM 	SVP11-10

11.1 HIHSM Setting Procedures 	SVP11-10

11.1.1 Basic Operations 	SVP11-10

11.2 HIHSM Screens 	SVP11-30

11.2.1 Main Screen 	SVP11-30

11.2.2 Manual Migration Main Screen 	SVP11-50

11.2.3 Automatic Migration Main Screen 	SVP11-70

11.2.4 Automatic Migration Setting Screen	SVP11-90

11.2.5 Data Gathering Screen 	SVP11-110

11.2.6 Data Term Screen 	SVP11-110

11.2.7 File Operation Screen 	SVP11-110

11.2.8 Change Attribute Screen 	SVP11-120

11.2.9 History Screen 	SVP11-120

11.2.10 Automatic Migration History Screen	SVP11-130

11.3 Start HIHSM	SVP11-180

11.4 Monitor the data 	SVP11-190

11.4.1 Start monitoring 	SVP11-190

11.4.2 Stop monitoring 	SVP11-200

11.4.3 Set the term to display the monitoring data 	SVP11-210

11.4.4 Save monitoring data to FD 	SVP11-220

11.5 Changing the volume attribution	SVP11-230

11.5.1 Changing the volume attribution to “Reserved” 	SVP11-230

11.5.2 Changing the volume attribute to “Normal” 	SVP11-240

11.6 Manual Migration 	SVP11-250

11.6.1 Manual Migration 	SVP11-250

11.6.2 Migration Status 	SVP11-270

11.6.3 Stop migration 	SVP11-280

11.7 Automatic Migration 	SVP11-290

11.7.1 Set/Reset the Fixed parity group 	SVP11-290

11.7.2 Set the evaluating schedule and migration time 	SVP11-300

11.7.3 Make the migration plan 	SVP11-310

11.7.4 Set the evaluating schedule and migration time	SVP11-350

11.7.5 Make the migration plan	SVP11-380

11.8 Display the volume usage rate 	SVP11-390

11.9 Display the history 	SVP11-400

11.9.1 History Screen	SVP11-400

11.9.2 Display the history for automatic migration function	SVP11-410



�1 How to Operate the SVP (PC)



This manual describes how to operate the SVP (PC) with a trackball.





1.1 How to use a trackball or touchpad

a)	If SVP model is Armada

�EMBED Word.Picture.6���

( : Trackball

	Rotate this device to move the pointer to a desired position.

( : Button

	Pressing this button selects an item on which the pointer is placed.

( : Not used



b)	If SVP model is Flora

� EMBED Word.Picture.6  ���

( : Touchpad

	Trace this device to move the pointer to a desired position.

( : Button

	Pressing this button selects an item on which the pointer is placed.

( : Not used



�1.2 Windows Screen Component Nomenclature



Either of the following windows is displayed.



�

����������











(Note)

Each SVP screen on this maintenance manual is a sample, and it may not be the same as the actual screen.



�1.3 How to use Windows



(1)	Notation

	In this manual, “select” has the following three meanings, and (CL), (DC), or (DR) is added to the word for each meaning.



	(CL) Click:

	Quickly press and release the button above the trackball.

	(DC) Double-click:

	Click the button above the trackball or touchpad twice in rapid succession.

	(DR) Drag:

	To hold down the button above the trackball or touchpad while you rotate the trackball or trace the touchpad to move the pointer to a desired position.  Then release the button.



example)

Select (DC) the [Install] icon in the ‘SVP’ window.

	Move the pointer to [Install] with the trackball or touchpad.  Then click the button above the trackball or touchpad twice in rapid succession.







(2)	Close

	“Close” means to close the aplication window.

	(Double-click the control menu box of the window or click the close button for Windows 95.)

�





�	�





�(3)	Moving the Window

	Move the pointer to the title bar with the trackball.

	While pressing the button, move the window with the trackball or touchpad (DR) to a desired position and release the button.



� EMBED Word.Picture.6  ���







(4)	Changing the window size

	Move the pointer to the window border (frame) (the pointer changes to the double- headed arrow).  While pressing the button, move the border (the border changes to the broken line) until the window becomes a desired size, and release the button.



� EMBED Word.Picture.6  ���







(5)	Switching the screen (when two or more screens are opened)



	While pressing the �eq \x(ALT)� key , press �eq \x(TAB)� (or �eq \x(ESC)� ) until your desired window title is displayed, and release the �eq \x(ALT)� key.



�1.4 Power On

Usually, SVP start automatically at the breakers-ON.

If some problem occurred and you must start SVP, do as following procedures.



a)	If SVP model is Armada

�EMBED PBrush���

�(1)	Power On SVP

 a.	Press Power Switch on the SVP keyboard.

 b.	Watch messages displayed on the SVP screen.

	If following messages are displayed, Go to (2).

	Otherwise, power off (press Alt + Ctrl + Power

	Switch) and on SVP to retry. (*1)

	After retrying twice, replace SVP.



[Message]

	xxxx KB OK

	Starting Windows95

	(Some other messages)



(2)	Windows Start (SVP Start)

 a.	Wait a few minutes until Windows system will start.

 b.	Read message displayed on the SVP screen. (Go to SVPMSG section)

	Otherwise, power off (press Alt + Ctrl + Power Switch) and on SVP to retry. (*1)

	After retrying twice, replace SVP.

	[End of Power On]





*1:	If you press only Power Switch on the SVP keyboard when SVP and Windows system are active, these systems will be frozen.

	You should be sure to press Alt + Ctrl + Power Switch for power off SVP.



*2:	If Windows 95 doesn’t start, check the following items.

	(1)	Is the DKC “CE mode” ?

	(2)	The two LEDs at the LAN cable socket are always on?



If above two conditions are satisfied, pull out LAN cable until Windows 95 is starting.



�b)	If SVP model is Flora

(1)	Power On SVP

 a.	Insert the Shut Down Jumpers in SVPPS1 and SVPPS2 (See LOCATION06-20).

 b.	Wait for one minute.

 c.	Remove the Shut Down Jumpers in SVPPS1 and SVPPS2 (See LOCATION06-20).

 d.	Watch messages displayed on the SVP screen.

	If following messages are displayed, Go to (2)

	Otherwise, Go to a, b and c.

	After retrying twice, replace SVP.



[Message]

	xxxx KB OK

	Starting Windows95

	(Some other messages)



(2)	Windows Start (SVP Start)

 a.	Wait a few minutes until Windows system will start.

 b.	Read message displayed on the SVP screen. (Go to SVPMSG section)

	Otherwise, Go to (1) a, (1) b and (1) c.

	After retrying twice, replace SVP.

	[End of Power On]







�1.5 Power Off

a)	If SVP model is Armada

(1)	What is running? 

�� EMBED Word.Picture.6  ���

 a.	See what is displayed.

	If Windows is displayed, Go to (2).

	If DOS prompt is displayed, like “C:\>”, Go to (3).



(2)	Exit Windows(Stop SVP)

 a.	Select (CL) “Start button”.

 b.	Select (CL) “Shut Down”.

 c.	Select (CL) “Shut Down the computer?”, and �	Select (CL) [Yes].

	[End of Power Off]



(3)	Power Off SVP

 a.	Press 2 keys ––– Ctrl-key and Alt-key –––, and Power Switch simultaneously.

	[End of Power Off]



Note :	Do not press only Power Switch key.�Because, since SVP does not stop to perfection, we do not guarantee the operation of SVP after re-power on.�If only Power Switch key was pressed, please power off SVP.





b)	If SVP model is Flora

(1)	What is running? 

a.	See what is displayed.

	If Windows is displayed, Go to (2).

	If DOS prompt is displayed, like “C:\>”, Go to (3).



(2)	Exit Windows(Stop SVP)

 a.	Select (CL) “Start button”.

 b.	Select (CL) “Shut Down”.

 c.	Select (CL) “Shut Down the computer?”, and �	Select (CL) [Yes].

	[End of Power Off]



(3)	Power Off SVP

 a.	If replace of SVP

	Insert the Shut down Jumpers in SVPPS1 and SVPPS2 (See LOCATION06-20).

	Wait for one minute.

	Replace SVP.

	Remove the Shut Down Jumpers in SVPPS1 and SVPPS2 (See LOCATION06-20).

 b.	If reboot of SVP

	Insert the Shut Down Jumpers in SVPPS1 and SVPPS2 (See LOCATION06-20).

	Wait for one minute.

	Remove the Shut Down Jumpers in SVPPS1 and SVPPS2 (See LOCATION06-20).

�1.6 Mode



(1)	View Mode

�

	In view mode, only referring the subsystem status can be allowed.



















(2)	Modify Mode

	In modify mode, referring and changing the subsystem status can be allowed.



�

	For example, log/pin data indication and status display on MAINTENANCE are available in any mode, but replacement is available in only modify mode.

















	Note : When view mode, pending SIMs (if exist) are reported to host.



(3)	Change Mode

	If you push (CL) [View Mode] button, it changes [Modify Mode], and SVP changes Modify Mode.

	If you push (CL) [Modify Mode] button, it changes [View Mode], and SVP changes View Mode.









�1.7 Run



(1)	<Select the [Run]>

	Select (CL) the [Run...] in the [Start button].



�





(2)	<Input file name>

	Input file name in ‘Open’ and select (CL) [OK].



�



�1.8 Screen Saver, and SVP reboot function



This function executes SVP reboot at intervals of a certain duration, one time per day.

Normally, the function displays a new windows for Screen Saver.



(1)	SVP (PC) reboot

	When SVP detects the reboot time under displaying the Screen Saver, the function will reboot SVP automatically.  But, if other applications of SVP are executing, the function will stop rebooting, and wait until the next time will arrive.















(2)	Screen Saver

 1.	If there is no access from keyboard or mouse during 60 minutes, and other SVP applications windows are closed, SVP will open the screen saver window.

�

































 2.	Normally, the function becomes icon.

�

















�(3)	Setting reboot timer

 1.	Please select (DC) “PC Watch” Icon.

� EMBED Word.Picture.6  ���

















 2.	SVP will displays “Input Password” screen.

�

	Please enter the password and select (CL) �“OK”.















 3.	Next, SVP will displays “Set Reboot Parameter” Screen.

	Please input Reboot interval and Reboot time, and select (CL) “OK”.

�



 (	Reboot Interval

	Every Day : one time per day.

	Not Reboot : not available to reboot.

 (	Reboot Time

	Reboot time will be set.

	Notice:

	The SVP usually receives a TH/ORM log �data from DKC-Main at 23:30 - 0:30, so �that you can not input those time.



�

	If you input the reboot time as 24:00 or above �timer 23:30 - 0:30, will move the focus.



















	K6601527-



REV.1�Nov.1997�Apr.1998������Copyright ©1997,1998, Hitachi, Ltd.

SVP00-� PAGE �0�0



REV.3�Nov.1997�Jan.1998�Apr.1998�Jun.1998����Copyright ©1997,1998, Hitachi, Ltd.

SVP00-� PAGE �1�0



REV.6�Apr.1998�Jun.1998�Oct.1998�Mar.1999�Aug.1999�Jan.2000��Copyright ©1998,2000, Hitachi, Ltd.

SVP00-� PAGE �2�0



REV.4�Jun.1998�Oct.1998�Mar.1999�Aug.1999�Jan.2000���Copyright ©1998,2000, Hitachi, Ltd.

SVP00-� PAGE �3�0



REV.3�Oct.1998�Mar.1999�Aug.1999�Jan.2000����Copyright ©1998,2000, Hitachi, Ltd.

SVP00-� PAGE �4�0



REV.1�Aug.1999�Jan.2000������Copyright ©1999,2000, Hitachi, Ltd.

SVP00-� PAGE �5�0



REV.1�Nov.1997�Apr.1998������Copyright ©1997,1998, Hitachi, Ltd.

SVP01-� PAGE �1�0



REV.2�Nov.1997�Apr.1998�Mar.1999�����Copyright ©1997,1999, Hitachi, Ltd.

SVP01-� PAGE �2�0



REV.1�Nov.1997�Apr.1998������Copyright ©1997,1998, Hitachi, Ltd.

SVP01-� PAGE �3�0



REV.2�Nov.1997�Jan.1998�Apr.1998�����Copyright ©1997,1998, Hitachi, Ltd.

SVP01-� PAGE �5�0



REV.0�Apr.1998�������Copyright ©1998, Hitachi, Ltd.

SVP01-� PAGE �55�



REV.2�Nov.1997�Jan.1998�Apr.1998�����Copyright ©1997,1998, Hitachi, Ltd.

SVP01-� PAGE �6�0



REV.2�Nov.1997�Jan.1998�Mar.1999�����Copyright ©1997,1999, Hitachi, Ltd.

SVP01-� PAGE �7�0



REV.0�Nov.1997�������Copyright ©1997, Hitachi, Ltd.

SVP01-� PAGE �9�0



REV.1�Nov.1997�Jan.1998������Copyright ©1997,1998, Hitachi, Ltd.

SVP01-� PAGE �10�0





Desktop



Mode button



Title bar



Menu bar



Window (entire box)



Start button



Task bar



Border (frame)



Control menu box



Start button

(for SVP application)



Control menu box



Close button



Power Switch



Alt-key



Ctrl-key



Power Switch



Alt-key



Ctrl-key



Delete-key








