








[PRE-PROCEDURE A]














— OUTLINE —





( Initial Screen.


( Change SVP operation mode.


( Open Maintenance window.





�
1.	<Initial screen>
































2.	<Operation mode change>


	Change the mode to [Modify Mode].


	Select [Maintenance] (CL).


























3.	<Maintenance window>


�


	‘Maintenance’ window is displayed.





























�









[PRE-PROCEDURE T3]














— OUTLINE —





( Select special (DKC) part (status check).


( Specify Replacement.


( Detach parts related to special part.


( Place parts related to special part into unpluggable state.





�
[1] Select special part





1.	<Maintenance window>


�


	In the ‘Maintenance’ window, check and select (CL) �DKC to be replaced.


























2.	<DKC window>


�


	Select (CL) [Cluster-n] in the ‘DKC’.





























3.	<Select special part>


�


	Select (CL) part [XXXX] to be replaced from [Cluster-n] �window and select (CL) [Execute].


	Valid [XXXX] values are listed below.





	• Breaker Box	[Breaker Box-n] 


	• DKC AC BOX	[AC BOX-Cn]





Go to [2] (REP02T-50)





�





























�
[2] Breaker Box, DKC AC BOX





1.


(	When redundant power supply is installed, 


�


	select (CL) [Yes].


	Go to 5.

















(	Otherwise


  (	When only serial or parallel channels, or they are mixed.


�


	“Please confirm you have already varied off the concerned channel path(s).  Do you want to replace the “XXXX?” ” is displayed.


	Select (CL) [Yes].











  (	When SCSI channels and other kinds are mixed.


�


	“Please confirm you have already varied off the concerned channel path(s).  Please confirm you have already shutdown concerned connecting SCSI host(s) or switched to the alternate SCSI channel path(s).  Do you want to replace the “XXXX?” ” is displayed. 


 Select (CL)[Yes].











  (	When only SCSI channels are installed.


�


	“Please confirm you have already shutdown concerned connecting SCSI host(s) or switched to the alternate SCSI channel path(s).  Do you want to replace the “XXXX?” ” is displayed. 


 Select (CL) [Yes].

















�
2.


	The SVP automatically checks the power supplies to see if Breaker Box is replaceable.


























3.	<Block cluster>


�














	Blocking cluster has finished normally, go to next step.




















�
4.


�


	In the case of Breaker Box replacement, SVP displays “Installed AC BOX is 1-Phase type.  Turn off the DKC AC BOX which supplies power source for XXXXX, when you replace it.” Message. Select (CL) [OK].











�


	The message shown on the right is displayed.


	(Reply with [OK] after replacing the special �part.)


	Go to [3] (REP02T-100).























�
5.


	The SVP automatically checks the power supplies to see if Breaker Box is replaceable.





























6.	<Check environment monitor stopped state>


�


	“Processing to disable the environment check...” is displayed.





























�
7.


�


	In the case of Breaker Box replacement, SVP displays “Installed AC BOX is 1-Phase type.  Turn off the DKC AC BOX which supplies power source for XXXXX, when you replace it.” Message.  Select (CL) [OK].











�


	The message shown on the right is displayed.


	(Reply with [OK] after replacing the special part.)


	Go to [3] (REP02T-100).























�
[3] Replacement





1.	<Replace special part>


	Replace the special part.








	Breaker Box-1	see HARDWARE TUV12	(REP03T-230)


	Breaker Box-2	see HARDWARE TUV13	(REP03T-320)


	AC BOX-C1	see HARDWARE UV1	(REP03T-10)


	AC BOX-C2	see HARDWARE UV2	(REP03T-120)








	[ End of PRE-PROCEDURE ]











�









[PRE-PROCEDURE T4]














— OUTLINE —





( Select special (DKU) part (status check).


( Specify Replacement.


( Detach parts related to special part.


( Place part into unpluggable state.





�
[1] Select special part





1.	<Select DKU-nn>


�


	Select (CL) [DKU-nn] from ‘Maintenance’.





























2.	<Specify special part>


�


	Select part [XXXXX] to be Replaced.





	• ‘AC BOX-nnn’	[AC BOX-nn]








�
3.	<Specify special part Replacement> 


�


	Select (CL) [Execute].


	• ‘AC BOX-nnn’	Go to [2] (REP02T-140)

















�
[2] DKU AC BOX





1.


�


	“Do you want to replace the "AC BOX-xxx?"” is displayed.


	Select (CL) [Yes].























2.


�


	“The AC BOX that you will replace is 1-Phase type.  Please confirm type of maintenance part.  Do you want to continue?” is displayed.


	Confirm that maintenance part is 1-Phase type, and select (CL) [Yes].











�
3.


	The SVP automatically checks the power supplies to see if Breaker Box is replaceable.























4.	<Check environment monitor stopped state>


�


	“Processing to disable the environment check...” is displayed.





























�
5.


�


	“Turn off the breaker which supplies power source for AC-BOX-xxx.  After that please replace it.  When replacement is completed, press OK.” is displayed.


	(Reply with [OK] after replacing the special part.)


	Go to [3] (REP02T-170).











�
[3] Replacement





1.	<Replace special part>


	Replace the special part .








	AC BOX	see HARDWARE TUV22 (REP03T-410)








	[ End of PRE-PROCEDURE ]





	K6602078-





REV.0�
Jul.1999�
�
�
�
�
�
�
Copyright ©1999, Hitachi, Ltd.


REP02T-�page �16�0








NOTICE


 When the screen prompting an operator to input a password in order to prevent a multiple maintenance, contact the technical support center to ask for an instruction.











