








[POST-PROCEDURE t3]














— OUTLINE —





( Specify end of special part replacement.


( Reinstall related parts.


( Start environment monitor.


( SIM Complete





�
[1] START OF POST-PROCEDURE





1.	<Check special part replacement>





�


	Select (CL) [OK] in response to “Turn off the breaker which supplies power source for XXXXX.  After that please replace it.  When replacement is completed, press OK.


	• ‘Breaker Box-n’


	• ‘AC BOX-Cn’


	  When redundant power supply is installed


		Go to [2] (REP04T-30)


	  Not installed


		Go to [3] (REP04T-40)














�
[2] Breaker Box, DKC AC BOX





1.	<Checking the environment monitor start processing>


�


	“Processing to enable the environment check... is displayed.


























2.	<Checking the end of replacement>


�


	Select (CL) [OK] in response to “Replacement of the "XXXXX" is done.





	Go to [4] (REP04T-50)




















�
[3] Breaker Box, DKC AC BOX





1.	<checking end of replacement>


  (	When only serial or parallel channels, or they are mixed.


�


	Select (CL) [OK] in response to “The replacement of XXXXX has finished and Cluster is enabled.  The concerned channel path(s) are available.  Please vary the path(s) on.”.











  (	When SCSI channels and other kinds are mixed.


�


	Select (CL) [OK] in response to “The replacement of XXXXX has finished and Cluster is enabled.  The concerned channel path(s) and SCSI channel path(s) are available.  Please vary the path(s) on.”.

















  (	When only SCSI channels are installed.


�


	Select (CL) [OK] in response to “The replacement of XXXXX has finished and Cluster is enabled.  The concerned SCSI channel path(s) are available.”.











	Go to [4] (REP04T-50).








�
[4] Confirm Cluster





	If Cluster is blocked, recover it.


	See SVP02-890.


	Go to [5] (REP04T-50).




















[5] SIM Log Complete





	See SVP02-500.


	Close ‘Cluster-X’ window.


	Close ‘DKC’ window.


	Close ‘Maintenance’ window.





	[ End of POST-PROCEDURE ]











�









[POST-PROCEDURE t4]














— OUTLINE —





( Specify end of special part replacement.


( Reinstall related parts.


( Start environment monitor.


( DKU Path Inline Test.


( SIM Complete.





�
[1] Start of POST-PROCEDURE





1.	<Making part replacement check>





	• AC BOX-n





�


	Select (CL) [OK] in response to “Turn off the breaker which supplies power source for AC BOX-XXX.  After that please replace it.  When replacement is completed, press OK.”.





		Go to [2] (REP04T-80).














�
[2] DKU AC BOX





1.	<End of replacement>


�


	If an error occurs, go to [3] (REP04T-90).


	Select (CL) [OK] in response to “The replacement of AC BOX-Rn/Ln has finished.”.


	Go to [4] (REP04T-90).




















�
[3] Error





1.	<Replacement error>


�


	Select (CL) [OK] in response to “The replacement has failed.”.








	[ End of POST-PROCEDURE ]

















[4] SIM Log Complete


	See SVP02-500.


	Close ‘DKU-xx’ window.


	Close ‘Maintenance’ window.





	[ End of POST-PROCEDURE ]





	K6602078-





REV.0�
Jul.1999�
�
�
�
�
�
�
Copyright ©1999, Hitachi, Ltd.


REP04T-�page �9�0











