3.6	Mounting on U6 Rack Frame


3.6.1	Mounting subsystem on rack frame


� EMBED Word.Picture.6  ���


If the RK or RKA falls when the elevator of the lifter is at a high position, a personal injury will be caused.�Perform the positioning, fastening, or other handlings very carefully.


�


Work carefully because the mass of the single RK is heavy with about 65 kg and that of the RKA is about 40 kg.


(1)	Get the subsystem off the tranceportion. (The tranceportion is a property of the customer.)


(2)	Place the RK/RKA on the elevator of the lifter.�Mount the subsystem on the lifter with its front bezel removed.


(3)	Move the lifter to the position for mounting. (just in front of the rack frame.)








�
(4)	Adjust the position of the RK/RKA so that it is seated in the center of the rack frame.�If it is mounted off-center to the left or right, the front bezel may be touched by screws and prevented from being opened or closed.


�


Do not move the lifter away from the rack frame nor lower the elevator until the red line on the label affixed on the subsystem enters the rack frame across the end of it. Otherwise, falling of the subsystem may be caused.


� EMBED Word.Picture.6  ���





(5)	Place the RK/RKA on the rails in the rack frame.


(6)	Then, push it in towards the rear of the rack frame gently.





� EMBED Word.Picture.6  ���


Figure 3.6.1  Installing the RK/RKA in the Rack Frame





�
3.6.2	Securing the RK/RKA


(1)	Fastening the front side of the subsystem


(a)	Fasten the subsystem to the rack frame with the four M5×10 binding screws.





� EMBED Word.Picture.6  ���


Figure 3.6.2  Fastening Front Side of the Subsystem





(2)	Fastening rear side of the subsystem


(a)	Shift each of the two stoppers in the direction shown by the arrow and make it contact the frame.


(b)	Tighten the binding screws.





� EMBED Word.Picture.6  ���


Figure 3.6.3  Securing Rear Side of the Subsystem





�
(3)	Setting device ID number


The device ID number of the RK has been set to #0.   Set the device ID numbers of the RKAs in serial order starting from the RKA next to the RK.   As the Disk drive numbers, #0 to #9 are assigned to Disk drives mounted on the RK or RKA in the order from left to right when the RK/RKA is placed laterally.





� EMBED Word.Picture.6  ���


Figure 3.6.4  Mounting Locations of Disk Drives

















DF500	K6602339-





REV.1�
Jul.2000�
Aug.2000�
�
�
�
�
�
Copyright © 2000, Hitachi, Ltd.


INST 03-0230





REV.1�
Jul.2000�
Aug.2000�
�
�
�
�
�
Copyright © 2000, Hitachi, Ltd.


INST 03-0240





REV.1�
Jul.2000�
Aug.2000�
�
�
�
�
�
Copyright © 2000, Hitachi, Ltd.


INST 03-0250





REV.1�
Jul.2000�
Aug.2000�
�
�
�
�
�
Copyright © 2000, Hitachi, Ltd.


INST 03-0260














