
[image: image1.wmf]
Hitachi Freedom Storage™
Thunder 9500™ V Series
LUN Management 9500V
Configuration Guide
© 2003 Hitachi Data Systems Corporation, ALL RIGHTS RESERVED

Notice: No part of this publication may be reproduced or transmitted in any form or by any means, electronic or mechanical, including photocopying and recording, or stored in a database or retrieval system for any purpose without the express written permission of Hitachi Data Systems Corporation.

Hitachi Data Systems reserves the right to make changes to this document at any time without notice and assumes no responsibility for its use. Hitachi Data Systems products and services can only be ordered under the terms and conditions of Hitachi Data Systems’ applicable agreements. All of the features described in this document may not be currently available. Refer to the most recent product announcement or contact your local Hitachi Data Systems sales office for information on feature and product availability.

This document contains the most current information available at the time of publication. When new and/or revised information becomes available, this entire document will be updated and distributed to all registered users.

Trademarks

Hitachi Data Systems is a registered trademark and service mark of Hitachi, Ltd., and the Hitachi Data Systems design mark is a trademark and service mark of Hitachi, Ltd.

SANtinel is a trademark of Hitachi Data Systems.

HP and HP-UX are trademarks or registered trademarks of Hewlett-Packard Company and/or its subsidiaries.

AIX is a registered trademark of International Business Machines Corp. in the United States.

Linux is a registered trademark of Linus Torvalds.

Microsoft, Windows, Windows NT, and the Windows logo are either registered trademarks or trademarks of Microsoft Corporation in the United States and/or other countries.

SGI and IRIX are trademarks or registered trademarks of Silicon Graphics Inc. in the United States.

Solaris is a trademark or registered trademark of Sun Microsystems, Inc.

All other brand or product names are or may be trademarks or service marks of and are used to identify products or services of their respective owners.

Notice of Export Controls

Export of technical data contained in this document may require an export license from the United States government and/or the government of Japan. Please contact the Hitachi Data Systems Legal Department for any export compliance questions.

Document Revision Level

	Revision
	Date
	Description

	MK-93DF660-P
	March 2003
	Preliminary Release

	MK-93DF660-0
	April 2003
	Initial Release


Source Documents Used in this Revision

The following source documents were used to produce this guide:

· LUN Management Configuration Guide (DF600), LUNManagementSysConfig_r0, dated 2/14/03 (RSD K6602950-0).
· LUN Management Configuration Guide (MK-93DF660-Pa7), Hitachi Data System review, dated 3/15/03.

· LUN Management Configuration Guide (RSD-93DF660-1), RSD review, dated 3/31/03.

· LUN Management Configuration Guide (Modified-MK-93DF660-Pb2), Hitachi Data System review, dated 4/09/03.

Changes in this Revision

Referenced Documents

· 9500V User and Reference Manual (MK-92DF601)

· LUN Management 9500V User’s Guide (MK-93DF657)

Preface

The Hitachi Freedom Storage™ Thunder 9500™ V Series LUN Management Configuration Guide describes and provides instructions for connecting the disk array subsystem to hosts via the SAN (Storage Area Network) using LUN Management. The document also provides notes on the SAN system configuration. This configuration guide assumes that:

· the user has a background in data processing and understands direct-access storage device subsystems and their basic functions,

the user is familiar with the Hitachi Freedom Storage™ Thunder 9500™ V Series array subsystem, and

· the user is familiar with Windows operating system, system commands, and utilities.

Note: The term “9500V” refers to the entire Hitachi Freedom Storage™ Thunder 9500™ V Series subsystem family, unless otherwise noted. Please refer to the Hitachi Freedom Storage™ Thunder 9500™ V Series User and Reference Guide (MK-92DF601) for further information on the 9500V disk array subsystems.

Note: The term “SANtinel™” refers to “LUN Security” unless otherwise noted.

Microcode Level

This document revision applies to microcode versions 0652/B and higher.
COMMENTS

Please send us your comments on this document: doc.comments@hds.com.

Make sure to include the document title, number, and revision.
Please refer to specific page(s) and paragraph(s) whenever possible.

(All comments become the property of Hitachi Data Systems Corporation.)

Thank you!

Contents
11
Overview of LUN Management

1.1
What is LUN Management?
2
1.2
Features
3
1.3
Specifications
4
1.3.1
Specifications of LUN Management
4
1.3.2
Requirements for Using LUN Management
6
1.4
System Configuration Example
7
1.5
Notes
9
1.5.1
The Default Queue Depth Configuration of Each System
10
1.5.2
Queue Depth Limits of System Combination
10
2
Using LUN Management
11
2.1
Flow of Operations Required
12
2.2
Operations Required for Installation and Operation
13
3
Configuration of the SAN System
15
3.1
System Design
17
3.2
System Configuration
18
3.3
Addition and Replacement of Equipment
19
3.4
Avoiding Contention and RAID Groups
20
3.5
Design for Preventing Illegal Access of SAN
21
3.6
Queue Depth Setting of SAN
23
3.6.1
Increasing Queue Depth Setting and Port Sharing
24
3.6.2
Increasing Queue Depth Through Path Switching
25
3.6.3
Allocation of Queue Depth According to Priority of Host Job
26
4
Troubleshooting
27
4.1
Troubleshooting
28
4.2
Calling the Hitachi Data Systems Technical Support Center
28
Acronyms and Abbreviations
29
Index

31


List of Figures

2Figure 1.1
Setting Access Paths between Hosts and Logical Units


5Figure 1.2
Mapping LU(s) in the range between LU256 and LU511 to the Host


6Figure 1.3
The number of LU Mappings


7Figure 1.4
System Configuration Sample


8Figure 1.5
Configuration 1 (One Host Storage Domain (Host Group))


8Figure 1.6
Configuration 2 (Two Host Storage Domains (Host Groups))


12Figure 2.1
Flow of Operations Required for Using LUN Management


20Figure 3.1
In the Case of the Same RAID Group


20Figure 3.2
In the Case of the Respective RAID Groups


21Figure 3.3
A Case Where Only LU Mapping Is Set


22Figure 3.4
A Case Where LUN Management Is Used


22Figure 3.5
A Case Where LUN Management and Switch Zoning is Used


24Figure 3.6
Queue Depth in the Case Where Port Is Shared


25Figure 3.7
Increment of Queue Depth Generated Due to Path Switching


26Figure 3.8
A Case Where Priority Order Is Given to Host Jobs




List of Tables

3Table 1.1
Features of LUN Management and Differences between LUN Management and SANtinel™


4Table 1.2
LUN Management Specifications


6Table 1.3
Requirements of Using LUN Management


9Table 1.4
Combinations of OS (Operating System) and HBA


10Table 1.5
The Queue Depth Configuration


10Table 1.6
Queue Depth Limits of System Combination


28Table 4.1
Troubleshooting




1 Overview of LUN Management
This chapter contains the following:
1.1
What is LUN Management?
1.2
Features
1.3
Specifications
1.4
System Configuration Example
1.5
Notes
1.1 What is LUN Management?
LUN Management is an optional function that manages access paths between hosts and logical units (LUs) for each port. XE "overview:LUN Management"By using LUN Management, two or more host systems or operating systems (Host Storage Domain(HostGroup)) can be connected to a port of the 9500V and LUs can be freely assigned to each Host system.

Illegal access to LUs from any host system can be prevented. Using system security, each host system can safely use a Disk Array Subsystem as if they are connected to several storage systems.

LUN Management function can set Option (Host Connection Mode), Logical Unit (LU) and WWN (World Wide Name) settings for each Connected-Host so that multiple Hosts can be connected to a Port. By using LUN Management function, each Host can access a Logical Unit as if a dedicated Port to the Host even if the Host shares the Port with other Hosts.

Figure 1.1 shows that a port can be shared by each host system.

Figure 1.1
Setting Access Paths between Hosts and Logical Units


[image: image2.wmf] 

Solaris

™

 

AIX

®

 

Windows

®

 

HP

-

UX

®

 

Port

 

LU 0

 

LU 0

 

LU 0

 

LU 0

 

1

 

1

 

1

 

1

 

2

 

Host Group

 

for 

Solaris

™

 

Host Group

 

for 

AIX

®

 

Host Group

 

for 

Windows

®

 

Host Group

 

for 

HP

-

UX

®

 

Switch

 

9500V

 


Features
Table 1.1 lists LUN Management features.XE "features of LUN Management "

XE "LUN Management:features"
Table 1.1
Features of LUN Management and Differences between LUN Management and SANtinel™

	Item
	Features
	LUN Management
	SANtinel™

	1
	Illegal access from the other hosts can be prevented.
	(
LUs are grouped and the group is registered in a port as a Host Storage Domain (Host Group). It specifies which host can access which LU through assignment of hosts and LUs to each Host Storage Domain (Host Group).
	(
For any LU, host access can be specified.

	2
	The host connection mode can be set for each host connected.
	(
The host connection mode can be set for each Host Storage Domain (Host Group).
“HP Connection Mode 2”, which enables the LU8 and above to be recognized in the HP® connection, can be connected to the same port to which the other hosts are connected.
	(
(The host connection mode is set for each port.)

	3
	The LU mapping can be set for each host connected.
	(
LU numbers (H-LUN) recognized by a host can be assigned to each Host Storage Domain (Host Group). By virtue of this, two or more hosts that require LU0 can be connected to the same port.
	(
(The LU mapping is set for each port.)


Note: More hosts can be connected when connections to hosts are concentrated at one port. On the other hand, the traffic on the disk array subsystem is increased. When you use LUN Management, you should design the system configuration to disperse the heavy traffic concentrated at the port, controller, and disk drive.

Specifications
1.1.1 Specifications of LUN Management
The specifications of LUN Management are shown in Table 1.2.XE "specifications of LUN Management "

XE "LUN Management:specifications"
Table 1.2
LUN Management Specifications

	Item
	Specifications

	Host Storage Domain (Host Group)
	•  Up to 128 Host Storage Domains (Host Groups) can be set for each Port.
•  Host Group 0 (zero) is required.

	Setting/Deleting a Host Storage Domain (Host Group)
	•  Host Groups 1 through 127 can be set or deleted.

•  Host Group 0 (zero) cannot be deleted. In order to delete WWN, options, and LU Mapping of Host Group 0 (zero), initialize Host Group 0 (zero).

	Host Storage Domain (Host Group) Name
	•  A name is assigned to a Host Storage Domain (Host Group) upon creation.  This Host Storage Domain (Host Group) Name can be changed.

	WWN (Node Name, Port Name)
	•  Up to 128 World Wide Names (WWN) of Host Bus Adapters (HBA), can be set for a Host Storage Domain (Host Group) or port.
•  An identical WWN cannot be assigned to another Host Storage Domain (Host Group) on the identical same Port.
•  WWN can also be set to the Host Storage Domain (Host Group) by selecting from a Logined WWN of the Host Bus Adapter that is connected to the Port.

	WWN Nickname
	•  An optional Nickname can be assigned to a WWN allocated to the Host Storage Domain (Host Group). A Nickname can also be omitted.
•  A Nickname assigned to a WWN is valid until the WWN is deleted.

	Host Connection Mode
	•  The Host Connection Mode of the Host Storage Domain (Host Group) can be changed.

	LU Mapping
	•  LU Mapping can be set to the Host Storage Domain (Host Group).

•  Up to 256 LU Mappings can be set for a Host Storage Domain (Host Group). Up to 256X128 LU Mappings can be set for a Port.

	Enable/Disable setting for each port
	•  LUN Management can be set enable or disable for each Port.
•  When LUN Management is disabled, previously set LUN Management information is kept and is available when it is enabled again.

	Online setting
	•  When adding, modifying, or deleting the setting by LUN Management, it is not necessary to reboot the Disk Array subsystem.

	Concurrent use with SANtinel™
	•  LUN Management cannot be used together with SANtinel™ concurrently.


Although LU(s) range between LU0 and LU511 for the subsystem, the host recognizes LUN(s) in the range between H-LUN0 and H-LUN255. When you assign LU(s) in the range between LU256 and LU511 to the host, you must set the Host Storage Domain (Host Group) LU Mapping to be between H-LUN0 and H-LUN255.

Figure 1.2
Mapping LU(s) in the range between LU256 and LU511 to the Host


[image: image3.wmf] 

LU512

 

LU256

 

LU0

 

LU255

 

H

-

LUN255

 

H

-

LUN0

 

H

-

LUN255

 

H

-

LUN0

 

Disk Array

 

HG

 

WWN2

 

mode

 

HG

 

WWN1

 

mode

 

Switch

 

HG

 

WWNx

 

mode

 

WWN1

 

Port

 

Port

 

...

 

WWN2

 

..

.

 

...

 

...

 

...

 

H

-

LUN0 to 255

 

Port

 

Port

 

Host1

 

Host2

 

H

-

LUN0 to 255

 

LU(s)

 

recognized

 

by a host

 


· The number of LU Mappings

Up to 256 LU Mappings can be set for a Host Storage Domain (Host Group). Up to 256×128 LU Mappings can be set for a Port.
Figure 1.3
The number of LU Mappings


[image: image4.wmf] 

LU0

 

LU255

 

LU512

 

LU256

 

H

-

LUN255

 

H

-

LUN0

 

H

-

LUN255

 

H

-

LUN0

 

H

-

LUN0

 

H

-

LUN255

 

Disk Array

 

HG

 

WWN2

 

mode

 

HG

 

WWN1

 

mode

 

Switch

 

HG

 

WWNx

 

mode

 

Port

 

Port

 

...

 

...

 

...

 

Port

 

Port

 

WWN1

 

...

 

WWN2

 

...

 

WWNx

 

...

 

...

 

H

-

LUN0 to 255

 

H

-

LUN0 to 255

 

H

-

LUN0 to 255

 

Host1

 

Host2

 

Hostx

 

HG#127

 

LU(s)

 

recognized

 

by a host

 


1.1.2 Requirements for Using LUN Management
The requirements for using LUN Management are shown in Table 1.3.XE "requirements of LUN Management "

XE "LUN Management:requirements"
Table 1.3
Requirements of Using LUN Management

	Item
	Conditions

	Means of making setting
	Resource Manager 9500V (Version 7.20 or later) is required.

	Microprogram revision
	Microprogram 0652 or later is required.

	Coexistence of LUN Management with other optional functions
	Optional functions (other than SANtinel™) can be used together with LUN Management.

	Queue Depth
	Max. 32 commands per LU

Max. 512 commands per port


System Configuration Example

The subsystem is connected to a host with an optical fibre cable. The end of the cable on the host side is connected to an HBA (Host Bus Adapter), and that on the subsystem side, to a port of the disk array subsystem.XE "system configuration"

XE "LUN Management:system configuration"
Using LUN Management, LUs can be grouped and assigned to a port as a Host Storage Domain (Host Group). The user can then specify which HBA can access the group by assigning the WWNs of the HBAs to each Host Storage Domain (Host Group). The idea is to identify which LUs you want to use through which physical port with a particular host, and define a host storage domain on that port for these LUNs.

Figure 1.4
System Configuration Sample


[image: image5.wmf] 

H

-

LUN0

 

H

-

LUN0

 

H

-

LUN1

 

LU0

 

LU1

 

LU2

 

LU3

 

LU4

 

LU5

 

Port

 

Port

 

IO path change

 

WWN2

 

Win2

 

WWN1

 

Win1

 

HBA

 

Solaris

 

WWN3

 

HP

 

WWN4

 

WWN5

 

WWN6

 

WWN7

 

AIX1

 

AIX2

 

(LU5)

 

(LU4)

 

H

-

LUN0

 

H

-

LUN1

 

(LU5)

 

(LU4)

 

H

-

LUN0

 

H

-

LUN1

 

(LU3)

 

H

-

LUN0

 

(LU3)

 

H

-

LUN

0

 

(LU2)

 

H

-

LUN0

 

(LU1)

 

H

-

LUN0

 

(LU0)

 

H

-

LUN0

 

Disk Array

 

HG

 

WWN4

 

mode

 

HG

 

WWN3

 

mode

 

HG

 

WWN2

 

mode

 

HG

 

WWN1

 

mode

 

HG

 

WWN7

 

mode

 

HG

 

WWN5

 

mode

 

Switch

 

Switch

 

HG

 

WWN6

 

mode

 

Group corresponding to each permission to access

 

H

-

LUN0

 

H

-

LUN0

 

H

-

LUN0

 

H

-

LUN0

 

H

-

LUN0

 

H

-

LUN1

 

for Win1

 

for Cluster of

 

AIX1 and AIX2

 

 

for Win2

 

for HP

 

for IO path change

 

of Solaris

 

LU(s) 

recognized 

by a

 host

 

Physical path (fibre cable)

 

Logical path (path via which access is permitted)

 

LU mapping

 

Cluster

 


Figure 1.5 and Figure 1.6 show possible configurations for creating the Host Storage Domain (Host Group) for multipathed and clustered environments.XE "creating Host Storage Domain (host group)"

XE "Host Storage Domain (host group):creating"
Figure 1.5
Configuration 1 (One Host Storage Domain (Host Group))

[image: image6.wmf] 

LU0

 

HG

 

WWN1 

WWN2

 

mode

 

Switch

 

WWN2

 

Host2

 

WWN1

 

Host1

 

Port

 

H

-

LUN0

 

HBA

 

Cluster

 


Figure 1.6
Configuration 2 (Two Host Storage Domains (Host Groups))

[image: image7.wmf] 

LU0

 

HG

 

WWN

2

 

mode

 

HG

 

WWN

1

 

mode

 

Switch

 

WWN2

 

Host2

 

WWN1

 

Host1

 

Port

 

H

-

LUN0

 

HBA

 

Cluster

 


Notes
· Verify that the Host Storage Domain (Host Group) information is correct and accurate. 

· This function is not available when a host (OS or driver) cannot identify an LU without LUN#0. It is necessary to set H-LUN = 0 to the Host Storage Domain (Host Group) in this case.

Table 1.4
Combinations of OS (Operating System) and HBA

	OS
	HBA*
	Remarks

	HP-UX®
	Genuine HP® HBA
	When HP connection mode2 = ON is selected

	IRIX®
	Genuine SGI™ HBA
	

	Windows® 2000
	Emulex HBA (with Miniport Driver)
Qlogic HBA
	

	Linux®
	Emulex HBA
Qlogic HBA
	


* Note: HBAs listed may not be all inclusive. For information on supported fibre-channel adapters (FCAs), optical cables, hubs, and fabric switches, please contact your Hitachi Data Systems account team or the Hitachi Data Systems Support Center (see section 4.2).

· When you replace an HBA in a host server attached to a 9500V using Host Storage Domain (Host Group), be sure to change the WWN setting in LUN Management.
· Host Storage Domain (Host Group) settings (enabling or disabling) are not valid immediately after the setting. To make the setting valid, reboot the host (to be recognized again).

· The WWN set up when using SANtinel™ is succeeded and used after uninstalling SANtinel™ and installing LUN Management. However, if the WWN is set as 0 when using SANtinel™, the WWN should be deleted before installing LUN Management.

The Default Queue Depth Configuration of Each System
The default Queue Depth setting for each host operating system is shown in Table 1.5. Each system has an individual Queue Depth value unit and setting unit.XE "Queue Depth:configuration"

XE "configuration:Queue Depth"
Table 1.5
The Queue Depth Configuration

	Platform
	HBA
	Queue Depth
	Unit of Setting

	
	
	Unit of Value
	Default
	

	Solaris™
	
	LU
	256
	OS

	HP-UX®
	
	LU
	8
	LU

	AIX®
	
	LU
	1
	LU

	IRIX®
	
	LU
	1
	LU

	Windows®
	Emulex (Port)
	LU
	8
	HBA

	
	Emulex (MiniPort)
	LU
	32
	HBA

	
	Qlogic
	Port
	16
	HBA


1.1.3 Queue Depth Limits of System Combination

When the following host operating system combination is connected to a single port of the disk array subsystem, you must set the Queue Depth value described in Table 1.6.
Please see 3.6 for more information on the setting of queue depth.XE "Queue Depth:limits"

XE "limits:Queue Depth"
Table 1.6
Queue Depth Limits of System Combination

	System Combination
	Setting Queue Depth

	Windows NT®/Windows® 2000

MSCS
	Max.16 commands per port (for Qlogic HBA）

	HP-UX®
MC/ServiceGuard
	No limit


2 Using LUN Management
This chapter explains procedures required when installing, using, and configuring LUN Management. Actions to be taken when failures occur are also explained.
This chapter contains the following:
2.1
Flow of Operations Required
2.2
Operations Required for Installation and Operation
2.1 Flow of Operations Required
The procedures required to install and use LUN Management are shown in Figure 2.1.
Note: The individual responsible for each procedure is the customer.XE "flow of operations:configuration"

XE "overview:flow of operations"
Figure 2.1
Flow of Operations Required for Using LUN Management

[image: image8.wmf] 

Planning 

and 

designing

 

Installation 

and 

configuration

 

Operation

 

System 

configuration 

designing

 

Hardware 

installation

 

Setting of disk 

array

 

Setting of optional 

function

 

Host connection 

check

 

System 

configuration 

change 

(addition/change 

of host(s)/ LU(s))

 

Correcting failures 

(HBA/switch 

replacement)

 

Procedures

 

Detail

ed Procedures

 

 

-

 

D

esigning 

and

 

configuring 

SAN environment

 

 

-

 

Carrying in o

f hardware

 

 

-

 

Offline setting of disk array 

subsystem

 

 

-

 

Check of LU recognition

 

 

-

 

Changing number of Queue 

Depth

 

 

-

 

Change of setting for LUN 

Management

 

 

-

 

Reset of switch zoning

 

 

 

-

 

Installation of and setting for 

LUN Man

agement

 

 

-

 

Obtaining of WWN of HBA

 

 

-

 

Setting of switch zoning

 

 

 

-

 

Change of setting for LUN 

Management

 

 

-

 

Obtaining of WWN of HBA

 

 

-

 

Change of switch zoning 

setting

 

 

Host connection 

check

 

 

-

 

Check of LU recognition

 

 

-

 

Changing number of Queue 

Depth

 


Operations Required for Installation and Operation
· Planning and designing: System configuration designingXE "installation and operation:required operations"

XE "required operations:installation and operation"
Design connections between hosts and the disk array subsystem for constructing the SAN (Storage Area Network) environment.XE "system configuration:designing"
Design connections to the switch, setting for LUN Management, and setting of the switch zoning. Additionally, Queue Depth for each host connected to the disk array subsystem must be identified.

· Installation and configuration: Setting of optional functionXE "setting:optional function "

XE "optional function:setting"
The WWN(s) of the HBA(s) to be set for LUN Management must be identified. Please refer to the LUN Management User’s Guide (MK-93DF657) for how to identify the WWN of each platform or HBA.

Install LUN Management and make setting for it according to the system configuration design.

Set a zoning of the switch when necessary.

· Installation and configuration: Host connection checkXE "installation and configuration:host connection:checking"

XE "host connection checking"
Check and verify that each host can recognize the LU(s).

Change the Queue Depth of each host when necessary. For instructions on changing the Queue Depth of each host or HBA, refer to the LUN Management User’s Guide.

· Operation: System configuration change (addition/change of host(s)/LU(s))XE "installation and integration:system configuration:change"

XE "changing system configuration"
It is necessary to add or change setting for LUN Management when the hosts connected to the disk array subsystem are added or changed, or when the number of LUs assigned to a Host Storage Domain (Host Group) is increased. When adding a host or changing a connected host, a WWN of an HBA of the host must be identified. 

Set a zoning of the switch when necessary.

· Operation: Host connection checkXE "operation:host connection:checking"

XE "checking:host connection"
When the setting for LUN Management is added to or changed, also check if each host can recognize the LU(s) in accordance with the change.

Change the Queue Depth of each host when necessary.

· Operation: Correcting failures (HBA/switch replacement)XE "correcting failures"

XE "opertion:correcting failures"
When an HBA is replaced because of an HBA failure, the WWN registered in LUN Management must be changed to that of the new HBA.

In addition, when the switch is replaced, the same zoning must be set for the new switch.

3 Configuration of the SAN System
This chapter contains the following:
3.1
System Design
3.2
System Configuration
3.3
Addition and Replacement of Equipment
3.4
Avoiding Contention and RAID Groups
3.5
Design for Preventing Illegal Access of SAN
3.6
Queue Depth Setting of SAN
When connecting multiple hosts to one port of the disk array subsystem, the system must be designed to take into account the following:

· System design

· 
Determining assignment of LUs to hosts

· 
Determining assignment of LUs to RAID groups

· 
Determining the system configuration

· 
Determining the method of illegal access prevention

· 
Determining the Queue Depth

· System configuration

· 
Setting LUN Management

· 
Setting the switch zoning

· Component addition and replacement

· 
Host and HBA addition

· 
LU addition

· 
HBA replacement

· 
Switch replacement

System Design
The following must be taken into consideration during system design:

· Assignment of LUs to HostsXE "system design:assignment of LUs to hosts"

XE "assigning LUs to hosts"
Group logical units (LUs) of the disk array subsystem into groups with permitted access. Manage the group of the LUs as a Host Storage Domain (Host Group). Use Host Storage Domain (Host Group) when making setting for LUN Management.

· Avoiding ContentionXE "system design:avoding contention"

XE "avoiding contention"
Each LU is defined as belonging to a RAID group. Therefore, when two LUs (defined as belonging to the same RAID group) are accessed at the same time, operational performance may decrease. When operating two or more hosts at the same time, it is recommended that LUs (assigned to each host) be assigned to separate RAID groups.

· System ConfigurationXE "system design:system configuration"

XE "system configuration"
When connecting more hosts than ports of the disk array subsystem, increase the number of ports to be connected to hosts using the fibre channel switch.

· Methods of Illegal Access PreventionXE "system design:preventing access"

XE "preventing access"
Determine input/output paths between hosts and LUs according to the assignment mode using LUN Management. The input/output path is a route permitting access from the host is permitted.

Set zoning for the switch in order to prevent an effect from the other hosts that share the switch. When the zoning is set, other ports outside the zone do not affect ports within the zone.

· Queue DepthXE "system design:Queue Depth"

XE "Queue Depth:limits"
Multiple hosts can now be connected to a single port. But, the Queue Depth that can be handled by one port is limited and performance drops if the limit is exceeded. Therefore, specify the Queue Depth so that the sum for all hosts does not exceed the value of the limit per port.

System Configuration
Configure the system as designed by:

· Making setting for LUN ManagementXE "system configuration:making settings"

XE "settings:system configuration"
Set the following for the disk array subsystem in order to specify input/output paths between hosts and LUs using LUN Management.

· 
Host Storage Domain (Host Group)

· 
WWN of HBA

· 
LU mapping

· 
Host connection mode

It is necessary to manage the settings that have been made for the disk array subsystem keeping a documented record. When an HBA is replaced, change the WWN accordingly.

In this way, you can identify the WWN that was set before the replacement by referring to the record.

· Zoning of switchXE "system configuration:switch zoning"

XE "zoning:setting"
Connect hosts and the disk array subsystem to the switch and set a zoning for the switch.

The diagram of connections between the switch and hosts and between the switch and disk array subsystem must be managed by keeping a documented record together with the zoning settings. When the switch is replaced, you can make the same connections as before the replacement by referring to the record.

Addition and Replacement of Equipment
The following are tasks required after introducing LUN Management:

· Adding host and HBAXE "adding:host and HBA"

XE "host and HBA:adding"
When assigning LUs within the disk array subsystem to a host and an HBA, add a Host Storage Domain (Host Group) to the disk array subsystem. When adding a setting of LUN Management, it is not necessary to restart the disk array subsystem. Execute a host boot or the disk recognition command after making a setting of LUN Management.

· Adding LUXE "adding:LU"

XE "LU:adding"
When adding an LU to the disk array subsystem and assigning it to a host that has been connected, add a mapping of the LU to the Host Storage Domain (Host Group) to which the host belongs. When adding the setting for LUN Management, it is not necessary to restart the disk array subsystem. Execute a host boot or the disk recognition command after making a setting of LUN Management.

· Replacing HBAXE "replacing:HBA"

XE "HBA:replacing"
When replacing an HBA of a host, the setting for LUN Management must be modified. Change the WWN of the HBA. If you do not make this change, the host will not be able to access the LU after the replacement of the HBA.

Each setting for LUN Management must be documented when the configuration is created so that the previous setting can be reviewed before making a change to the setting.

· Replacing switchXE "replacing:switch"

XE "switch:replacing"
When replacing the switch, the same connections and zoning setting used before the replacement must be made for the new switch. If the setting of the port and zoning for the connected switch do not agree with each other, the host will not be able to access the LU. In order to make the same connections as before the replacement, the diagram of connections between the switch and hosts and between the switch and disk array subsystem must be documented together with the setting of zoning.

Avoiding Contention and RAID Groups
When two or more hosts are connected to a disk array subsystem and LUs assigned to each host belong to the same RAID group, concurrent access to the same disk drive will occur. Because of this conflict for access to the disk drive, performance may decrease. Basically, LUNs in one RAID group should only be accessed by one host. To avoid a conflict, you should never have more than one host access multiple LUNs in one RAID group.XE "avoiding contention"
The number of RAID groups that can be created is determined according to the number of mounted disk drives and the RAID level of the RAID groups to be created. If unable to create as many RAID groups as hosts to be connected, organize the RAID groups according to operational states of the hosts (see Figure 3.1 and Figure 3.2).
Figure 3.1
In the Case of the Same RAID Group

[image: image9.wmf] 

Host1

 

Host2

 

Switch

 

WWN1

 

WWN2

 

H

-

LUN0 (LU3)

 

H

-

LUN1 (LU4)

 

RAID Group 0

 

RAID Group 1

 

LU4

 

LU3

 

Port

 

Disk Drive

 


Figure 3.2
In the Case of the Respective RAID Groups

[image: image10.wmf] 

Switch

 

Host2

 

Host1

 

WWN1

 

Port

 

WWN2

 

H

-

LUN1 (LU4)

 

H

-

LUN0 (LU3)

 

RAID Group 0

 

RAID Group 1

 

LU4

 

LU3

 

Disk Drive

 


Design for Preventing Illegal Access of SAN
When connecting hosts to one port of the disk array subsystem using a switch, a control to distinguish accessible hosts is required for each LU.XE "designing:for access prevention"

XE "access prevention:designing for"
When assigning LU3 to Host 1 and LU4 to Host 2, both hosts can access the same LU if the LU mapping is set alone as shown in Figure 3.3. When LUN Management is used in this case, the host (WWN: World Wide Name) access to each LU can be distinguished even in the same port. It is recommended that zoning be set in order to prevent ports of the disk array subsystem from being affected by the other host even when LUN Management is used (see Figure 3.5).

Figure 3.3
A Case Where Only LU Mapping Is Set

[image: image11.wmf] 

LU4

 

HG#0

 

LU3

 

H

-

LUN0 (LU3)

 

H

-

LUN1 (LU4)

 

Host2

 

Host1

 

H

-

LUN0 (LU3)

 

H

-

LUN1 (LU4)

 

WWN1

 

Port

 

WWN2

 

LU4

 

LU3

 

H

-

LUN0

 

H

-

LUN1

 

Switch

 


Figure 3.4
A Case Where LUN Management Is Used

[image: image12.wmf] 

H

-

LUN0 (LU3)

 

Host1

 

H

-

LUN1 (LU4)

 

Host2

 

WWN1

 

Port

 

HG#1

 

WWN2

 

LU4

 

HG#0

 

WWN1

 

LU3

 

WWN2

 

LU4

 

LU3

 

H

-

LUN0

 

H

-

LUN1

 

Switch

 


Figure 3.5
A Case Where LUN Management and Switch Zoning is Used

[image: image13.wmf] 

H

-

LUN0 (LU3)

 

Host1

 

H

-

LUN1 (LU4)

 

Host2

 

WWN1

 

Port

 

HG#1

 

WWN2

 

LU4

 

HG#0

 

WWN1

 

LU3

 

WWN2

 

: Zoning

 

LU4

 

LU3

 

H

-

LUN1

 

H

-

LUN0

 

Switch

 


Queue Depth Setting of SAN
A host can queue successive commands to the disk array subsystem before execution of a previous command can complete. The number of times successive commands are issued is called Queue Depth. When two or more hosts are connected to a port of the disk array subsystem, the number of Queue commands for the port is increased because the host issues commands to the disk array subsystem successively.XE "Queue Depth:setting"

XE "setting:Queue Depth"
Multiple hosts can now be connected to a single port. But, the Queue Depth that can be handled by one port is limited and performance drops if the limit is exceeded. Therefore, specify the Queue Depth so that the sum for all hosts does not exceed the value of the limit per port. The disk array subsystem reports the Queue Full status with commands received after the limit is exceeded. Since the host that has received the report of the Queue Full status stops issuing the multiplex command, a decrease in host performance may occur.

In order to avoid such a situation, Queue Depth should be carefully considered when designing the system.

Note: When the Queue Depth setting is increased, traffic on the disk array subsystem is also increased. At the same time, traffic on the host and switch may also be increased. This should be taken into consideration when changing the Queue Depth setting.

Note: The formula for defining Queue Depth on the host side varies depending on the type of operating system or HBA. When laying out the overall Queue Depth settings for all hosts, consideration should be given to the port limit.

Increasing Queue Depth Setting and Port Sharing
Figure 3.6 shows how to determine the Queue Depth when a port is shared.XE "Queue Dept:increasing setting:port sharing"

XE "setting:increasing Queue Depth setting:port sharing"
In this system, Host 1, Host 2, Host 3, and Host 4 are connected to a port that is given a multiple number of commands limited to 512. In this case, the Queue Depth for the port must be specified so that a sum of numbers of Queue Depth for each host, A, B, C, and D, does not exceed X.

Figure 3.6
Queue Depth in the Case Where Port Is Shared

[image: image14.wmf] 

WWN1

 

Port

 

WWN2

 

Host1

 

Host2

 

WWN3

 

Host3

 

WWN4

 

Host4

 

···

 

Switch

 

·

 

·

 

·

 

Limit of the number of Queue Depth specified 

for the port = X

 (512 per port)

 

Numbers of Queue Depth 

=

 

A

 

Design of a number of Queue Depth

 

A+B+C+D+

···

 

£

 X

 

Numbers of Queue Depth 

= B

 

Numbers of Queue Depth 

= C

 

Numbers of Queue Depth 

= D

 


Increasing Queue Depth Through Path Switching
Figure 3.7 shows how to determine the Queue Depth when an alternative path is configured. The system shown has a configuration with an alternative path: the Host 1 and Host 2 sides of which are assigned to the Primary and Secondary paths respectively.XE "Queue Depth:increasing setting:path switching"

XE "setting:increasing Queue Depth setting:path switching"
Normally, commands are supposed to be issued to a LU via the Primary path on the Host 1 side. In a configuration like this, when commands to be issued via the Primary path are moved to the Secondary path because a path switching is done, the Queue Depth for a port connected to a host on the Secondary path is increased. It is necessary to specify the Queue Depth for each host so that the number does not exceed its limit even after the path switching is done.

Figure 3.7
Increment of Queue Depth Generated Due to Path Switching

[image: image15.wmf] 

WWN1

 

Port

 

WWN2

 

Host1

 

Host2

 

WWN3

 

Host3

 

WWN4

 

Host4

 

Switch

 

Primary

 

Secondary

 

Path switching is done

 

Port

 

···

 

Limit of the number of Queue Depth specified 

fo

r the port = X

 (512 per port)

 

Numbers of Queue Depth 

= 

A

 

Design of a number of Queue Depth

 

C+D+

···

+A

 

£

 X

 

Numbers of Queue Depth 

= C

 

Numbers of Queue Depth 

= D

 


Allocation of Queue Depth According to Priority of Host Job
Figure 3.8 shows how to determine the Queue Depth when the priority order is given to jobs of connected hosts. To raise the priority of the host job individually, increase the Queue Depth for the host. When a Queue Depth for a certain host is increased, it is necessary to have the number for the port concerned not exceed its limit. This is done by reducing the numbers for the other hosts connected to the port.XE "determining Queue Depth"

XE " Queue Depth:determining"
Note: If the disk array subsystem does not have prioritized order control, you must allocate the number of host Queue Depth.

Figure 3.8
A Case Where Priority Order Is Given to Host Jobs

[image: image16.wmf] 

WWN1

 

Port

 

WWN2

 

Host1

 

Host2

 

WWN3

 

Host3

 

WWN4

 

Host4

 

Switch

 

···

 

Limit of the number of Queue Depth specified 

for the port = X

 (512 per port)

 

 

Numbers of Queue Depth 

= 

A

 

Design of a number of Queue Depth

 

The ratio A:B:C:D: 

···

 is changed on condition 

that the sum of them does not exceed X.

 

Numbers of Queue Depth 

= B

 

Numbers of 

Queue Depth 

= C

 

Numbers of Queue Depth 

= D

 

·

 

·

 

·

 

Priority ranking of job = 4

 

Priority ranking of job = 3

 

Priority ranking of job = 2

 

Priority ranking of job = 1

 


4 Troubleshooting
This chapter contains the following:
4.1
Troubleshooting
4.2
Calling the Hitachi Data Systems Technical Support Center
Troubleshooting
The Freedom Storage™ Thunder 9500™ V Series subsystem provides continuous data availability. For troubleshooting information on the 9500V subsystem, please refer to the 9500V User and Reference Manual (MK-92DF601).XE "troubleshooting"
Table 4.1 lists potential error conditions during 9500V LUN Management configuration and provides instructions for resolving each condition. If you are unable to resolve an error condition, please contact your Hitachi Data Systems representative, or call the Hitachi Data Systems Support Center for assistance.

Table 4.1
Troubleshooting

	Error Condition
	Recommended Action

	LUN Management cannot be used.
	Check if SANtinel™ function is used or not. If SANtinel™ is used, uninstall SANtinel™. LUN Management cannot be used together with SANtinel™.

	Host cannot access the LU.
	-Check the configuration regarding WWN of HBA.

-Check the setting regarding the number of the LU to
 the Host Storage Domain(Host Group).


4.1 Calling the Hitachi Data Systems Technical Support Center
If you need to call the Hitachi Data Systems Technical Support Center, be sure to provide as much information about the problem as possible. Include the circumstances surrounding the error or failure and the exact content of any error codes and/or messages displayed. The worldwide Hitachi Data Systems Technical Support Centers are:XE "technical support"
· Hitachi Data Systems North America/Latin America
San Diego, California, USA
1-800-348-4357

· Hitachi Data Systems Europe
Contact Hitachi Data Systems Local Support

· Hitachi Data Systems Asia Pacific
North Ryde, Australia
011-61-2-9325-3300

Acronyms and Abbreviations

H-LUN
host-logical unit number

HBA
host bus adapter

FCA
fibre-channel adapter

IO
input/output

LU
logical unit

LUN
logical unit number

LUN Security
Currently known as SANtinel™

OS
operating system

SAN
storage area network

SANtinel™
Previously known as LUN Security

WWN
world wide name

Index



A
access prevention
designing for, 21

adding
host and HBA, 19

LU, 19

assigning LUs to hosts, 17

avoiding contention, 17, 20

C
changing system configuration, 13

checking
host connection, 13

configuration
Queue Depth, 10

correcting failures, 13

creating Host Storage Domain (host group), 8

D
designing
for access prevention, 21

determining Queue Depth, 26

F
features of LUN Management, 3

flow of operations
configuration, 12


H
HBA
replacing, 19

host and HBA
adding, 19

host connection checking, 13

Host Storage Domain (host group)
creating, 8

I
installation and configuration
host connection
checking, 13

installation and integration
system configuration
change, 13

installation and operation
required operations, 13

L
limits
Queue Depth, 10

LU
adding, 19

LUN Management
features, 3

requirements, 6

specifications, 4

system configuration, 7


O
operation
host connection
checking, 13

opertion
correcting failures, 13

optional function
setting, 13

overview
flow of operations, 12

LUN Management, 2

P
preventing access, 17

Q
Queue Dept
increasing setting
port sharing, 24

Queue Depth
determining, 26

Queue Depth
configuration, 10

increasing setting
path switching, 25

limits, 10, 17

setting, 23

R
replacing
HBA, 19

switch, 19

required operations
installation and operation, 13

requirements of LUN Management, 6


S
setting
increasing Queue Depth setting
path switching, 25

port sharing, 24

optional function, 13

Queue Depth, 23

settings
system configuration, 18

specifications of LUN Management, 4

switch
replacing, 19

system configuration, 7, 17

designing, 13

making settings, 18

switch zoning, 18

system design
assignment of LUs to hosts, 17

avoding contention, 17

preventing access, 17

Queue Depth, 17

system configuration, 17

T
technical support, 28

troubleshooting, 28

Z
zoning
setting, 18



iii

_1106375662.doc


Cluster







IO path change







LU mapping







Logical path (path via which access is permitted)







Physical path (fibre cable)







LU(s) recognized by a host







LU5







LU4







LU3







LU2







LU1







for IO path change



of Solaris







for HP







for Win2







for Cluster of



AIX1 and AIX2











for Win1







HBA







H-LUN1







H-LUN0







H-LUN0







H-LUN1







H-LUN0







H-LUN0







H-LUN0







H-LUN0







H-LUN0







Port







Port







H-LUN1







H-LUN0







H-LUN1







H-LUN0







H-LUN0







H-LUN0







H-LUN0







H-LUN0







H-LUN0







(LU4)







(LU4)







(LU5)







(LU5)







(LU2)







(LU1)







(LU3)







(LU3)







(LU0)







Group corresponding to each permission to access







Win1







WWN1







Win2







WWN2







HP







WWN3







AIX2







WWN7







AIX1







WWN6







WWN5







Solaris







WWN4







HG



WWN6



mode







Switch







Switch







HG



WWN5



mode







HG



WWN7



mode







HG



WWN1



mode







HG



WWN2



mode







HG



WWN3



mode







HG



WWN4



mode







LU0







Disk Array












_1114410726.doc


Design of a number of Queue Depth



A+B+C+D+··· ( X







Numbers of Queue Depth = B







Numbers of Queue Depth = C







Numbers of Queue Depth = D







Numbers of Queue Depth = A







Limit of the number of Queue Depth specified for the port = X (512 per port)







·



·



·







Switch







···







Host4







WWN4







Host3







WWN3







Host2







Host1







WWN2







Port







WWN1












_1114414191.doc


LU(s)



recognized



by a host







H-LUN0 to 255







Host2







Host1







Port







Port







H-LUN0 to 255







...







...







...







LU512







LU256







...







H-LUN255







H-LUN0







WWN2







...







H-LUN255







H-LUN0







LU255







Port







Port







WWN1







HG



WWNx



mode







Switch







HG



WWN1



mode







HG



WWN2



mode







LU0







Disk Array












_1114414368.doc


LU(s)



recognized



by a host







HG#127







Hostx







Host2







Host1







H-LUN0 to 255







H-LUN0 to 255







H-LUN0 to 255







...







...







H-LUN255







H-LUN0







WWNx







...







H-LUN255







H-LUN0







WWN2







...







H-LUN255







H-LUN0







WWN1







Port







Port







...







...







...







LU512







LU256







LU255







Port







Port







HG



WWNx



mode







Switch







HG



WWN1



mode







HG



WWN2



mode







LU0







Disk Array












_1114410758.doc


Port







Path switching is done







Secondary







Primary







Design of a number of Queue Depth



C+D+···+A ( X







Limit of the number of Queue Depth specified for the port = X (512 per port)







Numbers of Queue Depth = C







Numbers of Queue Depth = D











Numbers of Queue Depth = A







···







Switch







Host4







WWN4







Host3







WWN3







Host2







Host1







WWN2







Port







WWN1












_1114410785.doc


Priority ranking of job = 1







Priority ranking of job = 2







Priority ranking of job = 3







Priority ranking of job = 4







Numbers of Queue Depth = D







Numbers of Queue Depth = C







Numbers of Queue Depth = B







Design of a number of Queue Depth



The ratio A:B:C:D: ··· is changed on condition that the sum of them does not exceed X.







Numbers of Queue Depth = A







Limit of the number of Queue Depth specified for the port = X (512 per port)











·�·�·







Switch







···







Host4







WWN4







Host3







WWN3







Host2







Host1







WWN2







Port







WWN1












_1110622454.doc
[image: image1.wmf]

Solaris™







AIX®







Windows®







HP-UX®







Port







LU 0







LU 0







LU 0







LU 0







1







1







1







1







2







Host Group�for Solaris™







Host Group�for AIX®







Host Group�for Windows®







Host Group�for HP-UX®







Switch







9500V












_1114410354.doc














 - Check of LU recognition



 - Changing number of Queue Depth







Host connection check







 - Change of setting for LUN Management



 - Obtaining of WWN of HBA



 - Change of switch zoning setting











 - Installation of and setting for LUN Management



 - Obtaining of WWN of HBA



 - Setting of switch zoning















 - Change of setting for LUN Management



 - Reset of switch zoning











 - Check of LU recognition



 - Changing number of Queue Depth







 - Offline setting of disk array subsystem







 - Carrying in of hardware







 - Designing and configuring SAN environment















Detailed Procedures







Procedures







Correcting failures (HBA/switch replacement)







System configuration change (addition/change of host(s)/ LU(s))







Host connection check







Setting of optional function







Setting of disk array







Hardware installation







System configuration designing







Operation







Installation and configuration







Planning and designing












_1106640182.doc


Cluster







HBA







H-LUN0







Port







Host1







WWN1







Host2







WWN2







Switch







HG



WWN1



mode







HG



WWN2



mode







LU0












_1106646014.doc


Switch







Disk Drive







Host2







Port







LU3







LU4







RAID Group 1







RAID Group 0







H-LUN1 (LU4)







H-LUN0 (LU3)







Host1







WWN2







WWN1












_1106639870.doc


Cluster







HBA







H-LUN0







Port







Host1







WWN1







Host2







WWN2







Switch







HG



WWN1 WWN2



mode







LU0












_1105958604.doc


Disk Drive







Switch







LU3







LU4







RAID Group 1







RAID Group 0







Host2







Host1







H-LUN0 (LU3)







H-LUN1 (LU4)







WWN2







Port







WWN1












_1105958763.doc


Switch







H-LUN1







H-LUN0







Host2







Host1







H-LUN0 (LU3)







H-LUN1 (LU4)







LU3







LU4







WWN2







LU3







WWN1







HG#0







LU4







WWN2







HG#1







Port







WWN1












_1105958946.doc


Switch







H-LUN0







H-LUN1







Host2







Host1







H-LUN0 (LU3)







H-LUN1 (LU4)







LU3







LU4







: Zoning







WWN2







LU3







WWN1







HG#0







LU4







WWN2







HG#1







Port







WWN1












_1105958732.doc


Switch







Host2







H-LUN1







H-LUN0







H-LUN1 (LU4)







H-LUN0 (LU3)







H-LUN1 (LU4)







H-LUN0 (LU3)







LU3







LU4







Host1







WWN2







LU3







HG#0







LU4







Port







WWN1












_1060662600.doc
[image: image1.png]HITACHI

DATA SYSTEMS







