B900-230 8.0.1.11(H191SP3C983)

Release Notes

| Prepared by | B900-230 Team | Date | 2019-03-11 |
| --- | --- | --- | --- |
| Reviewed by | B900-230 Team | Date | 2019-03-11 |
| Approved by | B900-230 Team | Date | 2019-03-11 |

Huawei Technologies Co., Ltd.

All rights reserved

Table of Contents

1.	Main Features	3

2.	Hardware	3

2.1	Version Description	3

2.2	Hardware Specifications	4

3.	Firmware	4

3.1	Version Description	4

3.2	Improvement in the Previous Version	4

3.3	Known Limitations and Issues	5

4.	Software Vulnerabilities Fixes	5

- Main Features

- The B900-230 supports the following standards:

FDD LTE Cat 6: 300/50Mbps @40M BW

TD-HSDPA: 2.8Mbps(DL) TD-HSUPA: 2.2Mbps(UL)

DC-PA+ 42Mbps; HSPA+: 21/5.76Mbps; 14/5.76Mbps

HSPA:7.2/5.76Mbps;

Support WiFi 802.11a/b/g/n/ac  2.4GHz 2*2 &5GHz 2*2 DBDC (AP mode only)

Support Amazon AVS.

Support BT 4.0.

- Hardware

  - Version Description

| Hardware Version: | WL1B900FM01 |
| --- | --- |
| Platform & Chipset: | BALONG V722C60 WiFi 1151 |

  - Hardware Specifications

| Form Factor | Size: 105*105*218mm |
| --- | --- |
| LTE Feature | LTE Cat6 2CC up to 300/50Mbps@40MHz FDD -LTE: B1/3/7/8/20/32/38 -UMTS: B1/8 -CA bands: CA_3C / CA_7C / CA_38C / CA_1A-8A / CA_1A-20A / CA_3A-7A / CA_3A-8A / CA_3A-20A / CA_7A-8A / CA_7A-20A / CA_20A-32A |
| Interface | 1*RJ-45 for data, GE |
|  | 1*SIM card slot (3FF),1* power interface |
|  | 1*Hi KEY/1*Reset KEY/1*Power KEY 2*Voice + - /1*Mute KEY/1*Pause KEY |
| Main Hardware | WiFi: 802.11a/b/g/n/ac 2.4GHz 2*2 &5GHz 2*2 DBDC (AP mode only) |
|  | Support BT 4.0 |
|  | 4*Mic |
|  | 1*Speaker , output power 12W |
|  | 1) modem memory 4Gb flash +2Gb RAM 2) AI memory 2Gb flash+2Gb RAM |
| Main Software | Huawei HiLink, Huawei Histen |
|  | Alexa voice service |
|  | High quality audio, far field voice recognition |
| Power | AC/DC:AC: 100 V ~ 240 V; DC: 12 V, 2 A |

- Firmware

  - Version Description

| Firmware Version: | 8.0.1.11(H191SP3C983) |
| --- | --- |
| Baseline information: | Balong V7R22 C60B191 |
| OS | Linux 3.10.100 |

  - Improvement in the Previous Version

| Index | Description |
| --- | --- |
| 1 | 1.Web Language Default:[Spanish]. 2.IMEI svn |

  - Known Limitations and Issues

| Index | Issue Description |
| --- | --- |
| NA |  |

- Software Vulnerabilities Fixes

| Software/Module name | Version | CVE ID | Vulnerability Description | Impact Description |
| --- | --- | --- | --- | --- |
| NA | NA | NA | NA | Baseline already has finished developing with security issues, not relate to this one. |