| Product name | Confidentiality level |
| --- | --- |
| E6878-870 | CONFIDENTIAL |
| Product version | Total 11pages |
| V1.0 |  |

E6878-870-WEBUI 10.0.3.1(W11SP54C233) Release Notes V1.0

| Prepared by | E6878-870 Team | Date | 2019-12-30 |
| --- | --- | --- | --- |
| Reviewed by | E6878-870 Team | Date | 2019-12-30 |
| Approved by | E6878-870 Team | Date | 2019-12-30 |

Huawei Technologies Co., Ltd.

All rights reserved

Revision Record

| Date | Revision version | FW-WebUI/HiLink Version | Change Description | Author |
| --- | --- | --- | --- | --- |
| 2020-1-15 | 1.0 | FW E6878-870 10.0.3.1(H563SP11C233) E6878-870-WEBUI 10.0.3.1(W11SP54C233) | The 1st Version | E6878-870 Team |

Table of Contents

1	Main Features	4

2	Hardware	4

2.1	Version Description	4

2.2	Hardware Specifications	5

2.3	Improvements in the Previous Version	6

2.4	Known Limitations and Issues	6

3	Firmware	7

3.1	Version Description	7

3.2	Improvement in the Previous Version	7

4	Web UI	7

4.1	Version Description	7

4.2	Web UI Specifications	7

4.3	Known Limitations and Issues	7

5	Software Vulnerabilities Fixes	7

HUAWEI E6878-870 WEBUI Release Notes V1.0

| - Abbreviations | Description |
| --- | --- |

Main Features

- The E6878-870 supports the following standards:

- LTE FDD Cat4 (DL) packet data service of up to 150 Mbps

- LTE FDD Cat6 (DL) packet data service of up to 300 Mbps

- LTE FDD Cat5 (UL) packet data service of up to 75 Mbps

- LTE TDD Cat4 (DL) packet data service of up to 112 Mbps

- LTE TDD Cat6 (DL) packet data service of up to 224 Mbps

- LTE TDD (UL) packet data service of up to 10 Mbps

- 5G (UL) packet data service of up to 250 Mbps (if the downlink-uplink ratio is 8:2)

- 5G (UL) packet data service of up to 1.65 Gbps (if the downlink-uplink ratio is 8:2)

- SMS (SMS over SGs)

- Built-in 5G/LTE and Wi-Fi antenna

- 2.4 GHz and 5 GHz Wi-Fi (non-simultaneous)

- 5G/LTE/Wi-Fi auto offload

- Menu display

- Compatible with the HUAWEI AI Life app (Android) and HUAWEI SmartHome app (iOS)

- IPv4v6 dual stack

- Built-in DHCP Server, DNS RELAY, and NAT

- Online software upgrade

- Traffic statistics

- WPS

- 1.45 inch LCD display

- USB Type-C port

- USB Type-A port

- Compatible with Windows 7, Windows 8, Windows 8.1, Windows 10 (excluding Windows RT), MAC OS X 10.9, 10.10, 10.11 and 10.12 with latest upgrades

Hardware

Version Description

| Hardware Version: | CL2E6878M |
| --- | --- |
| Platform & Chipset: | Balong5000C11B563 |

Hardware Specifications

| Item | Specifications |  |
| --- | --- | --- |
| Technical standard | - WAN: 5G NR/LTE/3GPP Rel 15 |  |
|  | - Wi-Fi/WLAN: IEEE 802.11a/b/g/n/ac |  |
| Operating frequency | 5G: n41/ n77/ n78/ n79(4800–4900 MHz) LTE FDD: B1,B3,B5,B7,B8,B20,B28,B32 LTE TDD: B34,B38,B39,B40,B41,B42 |  |
|  | - Wi-Fi/WLAN: 2.4 GHz |  |
|  | - Wi-Fi/WLAN: 5 GHz |  |
| Transmit power | LTE: Conforms to Power Class 3 Definition |  |
|  | 5G: Conforms to Power Class 3 Definition |  |
|  | Wi-Fi/WLAN 2.4 GHz | 802.11b：14.5 dBm |
|  |  | 802.11g: 14 dBm |
|  |  | 802.11n: 14(20 MHz/40 MHz) dBm |
|  | Wi-Fi/WLAN 5 GHz | 802.11a: 14.5 dBm |
|  |  | 802.11ac: 14.5 (20 MHz)/15.5 (40 MHz)/10(80 MHz) dBm |
|  |  | 802.11n: 14.5 (20 MHz)/15.5 (40 MHz) dBm |
|  | Note: The value above represents a typical transmit power in Wi-Fi/WLAN mode, and may vary slightly by device. |  |
| Receiver sensitivity | LTE: Conforms to 3GPP |  |
|  | 5G: Conforms to 3GPP |  |
|  | Wi-Fi/WLAN 2.4 GHz | 802.11b: -85 dBm@11 Mbps |
|  |  | 802.11g: -72 dBm@54 Mbps |
|  |  | 802.11n: -70 dBm@65 Mbps |
|  | Wi-Fi/WLAN 5 GHz | 802.11a: -75 dBm@54 Mbps |
|  |  | 802.11n: -73 dBm@65 Mbps |
|  |  | 802.11ac: -64 dBm@180 Mbps |
|  | Note: The sensitivity data above represent the typical values, and may vary slightly by device. |  |
| Wi-Fi/WLAN speed | 802.11a: Up to 54 Mbps |  |
|  | 802.11b: Up to 11 Mbps |  |
|  | 802.11g: Up to 54 Mbps |  |
|  | 802.11n | HT20: Supports MCS0–MCS7; Up to 72.2 Mbps. Supports MCS8–MCS15; Up to 144.4 Mbps. HT40: Supports MCS0–MCS7; Up to 150 Mbps. Supports MCS8–MCS15; Up to 300 Mbps. |
|  | 802.11ac: Up to 867 Mbps |  |
| Battery | - Type: Rechargeable lithium battery (non-removable) |  |
|  | - Typical capacity: 8000 mAh (actual capacity may be slightly smaller) |  |
| External ports | USB Type-C port |  |
|  | USB Type-A port |  |
|  | nano-SIM card slot (4FF) |  |
| Display | 1.45 inch LCD display |  |
| Indicators | LED indicator: indicates wireless charging status |  |
| Buttons | Power button, Menu button |  |
| Antenna | - Built-in 5G/LTE antenna |  |
| Dimensions | 148 mm x 72 mm x 18 mm |  |
| Weight | Approximately 280 g |  |
| Temperature | - Operating temperature: 0°C to +35°C |  |
|  | - Storage temperature: –20°C to +60°C |  |
| Humidity | 5% to 95% (non-condensing) |  |

Improvements in the Previous Version

| Index | Case ID | Issue Description |
| --- | --- | --- |
| Hardware Version |  | CL2E6878M |
| Previous Hardware Version |  | Pre-verification Samples |
| NA |  |  |

Known Limitations and Issues

| Index | Case ID | Issue Description |
| --- | --- | --- |
| NA |  |  |

Firmware

Version Description

| Firmware Version: | E6878-870 10.0.3.1(H563SP11C233) |
| --- | --- |
| Baseline information | Balong5000C11B563 |

Improvement in the Previous Version

| Index | Case ID | Issue Description |
| --- | --- | --- |

Web UI

Version Description

| Web UI Version: | WEBUI 10.0.3.1(W11SP54C233) |
| --- | --- |

Web UI Specifications

| Item | Specifications |
| --- | --- |

Known Limitations and Issues

| Index | Case ID | Issue Description |
| --- | --- | --- |

Software Vulnerabilities Fixes

| Software/Module name | Version | CVE ID | Vulnerability Description | Impact Description |
| --- | --- | --- | --- | --- |