# Packet Tracer - Configure AAA Authentication on Cisco Routers (Instructor Version)

Instructor Note: Red font color or gray highlights indicate text that appears in the instructor copy only.

# Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway | Switch Port |
| --- | --- | --- | --- | --- | --- |
| R1 | G0/1 | 192.168.1.1 | 255.255.255.0 | N/A | S1 F0/1 |
| R1 | S0/0/0 (DCE) | 10.1.1.2 | 255.255.255.252 | N/A | N/A |
| R2 | G0/0 | 192.168.2.1 | 255.255.255.0 | N/A | S2 F0/2 |
| R2 | S0/0/0 | 10.1.1.1 | 255.255.255.252 | N/A | N/A |
| R2 | S0/0/1 (DCE) | 10.2.2.1 | 255.255.255.252 | N/A | N/A |
| R3 | G0/1 | 192.168.3.1 | 255.255.255.0 | N/A | S3 F0/5 |
| R3 | S0/0/1 | 10.2.2.2 | 255.255.255.252 | N/A | N/A |
| TACACS+ Server | NIC | 192.168.2.2 | 255.255.255.0 | 192.168.2.1 | S2 F0/6 |
| RADIUS Server | NIC | 192.168.3.2 | 255.255.255.0 | 192.168.3.1 | S3 F0/1 |
| PC-A | NIC | 192.168.1.3 | 255.255.255.0 | 192.168.1.1 | S1 F0/2 |
| PC-B | NIC | 192.168.2.3 | 255.255.255.0 | 192.168.2.1 | S2 F0/1 |
| PC-C | NIC | 192.168.3.3 | 255.255.255.0 | 192.168.3.1 | S3 F0/18 |

# Objectives

Configure a local user account on R1 and configure authenticate on the console and vty lines using local AAA.

Verify local AAA authentication from the R1 console and the PC-A client.

Configure server-based AAA authentication using TACACS+.

Verify server-based AAA authentication from the PC-B client.

Configure server-based AAA authentication using RADIUS.

Verify server-based AAA authentication from the PC-C client.

# Background / Scenario

The network topology shows routers R1, R2 and R3. Currently, all administrative security is based on knowledge of the enable secret password. Your task is to configure and test local and server-based AAA solutions.

You will create a local user account and configure local AAA on router R1 to test the console and vty logins. You will then configure router R2 to support server-based authentication using the TACACS+ protocol. Finally, you will configure router R3 to support server-based authentication using the RADIUS protocol.

The devices in the topology are initially configured with the following passwords:

Console password: ciscoconpa55

Enable secret password: ciscoenpa55

Note: IOS version 15.3 uses SCRYPT as a secure encryption hashing algorithm; however, the IOS version that is currently supported in Packet Tracer uses MD5. Always use the most secure option available on your equipment.

# Instructions

## Configure Local AAA Authentication for Console Access on R1

### Test connectivity.

Ping from PC-A to PC-B.

Ping from PC-A to PC-C.

Ping from PC-B to PC-C.

### Configure a local username on R1.

Configure a username of Admin1 with a secret password of admin1pa55.

R1(config)# username Admin1 secret admin1pa55

### Configure local AAA authentication for console access on R1.

Enable AAA on R1 and configure AAA authentication for the console login to use the local database.

R1(config)# aaa new-model

R1(config)# aaa authentication login default local

### Configure the line console to use the defined AAA authentication method.

Enable AAA on R1 and configure AAA authentication for the console login to use the default method list.

R1(config)# line console 0

R1(config-line)# login authentication default

### Verify the AAA authentication method.

Verify the user EXEC login using the local database.

R1(config-line)# end

%SYS-5-CONFIG_I: Configured from console by console

R1# exit

R1 con0 is now available

Press RETURN to get started.

************ AUTHORIZED ACCESS ONLY *************

UNAUTHORIZED ACCESS TO THIS DEVICE IS PROHIBITED.

User Access Verification

Username: Admin1

Password: admin1pa55

R1>

## Configure Local AAA Authentication for vty Lines on R1

### Configure domain name and crypto key for use with SSH.

      - Use ccnasecurity.com as the domain name on R1.

R1(config)# ip domain-name ccnp_v8.com

      - Create an RSA crypto key using 1024 bits.

R1(config)# crypto key generate rsa

Choose the size of the key modulus in the range of 360 to 2048 for your

General Purpose Keys. Choosing a key modulus greater than 512 may take

a few minutes.

How many bits in the modulus [512]: 1024

% Generating 1024 bit RSA keys, keys will be non-exportable...[OK]

### Configure a named list AAA authentication method for the vty lines on R1.

Configure a named list called SSH-LOGIN to authenticate logins using local AAA.

R1(config)# aaa authentication login SSH-LOGIN local

### Configure the vty lines to use the defined AAA authentication method.

Configure the vty lines to use the named AAA method and only allow SSH for remote access.

R1(config)# line vty 0 4

R1(config-line)# login authentication SSH-LOGIN

R1(config-line)# transport input ssh

R1(config-line)# end

### Verify the AAA authentication method.

Verify the SSH configuration SSH to R1 from the command prompt of PC-A.

C:\> ssh –l Admin1 192.168.1.1

Open

Password: admin1pa55

R1>

## Configure Server-Based AAA Authentication Using TACACS+ on R2

### Configure a backup local database entry called Admin.

For backup purposes, configure a local username of Admin2 and a secret password of admin2pa55.

R2(config)# username Admin2 secret admin2pa55

### Verify the TACACS+ Server configuration.

      - Ping the TACACS+ Server to verify that it is reachable from R2.

      - Click the TACACS+ Server. On the Services tab, click AAA. Notice that there is a Network configuration entry for R2 and a User Setup entry for Admin2.

### Configure the TACACS+ server specifics on R2.

Configure the AAA TACACS server IP address and secret key on R2.

Note: The commands tacacs-server host and tacacs-server key are deprecated. Currently, Packet Tracer does not support the new command tacacs server.

R2(config)# tacacs-server host 192.168.2.2

R2(config)# tacacs-server key tacacspa55

### Configure AAA login authentication for console access on R2.

Enable AAA on R2 and configure all logins to authenticate using the AAA TACACS+ server. If it is not available, then use the local database.

R2(config)# aaa new-model

R2(config)# aaa authentication login default group tacacs+ local

### Configure the line console to use the defined AAA authentication method.

Configure AAA authentication for console login to use the default AAA authentication method.

R2(config)# line console 0

R2(config-line)# login authentication default

### Verify the AAA authentication method.

Verify the user EXEC login using the AAA TACACS+ server.

R2(config-line)# end

%SYS-5-CONFIG_I: Configured from console by console

R2# exit

R2 con0 is now available

Press RETURN to get started.

************ AUTHORIZED ACCESS ONLY *************

UNAUTHORIZED ACCESS TO THIS DEVICE IS PROHIBITED.

User Access Verification

Username: Admin2

Password: admin2pa55

R2>

## Configure Server-Based AAA Authentication Using RADIUS on R3

### Configure a backup local database entry called Admin.

For backup purposes, configure a local username of Admin3 and a secret password of admin3pa55.

R3(config)# username Admin3 secret admin3pa55

### Verify the RADIUS Server configuration.

Click the RADIUS Server. On the Services tab, click AAA. Notice that there is a Network configuration entry for R3 and a User Setup entry for Admin3.

### Configure the RADIUS server specifics on R3.

Configure the AAA RADIUS server IP address and secret key on R3. Configure the server name as RAD-SERVER.

R3(config)# radius server RAD-SERVER

R3(config-radius-server)# address ipv4 192.168.3.2

R3(config-radius-server)# key radiuspa55

Note: Packet Tracer does not correctly score configuration using the radius server command.

### Configure AAA login authentication for console access on R3.

Enable AAA on R3 and configure all logins to authenticate using the AAA RADIUS server. If it is not available, then use the local database.

R3(config)# aaa new-model

R3(config)# aaa authentication login default group radius local

### Configure the line console to use the defined AAA authentication method.

Configure AAA authentication for console login to use the default AAA authentication method.

R3(config)# line console 0

R3(config-line)# login authentication default

### Verify the AAA authentication method.

Verify the user EXEC login using the AAA RADIUS server.

R3(config-line)# end

%SYS-5-CONFIG_I: Configured from console by console

R3# exit

R3 con0 is now available

Press RETURN to get started.

************ AUTHORIZED ACCESS ONLY *************

UNAUTHORIZED ACCESS TO THIS DEVICE IS PROHIBITED.

User Access Verification

Username: Admin3

Password: admin3pa55

R3>

### Check results.

Your completion percentage should be 100%. Click Check Results to see feedback and verification of which required components have been completed.

End of document

# !!!Script for R1

!!!Part 1

config t

username Admin1 secret admin1pa55

aaa new-model

aaa authentication login default local

line console 0

login authentication default

!!!Part 2

ip domain-name ccnp_v8.com

crypto key generate rsa

1024

aaa authentication login SSH-LOGIN local

line vty 0 4

login authentication SSH-LOGIN

transport input ssh

# !!!!Script for R2

conf t

username Admin2 secret admin2pa55

tacacs-server host 192.168.2.2

tacacs-server key tacacspa55

aaa new-model

aaa authentication login default group tacacs+ local

line console 0

login authentication default

# !!!!Script for R3

conf t

username Admin3 secret admin3pa55

radius server RAD-SERVER

address ipv4 192.168.3.2

key radiuspa55

aaa new-model

aaa authentication login default group radius local

line console 0

login authentication default