# Packet Tracer - Configure OSPF Advanced Features (Instructor Version)

Instructor Note: Red font color or gray highlights indicate text that appears in the instructor copy only.

# Addressing Table

| Device | Interface | IPv4 Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/0 | 172.16.1.1 | 255.255.255.0 | N/A |
| R1 | S0/0/0 | 172.16.3.1 | 255.255.255.252 | N/A |
| R1 | S0/0/1 | 192.168.10.5 | 255.255.255.252 | N/A |
| R2 | G0/0 | 172.16.2.1 | 255.255.255.0 | N/A |
| R2 | S0/0/0 | 172.16.3.2 | 255.255.255.252 | N/A |
| R2 | S0/0/1 | 192.168.10.9 | 255.255.255.252 | N/A |
| R2 | S0/1/0 | 209.165.200.225 | 255.255.255.224 | N/A |
| R3 | G0/0 | 192.168.1.1 | 255.255.255.0 | N/A |
| R3 | S0/0/0 | 192.168.10.6 | 255.255.255.252 | N/A |
| R3 | S0/0/1 | 192.168.10.10 | 255.255.255.252 | N/A |
| PC1 | NIC | 172.16.1.2 | 255.255.255.0 | 172.16.1.1 |
| PC2 | NIC | 172.16.2.2 | 255.255.255.0 | 172.16.2.1 |
| PC3 | NIC | 192.168.1.2 | 255.255.255.0 | 192.168.1.1 |

# Objectives

Part 1: Modify OSPF Default Settings

Part 2: Verify Connectivity

# Scenario

In this activity, OSPF is already configured and all end devices currently have full connectivity. You will modify the default OSPF routing configurations by changing the hello and dead timers and adjusting the bandwidth of a link. Then you will verify that full connectivity is restored for all end devices.

# Instructions

## Modify OSPF Default Settings

### Test connectivity between all end devices.

Before modifying the OSPF settings, verify that all PCs can ping the web server and each other.

### Adjust the hello and dead timers between R1 and R2.

    - Enter the following commands on R1.

R1(config)# interface s0/0/0

R1(config-if)# ip ospf hello-interval 15

R1(config-if)# ip ospf dead-interval 60

    - After a short period of time, the OSPF connection with R2 will fail. Both sides of the connection need to have the same timers for the adjacency to be maintained. Adjust the timers on R2.

### Adjust the bandwidth setting on R1.

Trace the path between PC1 and the web server located at 64.100.1.2. Notice that the path from PC1 to 64.100.1.2 is routed through R2. OSPF prefers the lower cost path.

On the R1 Serial 0/0/0 interface, set the bandwidth to 64 Kb/s. This does not change the actual port speed, only the metric that the OSPF process on R1 will use to calculate best routes.

R1(config-if)# bandwidth 64

Trace the path between PC1 and the web server located at 64.100.1.2. Notice that the path from PC1 to 64.100.1.2 is redirected through R3. OSPF prefers the lower cost path.

## Verify Connectivity

Verify all PCs can ping the web server and each other.

End of document

# Answer Scripts

# Router R1

interface Serial0/0/0

bandwidth 64

ip ospf hello-interval 15

ip ospf dead-interval 60

# Router R2

interface Serial0/0/0

ip ospf hello-interval 15

ip ospf dead-interval 60