[image: image1.png]


[image: image2.png]COMPAQ
“Your partner through time”

000
Ready



QuickSpecs

Overview

QuickSpecs

Overview

QuickSpecs

Overview


[image: image3.png]COMPAQ
“Your partner through time”

000
Ready





	Processor

	Alpha 21264 500 MHz or 667 MHz processor

	Cache Memory

	4-MB level 2 writeback cache per 500 MHz processor 

        8-MB level 2 Dual Data Rate cache per 667 MHz processor

	Upgradability

	System upgradable to dual processing

	Memory

	Standard
	256-MB, 512-MB, or 1-GB ECC SDRAM memory standard

	Maximum
	4 GB

	Network Controller

	Choice of Ethernet controllers

	Expansion

	I/O (Total)
	6

	PCI
	5

	Shared PCI/ISA
	1

	Storage Controller

	Choice of storage controllers

	Storage

	Diskette Drive
	1.44 MB

	CD-ROM
	24X Max IDE CD-ROM Drive

	Internal Hard Drives
	Choice of 9.1 GB, or 18.2 GB

	Maximum Internal Storage
	72 GB

	Expansion Ports

	Parallel

	1

	Serial

	2

	Pointing Device (Mouse)

	1

	Keyboard

	1

	SCSI knockouts on rear of chassis       
	4

	USB

	2 (future support)

	Form Factor

	Rackmount or Tower

	Manageability

	Compaq Insight Manager

	Hot plug redundant power supply support; hot plug fan support

	Power Supply

	Two 375-Watt Hot Plug Power Supplies, standard; optional third N+1 for redundancy

	OS Support

	Tru64 UNIX AlphaServers include pre-installed software, base license, unlimited user license, server extension license, Open Source Internet Solutions, and Netscape Enterprise Server 3.0. Tru64 UNIX AlphaStations include a base 2-user and Compaq Open3D license.

OpenVMS AlphaServer include pre-installed software, base license, and Enterprise Integration Server License Package V3.0A. OpenVMS AlphaStations include an OpenVMS base license, concurrent use 1-user license, and Compaq Open3D license.

Minimum OS support: Tru64 UNIX V4.0F, OpenVMS V7.1-2 or V7.2-1

	Service and Support

	Compaq Services provides a wide range of service offerings including a three-year limited warranty2 fully supported by a worldwide network of resellers and service providers; lifetime toll-free 7 x 24 hardware technical phone support.


	Step 1 ( Select system 

	AlphaServer DS20E systems require the mandatory selection of the following items:

· Pedestal/tower or rackmount bezel kit

· Disk drive cage

· Ultra2 SCSI adapter 

· System disk drive

· Ethernet adapter

· Power cord

· Software media and documentation for first system onsite

Notes: DS20E minimum OS support (Tru64 UNIX V4.0F, OpenVMS V7.1-2 or V7.2-1
Keyboard not included 
For system integration of options with 6-3 part numbers, contact CustomSystems 

	

	AlphaServers

	OS
	CPU
	Memory / Cache
	Order No.

	Tru64 UNIX
	500 MHz
	256 MB / 4MB
	DA-56NRA-DA

	Tru64 UNIX
	500 MHz
	512 MB / 4MB
	DA-56NRA-EA

	Tru64 UNIX
	500 MHz
	1 GB / 4MB
	DA-56NRA-FA

	

	Tru64 UNIX
	667 MHz
	256 MB / 8MB
	DA-56PAA-DA

	Tru64 UNIX
	667 MHz
	512 MB / 8MB
	DA-56PAA-EA

	Tru64 UNIX
	667 MHz
	1 GB / 8MB
	DA-56PAA-FA

	
	
	
	

	OpenVMS
	500 MHz
	256 MB / 4MB
	DY-56NRA-DA

	OpenVMS
	500 MHz
	512 MB / 4MB
	DY-56NRA-EA

	OpenVMS
	500 MHz
	1 GB / 4MB
	DY-56NRA-FA

	

	OpenVMS
	667 MHz
	256 MB / 8MB
	DY-56PAA-DA

	OpenVMS
	667 MHz
	512 MB / 8MB
	DY-56PAA-EA

	OpenVMS
	667 MHz
	1 GB / 8MB
	DY-56PAA-FA

	
	
	
	

	AlphaStations

	OS
	CPU
	Memory / Cache
	Order No.

	Tru64 UNIX
	500 MHz
	256 MB / 4MB
	DA-56NRA-DW

	Tru64 UNIX
	500 MHz
	512 MB / 4MB
	DA-56NRA-EW

	Tru64 UNIX
	500 MHz
	1 GB / 4MB
	DA-56NRA-FW

	

	Tru64 UNIX
	667 MHz
	256 MB / 8MB
	DA-56PAA-DW

	Tru64 UNIX
	667 MHz
	512 MB / 8MB
	DA-56PAA-EW

	Tru64 UNIX
	667 MHz
	1 GB / 8MB
	DA-56PAA-FW

	
	
	
	

	OpenVMS
	500 MHz
	256 MB / 4MB
	DY-56NRA-DW

	OpenVMS
	500 MHz
	512 MB / 4MB
	DY-56NRA-EW

	OpenVMS
	500 MHz
	1 GB / 4MB
	DY-56NRA-FW

	

	OpenVMS
	667 MHz
	256 MB / 8MB
	DY-56PAA-DW

	OpenVMS
	667 MHz
	512 MB / 8MB
	DY-56PAA-EW

	OpenVMS
	667 MHz
	1 GB / 8MB
	DY-56PAA-FW

	Step 2 ( Select mandatory options

	Step 2a ( Bezel Kits ( Mandatory

	· Pedestal/Tower kit or Rackmount kit is mandatory for each system

	Pedestal/Tower bezel kit for 500 MHz systems
	3X-BA56P-PA

	Rackmount bezel kit (M-Series Cabinets) for 500 MHz systems
	3X-BA56R-RA

	Pedestal/Tower bezel kit for 667 MHz systems
	3X-BA56P-PB

	Rackmount bezel kit (M-Series Cabinets) for 667 MHz systems
	3X-BA56R-RC

	
	

	Step 2b ( Disk Drive Cage ( Mandatory 

	· Systems support one disk drive cage; disk drive cage supports four 3.5” hard disk drives

	· Each drive cage includes Ultra Universal drive height converter (4-pack) to install 1.0” Universal drives. 

	4-bay disk drive cage
	3X-BA611-4D

	

	Step 2c ( Ultra2 SCSI Adapter ( Mandatory

	· Selection of Ultra2 SCSI adapter to connect to internal disk drive cage is mandatory; SCSI cable included with system

	Single channel LVD adapter, maximum 4 per system
	3X-KZPCA-AA

	
	

	Step 2d( System Disk Drive (Supported in Disk Drive Cage) ( Mandatory

	· Selection of one disk drive is mandatory

	9.1-GB 7200 RPM 1.0( Universal Ultra2 SCSI disk drive
	3R-A0525-AA

	9.1-GB 10000 RPM 1.0( Universal Ultra2 SCSI disk drive
	3R-A0526-AA

	18.2-GB 7200 RPM 1.0( Universal Ultra2 SCSI disk drive
	3R-A0527-AA

	18.2-GB 10000 RPM 1.0( Universal Ultra2 SCSI disk drive
	3R-A0561-AA

	Ultra Universal drive height converter, 4-pack (included in disk drive cage). Customers with earlier 4-slot drive cages (3X-BA611-4D) need to order separately when installing 1” Universal drives
	3R-A0528-AA

	
	

	Step 2e ( Ethernet Adapters ( Mandatory

	· Selection of one Ethernet adapter is mandatory (Ethernet adapter is not required if KZPCM adapter is selected)

	· See Step 12 for additional networking options

	PCI 1-port 10/100 Mbit Fast Ethernet adapter, UTP, maximum four per system
	3X-DE600-AA

	PCI 2-port 10/100 Mbit, Fast Ethernet adapter, UTP, maximum two per system
	3X-DE602-AA

	Multi-mode fiber (MMF) add-on daughter card for 3X-DE602-AA
	3X-DE602-FA

	PCI 10/100 Mbit 4-port Ethernet adapter, UTP, maximum one per system   
	DE504-BA

	Cable for 10BaseT twisted pair connection
	BN25G-xx


	

	Step 2f ( Power Cords ( Mandatory

	· Selection of power cord is mandatory for all systems

	Pedestal Power Cords

	North American 120V, 75(
	BN26J-1K

	Australia, New Zealand, 2.5m
	BN19H-2E

	Central Europe, 2.5m
	BN19C-2E

	UK, Ireland, 2.5m
	BN19A-2E

	Switzerland, 2.5m
	BN19E-2E

	Denmark, 2.5m
	BN19K-2E

	Italy, 2.5m
	BN19M-2E

	Egypt, India, South Africa, 2.5m
	BN19S-2E

	Israel, 2.5m
	BN18L-2E

	Japan, 2.5m
	3X-BN46F-02

	
	

	Rackmount Power Cords

	US/Canada 
	BN20Z-4E

	Non-US 
	BN35S-4E

	Japan, 2.5m
	3X-BN46D-4E

	

	Step 3 ( Additional CPU Modules ( Optional 

	· One 6/500-MHz or 6/667-MHz CPU included, one additional CPU supported. Second CPU must be the same speed. A 500 MHz system can be upgraded to 667 MHz by swapping out the 500 MHz CPU(s)

	SMP upgrade includes one 6/500-CPU with 4-MB cache and Tru64 UNIX SMP license
	KN310-BB

	SMP upgrade includes one 6/500-CPU with 4-MB cache and OpenVMS SMP license
	KN310-BC

	SMP upgrade includes one 6/667-CPU with 8-MB cache and Tru64 UNIX SMP license
	KN311-BB

	SMP upgrade includes one 6/667-CPU with 8-MB cache and OpenVMS SMP license
	KN311-BC

	

	Step 4 ( Memory ( Optional

	· System includes one memory option; up to three additional memory options can be added

	256-MB memory option (4 x 64 MB DIMMs)
	MS340-CA

	512-MB memory option (4 x 128 MB DIMMs)
	MS340-DA

	1-GB memory option (4 x 256 MB DIMMs)
	MS340-EA

	

	Step 5 ( Additional Power Supplies ( Optional

	· Two 375 watt power supplies included with system; optional third power supply provides N+1 redundancy

	DS20E redundant power supply
	FR-H7910-AA

	

	Step 6( Internal Removable Storage ( Optional

	· System supports three internal removable media devices:
One combo 24X CD-ROM drive and 1.44 MB disk drive
One additional removable media slot provides support for one 5.25( half-height device; 
Requires KZPBA-CA or KZPCM-DA  UltraSCSI adapter, or 3X-KZPCA-AA Ultra2 (LVD) adapter, and SCSI cable (3X-BC56C-2F). For optimum performance it is recommended that Ultra2 capable devices be connected to an Ultra2 adapter  

	600-MB 32X CD-ROM drive (RRD47) 5.25( half-height, top gun blue
	RRD47-VD

	12/24-GB 4 mm DAT SCSI tape drive (TLZ10) 5.25( half-height, top gun blue
	TLZ10-LB

	5.25” half-height 20/40-GB 4 mm DAT Ultra 2 (LVD) SCSI tape drive, top gun blue
	3X-SD20X-LB

	5.25” half-height 35/70-GB 8 mm AIT Wide Ultra SCSI tape drive, top gun Blue
	    3X-SZ35X-LB

	5.25” half-height 50/100-GB 8 mm AIT Ultra 2 (LVD) SCSI tape drive, top gun blue
	3X-SZ50X-LB

	SCSI cable with one connector and active terminator to connect optional PCI adapter to 5.25( device
	3X-BC56C-2F

	

	Step 7 ( M-series Cabinets ( Optional 

	M-series enclosures require selection of the following items:

	· H9A10 or H9A15 cabinet, 120V or 240V

	· Rack kit for each system

	· H9A10 or H9A15 rear cabinet extender

	· Cabinet front door based on cabinet height

	· Factory integration

	· Ballast kit if one or two systems are ordered for the H9A15 cabinet

	· Power Cord

	Note: For more information on M-Series Cabinet Enclosures, see www.digital.com/alphaserver/products/m-series/index.html

	

	Cabinet Configuration Chart for AlphaServer DS20E (5U) with StorageWorks Shelves (4U)

	Cabinet
	Rack Kit
	Cabinet Height/Space
	Systems
	Shelves

EV6        EV67
	Ballast Kit

	H9A10-MC/MD
	3X-BA56R-RA / RC
	56” / 32U
	1
	6                6
	-

	
	
	
	2
	6                6
	-

	
	
	
	3
	6                6
	-

	
	
	
	4
	4                6
	-

	
	
	
	5
	0                2
	-

	
	
	
	6 (EV67 667   MHz only)
	0                0
	

	H9A15-MC/MD
	3X-BA56R-RA / RC
	66.5” / 39U
	1
	6                6
	1

	
	
	
	2
	6                6
	1

	
	
	
	3
	6                6
	-

	
	
	
	4
	6                6
	-

	
	
	
	5
	4                6
	-

	
	
	
	6
	0                4
	-

	Note:  1.75” or 1U space is required between each system unit for EV6 500 MHz systems

	

	Cabinet Configuration Chart for AlphaServer DS20E (5U) with StorageWorks RA3000 and Disk Shelves (12U)

	· 12U of vertical cabinet space is reserved for one RA3000 controller shelf and up to five disk shelves mounted in top of cabinet, front to back; an additional 2U of vertical space is required for RA3000 controller shelf UPS.

	· Disk shelves can be a combination of RA3000 DS-SWXRA-GN disk shelves and StorageWorks BA36R-RC/RD disk shelves.

	· RA3000 controller shelf supports a maximum of four DS-SWXRA-GN disk shelves; third and fourth DS-SWXRA-GN disk shelf ordered will be installed in rear of M-series cabinet; rear mounted shelves require that a 2-m SCSI cable (BN37A-02) be ordered separately.

	· Systems are supported in remaining vertical space.

	· 1.75( or 1U space is required between each system unit for EV6 500 MHz systems

	Cabinet
	Mounting Kit
	Cabinet Height / Space
	Systems
	RA3000 + Shelves
	Ballast Kit

	H9A10-MC/MD
	3X-BA56R-RA / RC
	54.2( / 32U
	1
	1 + 5
	-

	
	
	
	2
	1 + 5
	-

	
	
	
	3
	1 + 5
	-

	H9A15-MC/MD
	3X-BA56R-RA / RC
	66.5( / 39U
	1
	1 + 5
	1

	
	
	
	2
	1 + 5
	1

	
	
	
	3
	1 + 5
	-

	
	
	
	4
	1 + 5
	-

	

	Cabinet Configuration Chart for AlphaServer DS20E (5U) with StorageWorks BA370 (20U) and Disk Shelves

	· 20U of vertical cabinet space is reserved for one front-mounted BA370 rackmount enclosure, or one front-mounted and one rear-mounted BA370 enclosure in top of cabinet, front to back. Note that top fan (1U) is removed from cabinet when BA370 rackmount enclosure is installed.

	· Systems and StorageWorks shelves are supported in remaining vertical space; StorageWorks shelves are not supported in rear of cabinet behind a front-mounted BA370 rackmount enclosure.

	· 1.75( or 1U space is required between each system unit.

	Cabinet
	Rack Kit
	Cabinet Height/Space
	Systems
	BA370
	Shelves

	H9A10-MC/MD
	3X-BA56R-RA / RC
	 
	1 
	1 (front) or
	2

	
	     
	
	2
	2 (front & back)
	0

	H9A15-MC/MD
	3X-BA56R-RA / RC
	66.5” / 40U
	1
	1 (front) or
	6

	
	      
	
	2
	2 (front & back)
	4

	
	
	
	3
	
	0

	Note: Installing one BA370 and one system in an H9A15 cabinet requires the mandatory selection of a ballast kit; a ballast kit is not required if two BA370s and one system are installed in H9A15 cabinet.

	M-series Cabinets

	67" cabinet with 56" or 32U of usable vertical space, 120V dual-24A controller, each with 10 NEMA 5-15R outlets, input power plug NEMA L5-30P, Americas, Asia Pacific; includes rear door and top fan tray
	H9A10-MC

	67" cabinet with 56" or 32U of usable vertical space, 240V dual-16A controller, each with 12 IEC C13 outlets, input power plug IEC 309, 16A, Europe; includes rear door and top fan tray
	H9A10-MD

	79" cabinet with 66.5" or 39U of usable vertical space, 120V dual-24A controller, each with 10 NEMA 5-15R outlets, input power plug NEMA L5-30P, Americas, Asia Pacific; includes rear door and top fan tray
	H9A15-MC

	79" cabinet with 66.5" or 39U of usable vertical space, 240V dual-16A controller, each with 12 IEC C13 outlets, input power plug IEC 309, 16A, Europe; includes rear door and top fan tray
	H9A15-MD

	

	Rear Cabinet Extender ( Required selection of one cabinet extender for each H9A10 or H9A15 cabinet

	Rear cabinet extender, top gun blue, for H9A10-MC/MD cabinets
	H9C10-TE

	Rear cabinet extender, top gun blue, for H9A15-MC/MD cabinets
	H9C15-TE

	

	Cabinet Front Door ( Required selection of door based on cabinet height 

	Front door, top gun blue, for H9A10-MC/MD cabinet enclosure
	H9C10-TF

	Front door, top gun blue, for H9A15-MC/MD cabinet enclosure
	H9C15-TF

	

	Factory Integration ( Required

	Factory Integration of systems and storage devices assembled and configured in cabinet enclosure in predefined locations
	YS-ASCAA-AA

	

	Ballast Kit  ( Required only if one system is ordered for the H9A15 cabinet

	· Ballast is added to a cabinet at particular locations for specific configurations to meet safety regulations for installation, operation, and service; see Cabinet Configuration Chart

	Two 35-40 lb. Steel plates
	H9C10-MC

	

	Rackmount  Power Cord ( Required for each system

	US/Canada 
	BN20Z-4E

	Non-US 
	BN35S-4E

	Japan, 2.5m
	3X-BN46D-4E

	

	Shelves Supported in M-series Cabinets ( Optional

	UltraSCSI single-ended RETMA rack, single-bus personality module, 180W power supply
	BA36R-RC

	UltraSCSI single-ended RETMA rack, split-bus personality module, 180W power supply
	BA36R-RD

	180W power supply, includes 120V North American and 240V IEC power cord
	CK-BA35X-HH

	

	Compaq CustomSystems Support

	· Cabinets containing systems and storage must be assembled by qualified assemblers: Compaq Manufacturing, Compaq CustomSystems, or an authorized Compaq Certified Integration Partner.

· For custom integration of systems, options, cabinet expansion offerings, and additional Compaq and third party devices, or for other cabinets, doors, and trim kits of a different color, contact Compaq CustomSystems at 1-800-AT COMPAQ, or your local authorized Compaq Certified Integration Partner.

	

	Step 8 (External Storage ( Optional

	UltraSCSI Configuration Rules

	· UltraSCSI configurations require UltraSCSI components (controllers, adapters, shelves, disks, and cables)

	· UltraSCSI adapters and RAID controllers support UltraSCSI disks at UltraSCSI speeds in UltraSCSI top gun blue shelves (DS-BA356-xx)

	UltraSCSI StorageWorks Pedestals

	· UltraSCSI top gun blue pedestals provide support for UltraSCSI disk drives; pedestal is available in single-bus or split-bus configurations and includes a personality module and 180W power supply

	· UltraSCSI shelf supports: seven 3.5( hard disk drives, or one 5.25( device and four 3.5( hard disk drives, or two 5.25( devices and one 3.5( hard disk drive

	· Installation of redundant power supply reduces number of 3.5( drive slots by one

	UltraSCSI single-ended pedestal, single-bus personality module, 180W power supply
	DS-BA356-KF

	UltraSCSI single-ended pedestal, split-bus personality module, 180W power supply
	DS-BA356-KG

	UltraSCSI pedestal, single-channel differential personality module, 180W power supply
	DS-BA356-KH

	180W power supply, includes 120V North American and 240V IEC power cord
	CK-BA35X-HH

	

	Tabletop Tape Drives

	· Tape drives require UltraSCSI adapter and country-specific power cord

	12/24-GB 4-mm DAT narrow single-ended tabletop SCSI tape, includes 120V North American power cord, requires BN31W-xx SCSI cable
	DS-TLZ10-DB

	35/70-GB DLT single-ended tabletop tape drive, requires BN21K-xx SCSI cable
	DS-TZ89N-TA

	50/100-GB US AIT tabletop tape drive  (includes SCSI cable and power cord)
	157767-001

	50/100-GB International AIT tabletop tape drive  (includes SCSI cable and power cord)
	157767-B31

	50/100-GB Japan AIT tabletop tape drive  (includes SCSI cable and power cord)
	157767-291

	

	Storage Controllers/Adapters for External StorageWorks Shelves

	· Storage controllers require Tru64 UNIX V4.0F or OpenVMS V7.1-2

	Note: The following controllers are only supported with StorageWorks shelves (not supported with the 3X-BA611-4D disk drive cage) 

	

	Maximum # Supported
	

	
	Tru64 UNIX
	OpenVMS
	

	PCI Fibre Channel adapter (KGPSA-BC), (uses one slot)
	4
	4
	380574-001

	Fibre Channel cable (BNGBX-xx) xx=02, 05, 15, 30, 50 meters
x=1, 2, 3, 4, 5
	
	234457-B2x

	

	PCI 2-port UltraSCSI single-ended adapter and 10/100 Mb Ethernet combo adapter (uses one PCI slot); includes internal 68-pin and 50-pin HD connectors and two external 68-pin VHDCI SCSI connectors, requires BN37A-xx cable to connect to single-ended UltraSCSI shelf
	4
	4
	KZPCM-DA

	VHDCI male-to-VHDCI male UltraSCSI cable, 
xx = 02, 2E [2.5], 03, 05, 10, 20 m
	
	BN37A-xx

	

	PCI 1-port UltraSCSI single-ended adapter (uses one PCI slot); includes  internal 68-pin and 50-pin HD connectors and external 68-pin HD connector, requires BN38C-xx cable (2 to 20 m) to connect to single-ended UltraSCSI shelf
	4
	4
	KZPBA-CA

	PCI 1-port UltraSCSI differential adapter  (uses one PCI slot); includes external 68-pin HD connector, requires BN38C-xx cable (2 to 25 m) to connect adapter to differential UltraSCSI shelf or RAID controller
	4
	4
	KZPBA-CB

	68-pin HD male-to-VHDCI male UltraSCSI cable; 
xx = 02, 03, 05, 10, 20, 25 m
	
	BN38C-xx

	

	PCI 1-port UltraSCSI RAID controller with 4-MB cache memory (uses one PCI slot), includes RAID Array 230/Plus Subsystem software and documentation kit; includes VHDCI female connector; tape drives not supported; 18.2-GB disk drives not supported, requires BN37A-xx cable (.3 to 20 m) to connect to single-ended UltraSCSI shelf
	4
	4
	KZPAC-AA

	PCI 3-port UltraSCSI RAID controller with 4-MB cache memory (uses one PCI slot), includes RAID Array 230/Plus Subsystem Software and documentation kit; tape drives not supported; 18.2-GB disk drives not supported; SCSI cable required for each active port; requires BN37X-xx cable (.3 to 20 m) to connect to single-ended UltraSCSI shelf, and BN31K-0E for external third port connection
	2
	2
	KZPAC-CA

	Same as KZPAC-CA with 8-MB cache memory
	2
	2
	KZPAC-CB

	Cache battery backup for KZPAC controller
	
	KZPSC-UB

	Factory configured RAID of KZPAC with StorageWorks shelves
Note: Factory configured RAID sets for disks in single-bus StorageWorks shelves connected to KZPAC require completion of questionnaire located at http://www.digital.com/alphaserver/fcr
	
	YS-ASDRD-BA

	VHDCI male-to-VHDCI male UltraSCSI cable;
xx = 02, 2E [2.5], 03, 05, 10, 20, 25 m 
Note: Manufacturing may substitute BN37A-02 cable for specific cabinet configurations.
	
	BN37A-xx

	SCSI cable/bulkhead assembly kit, supports one third-port connection from  KZPAC-CA/CB, allows external connection of one third port
	
	BN31K-0E

	SCSI cable/bulkhead assembly kit, supports two third-port connections  from KZPAC-CA/CB, allows external connection of two third ports
	
	KZPAC-SB

	

	Removable Media Devices for External StorageWorks Shelves

	· For maximum performance, tape libraries must be configured with Fast Wide Differential (FWD) controllers for tape backup

	12/24-GB 4 mm DAT narrow SCSI tape drive in StorageWorks carrier
	DS-TLZ10-VA

	35/70-GB DLT single-ended tape drive in StorageWorks carrier
	DS-TZ89N-VW

	35/70-GB AIT tape drive in StorageWorks carrier
	DS-AIT35-VW

	600-MB 32X CD-ROM drive in StorageWorks carrier
	DS-RRD47-VA

	

	BA370 Rackmount Enclosure

	· Mandatory rear cabinet extender ordered for systems installed in M-series cabinets (H9A10 and H9A15) allows two BA370 enclosures to be installed front to back in cabinet.

	· Host adapter, RAID Array controller, cache battery, platform specific Solutions Software Kit, and disk drives must be ordered separately.

	BA370 front mount rackmount enclosure with five 180W power supplies, includes RETMA rackmount kit; first BA370 enclosure must be front mounted in cabinet, 20U height
	2T-BA370-AS

	BA370 rear mount rackmount enclosure with five 180W power supplies, includes RETMA rackmount kit; second BA370 enclosure must be rear mounted in cabinet, 20U height
	2T-BA370-AR

	Factory configured RAID of rackmounted BA370/HSZ70
	YS-ASCRD-AA

	Note: Factory configured RAID sets for disks in rackmounted BA370 StorageWorks shelf controlled by one or two HSZ70s require completion of questionnaire located at http://www.digital.com/alphaserver/fcr

	

	StorageWorks Shelf ( Requires selection of UltraSCSI adapter and SCSI cable

	StorageWorks UltraSCSI single-bus shelf kit
	BA36R-RC

	StorageWorks UltraSCSI split-bus shelf kit
	BA36R-RD

	

	External Disk Drive  (Supported in StorageWorks Shelf)

	9.1-GB 7200 RPM UltraSCSI drive
	DS-RZ1DA-VW

	18.2-GB 7200 RPM UltraSCSI drive
	DS-RZ1EA-VW

	36.4-GB 7200 RPM UltraSCSI drive
	DS-RZ1FB-VW

	9.1-GB 10000 RPM UltraSCSI drive
	DS-RZ1DD-VW

	18.2-GB 10000 RPM UltraSCSI drive
	DS-RZ1ED-VW

	36.4-GB 10,000 rpm UltraSCSI drive
	DS-RZ1FC-VW

	Note- A maximum of (6) 18.2-GB and 36.4 GB 10000 RPM drives can be configured in a Storageworks shelf 

	Step 9 (UltraSCSI RAID Array Controllers and UltraSCSI RAID Packaged Solutions (Optional

	RAID Array 3000 Rackmount Controller Shelf 

	· Selection of RA3000 Solutions Software Kit is required

	· Supported SCSI adapter: KZPBA-CB PCI 1-port UltraSCSI differential adapter

	RA3000 rackmount controller shelf, blue, with one controller, 110V 
Includes: UltraSCSI BA356 controller shelf, one HSZ22 2-channel controller with 16-MB cache, integrated Environmental Monitoring Unit (EMU), two 180W power supplies, two dual-port host differential UltraSCSI I/O SBBs, dual-channel UltraSCSI single-ended personality modules, one 120V 1000VA rackmount UPS, 5-m host SCSI cable (BN37A-05), BN38E-0B adapter, UPS serial cable, PC connection serial cable (DB-9), two 68-pin HD tri-link adapters (H8861-AA), two North American power cords, and RETMA mounting bracket (disk shelves and disks not included)
Requires: Solutions Software Kit for platform, host adapter, RA3000 disk shelf, and disks
Options: Second HSZ22 controller, up to four additional RA3000 disk shelves, cache memory upgrade
	DS-SWXRA-GH

	RA3000 rackmount controller shelf, blue, with one controller, 230V 
Includes: UltraSCSI BA356 controller shelf, one HSZ22 2-channel controller with 16-MB cache, integrated Environmental Monitoring Unit (EMU), two 180W power supplies, two dual-port host differential UltraSCSI I/O SBBs, dual-channel UltraSCSI single-ended personality modules, one 230V 1000VA rackmount UPS, 5-m host SCSI cable (BN37A-05), BN38E-0B adapter, UPS serial cable, PC connection serial cable (DB-9), two 68-pin HD tri-link adapters (H8861-AA), RETMA mounting bracket (disk shelves and disks not included)
Requires: Solutions Software Kit for platform, two country-specific power cords, host adapter, RA3000 disk shelf, and disks
Options: Second HSZ22 controller, up to four additional RA3000 disk shelves, cache memory upgrade
	DS-SWXRA-GK

	

	RAID Array 3000 Rackmount Disk Shelf

	· Select one to four disk shelves

	· Third and fourth DS-SWXRA-GN disk shelf ordered will be installed in rear of M-series cabinet; rear mounted shelves require that a 2-m BN37A-02 SCSI cable be ordered

	RA3000 rackmount disk shelf, blue, includes UltraSCSI BA356 disk shelf, two 180 W power supplies, single-ended UltraSCSI personality card, one half meter SCSI cable (BN37A-0E), two North American power cords, and RETMA mounting bracket (disks not included); order two country-specific power cords for non-North American use
	DS-SWXRA-GN

	

	HSZ22 Controller

	· Note that second HSZ22 controller ordered is field installed only, not factory installed; requires BN37A-0E SCSI jumper cable be ordered separately

	Second HSZ22 controller with 16-MB cache and two 16-MB SIMMs for mirror cache, one BN37A-05 5-m UltraSCSI cable, one BN38E-0B cable adapter ( field installable only; note that second HSZ22 controller does not require additional Solution Software Kit
	DS-HSZ22-AA

	Second HSZ22 controller with 32-MB cache and two 32-MB SIMMs for mirror cache, one BN37A-05 5-m UltraSCSI cable, one BN38E-0B cable adapter ( field installed only; note that second HSZ22 controller does not require additional Solution Software Kit
	DS-HSZ22-AB

	

	RA3000 Solutions Software Kit 

	· StorageWorks Command Console (SWCC) software 

	· Platform specific software

	· Installation scripts and installation documentation

	RA3000 Solutions Software Kit for Tru64 UNIX
	QB-5TWAB-SA

	RA3000 Solutions Software Kit for OpenVMS
	QB-5TWAC-SA

	

	HSG80/HSZ80 External Controllers

	Fibre Channel controller (DS-HSG80-BJ), includes 2-MB cache expandable to 128 MB; requires 128697-B21 HSG80 ACS V8.4F software kit or 128698-B21 V8.4P software kit for each HSG80 ordered; 
	380672-B21

	Fibre Channel controller (DS-HSG80-BH), includes 64-MB cache expandable to 128 MB; requires 128697-B21 HSG80 ACS V8.4F software kit or 128698-B21 V8.4P software kit for each HSG80 ordered; 
	380671-B21

	UltraSCSI controller (DS-HSZ80-AH), includes 64-MB cache expandable to 128 MB; requires 400569-001 for Tru64 UNIX or 400571-001 for OpenVMS
	400564-B21

	UltraSCSI controller (DS-HSZ80-AJ), includes 256-MB cache expandable to 512 MB; requires 400569-001 for Tru64 UNIX or 400571-001 for OpenVMS
	400565-B21

	

	ESA 10000 , RAID Array 7000, RAID Array 8000

	· Enterprise Storage Array 10000 and RAID Array 7000 (HSZ70) product set are supported on AlphaServer DS20E systems

	· Supported SCSI adapter for HSZ80/HSZ70 ( KZPBA-CB PCI 1-port UltraSCSI differential adapter

	· Supported SCSI adapter for HSG80 ( KGPSA-BC PCI 1-port Fiber-Channel adapter

	

	Step 10 (Graphics Adapters (Optional

	· RAID Configuration Utility is accessible via console port for Tru64 UNIX and OpenVMS systems; optional graphics capability available

	· Select graphics adapter, monitor, and country-specific keyboard if required

	
	Maximum # Supported
	

	
	Tru64 UNIX
	OpenVMS
	

	ELSA Gloria synergy graphics with 8-MB SGRAM 1600 x 1200, 2D/3D graphics accelerator
	3
	1
	SN-PBXGK-BB

	PowerStorm 4D51T advanced 3D graphics accelerator
	4
	-
	SN-PBXGI-AD

	PowerStorm 300 3D multi-head graphics accelerator with 15-MB frame buffer memory and 16-MB texture memory 
	3
	-
	SN-PBXGD-AD

	PowerStorm 350 3D multi-head graphics accelerator with 30-MB frame buffer memory and 32-MB texture memory
	3
	-
	SN-PBXGD-AE

	

	Step 11 (Monitors (Optional

	· RAID Configuration Utility is accessible via the console port for Tru64 UNIX and OpenVMS systems; optional graphics capability is available

	· Graphics monitors other than those listed can be used if compatible with SVGA graphics ordered with system

	· Selection of a video extension cable and country-specific power cord is mandatory for all monitors

	15( Corporate Series auto-scan color monitor, VGA to 1024 x 768 @ 85 Hz; Northern Hemisphere without power cord
	3R-VRQV5-24

	Same as above except Southern Hemisphere without power cord
	3R-VRQV5-11

	17( (16.0( viewable image size) professional series auto-scanning color monitor, Trinitron CRT, 0.25 mm aperture grill pitch, VGA to 1280 x 1024 @75 Hz, TCO 99, MPR-II, Energy Star, attached 1.8-m video cable; Northern Hemisphere without power cord
	3R-VRQP7-24

	Same as above except Southern Hemisphere without power cord
	3R-VRQP7-23

	21( (19.6( viewable image size) auto-scanning color monitor, Trinitron CRT, 0.28 mm aperture grill pitch, VGA to 1600 x 1200 @85 Hz NI, TCO 95, Energy Star, includes 1.8-m video cable; Northern Hemisphere without power cord
	3R-VRQP1-24

	Same as above except Southern Hemisphere without power cord
	3R-VRQP1-23

	


	Video Extension Cable

	1.8-m video extension cable—mandatory for each monitor ordered
	BN39C-02

	

	Monitor Power Cords

	North American, 120V, 75(
	BN26J-1K

	Australia, New Zealand, 2.5m 
	BN19H-2E

	Central Europe, 2.5m
	BN19C-2E

	UK, Ireland, 2.5m
	BN19A-2E

	UK, Ireland, 2.5m
	BN19A-2E

	Switzerland, 2.5m
	BN19E-2E

	Denmark, 2.5m
	BN19K-2E

	Italy, 2.5m
	BN19M-2E

	Egypt, India, South Africa, 2.5m
	BN19S-2E

	Israel, 2.5m
	BN18L-2E

	Japan, 2.5m 
	3X-BN46F-02

	

	Step 12 (Networks and Communications ( Optional

	· Controllers/adapters require Tru64 UNIX V4.0F or OpenVMS V7.1-2

	· See Step 2 for mandatory supported Ethernet adapters

	
	Maximum # Supported
	

	
	Tru64 UNIX
	OpenVMS
	

	FE FX Daughtercard for 3X-DE602-AA (3X-DE602-AA is a prerequisite)
	2
	2
	3X-DE602-FA

	PCI Gigabit Ethernet adapter, SC connectors, MMF, no boot support
	2
	1
	DEGPA-SA

	SC-to-SC dual fiber optic cable
	
	BN34B-xx

	
	
	

	PCI FDDI adapter, single attachment station (SAS), multi-mode fiber (MMF), SC connector
	2
	2
	3X-DEFPA-AC

	PCI FDDI adapter, DAS, MMF, SC connector
	2
	2
	3X-DEFPA-DC

	MIC-SC dual fiber optic cable
	
	BN34D-xx

	SC-to-SC dual fiber optic cable
	
	BN34B-xx

	PCI FDDI adapter, SAS, TP-PMD, requires BN25H-03 cable
	2
	2
	3X-DEFPA-UC

	PCI FDDI dual adapter, DAS, TP-PMD, requires BN25H-03 cable
	2
	2
	3X-DEFPA-MC

	3m (9.8") FDDI cross-over cable
	
	BN25H-03

	

	PCI 4-port asynchronous controller
	2
	2
	PBXDA-AA

	PCI 8-port asynchronous controller, cable included
	2
	2
	PBXDA-AB

	PCI 16-port asynchronous controller, cable included
	2
	2
	PBXDA-AC

	

	PCI-to-ATM 155 Mb adapter - Fiber
	4
	-
	3X-DAPBA-FA

	PCI-to-ATM 155 Mb adapter - UTP
	4
	-
	3X-DAPBA-UA

	PCI-to-ATM 622 Mb adapter - Fiber
	2
	-
	3X-DAPCA-FA

	

	Asynchronous multi-port MUX expander XEM
	
	CXI01-AB

	

	Step 13 ( MEMORY CHANNEL (Optional

	Tru64 UNIX Systems

	· Requires a minimum of Tru64 UNIX V4.0F with TruCluster V1.6 or later

	· Each system node in the cluster requires a TruCluster Production Server (QB-3RLAE-KA) or TruCluster MEMORY CHANNEL software license (QB-4ZCAE-AA)

	· TruCluster MEMORY CHANNEL license (QB-4ZCAG-AA), normally used for high performance technical computing applications, not required if systems have a TruCluster Production Server license (QB-3RLAE-KA)

	

	OpenVMS Systems

	· Requires OpenVMS V7.1-2 or later and OpenVMS Cluster license (QL-MUZAE-AA)

	

	Configuring Information

	· Up to two PCI to System Area Network adapters (CCMAB-AA) are supported in a single system

	· Two-node clusters can be configured by ordering a CCMAB-AA for each system and one BN29B-04 or BN29B-10 cable; the cable connects directly to the CCMAB-AA in each system

	· For three or four system nodes, order one CCMAB-AA adapter, one BN29B-04 or BN29B-10 cable and one CCMHB-AA; make connections from the CCMAB-AA in a system to the CCMHB-AA

	· CCMHB-AA includes four CCMLB-AA line cards and supports up to four nodes; expansion up to eight system nodes can be achieved by adding up to four additional CCMLB-AA line cards

	· If two CCMAB-AA’s are configured in each system, a second CCMHB-AA is required for three or more nodes; in two-node clusters, the CCMAB-AA’s may be directly connected

	

	MEMORY CHANNEL Controller

	PCI System Area Network Controller, maximum two
	CCMAB-AA

	System Area Network Hub with four line cards; includes BN19P-2E power cord for Canada, Japan, and US operations(for other regions, order appropriate power cord from list below
	CCMHB-AA

	Expansion line card for CCMHB hub
	CCMLB-AA

	4-m cable for CCMAB and CCMHB
	BN39B-04

	10-m cable for CCMAB and CCMHB
	BN39B-10

	TruCluster Production Server Software for Tru64 UNIX
	QB-3RLAE-AA

	Tru64 UNIX Driver for MEMORY CHANNEL license
	QB-4ZCAE-AA

	OpenVMS Cluster license for Alpha systems
	QL-MUZAE-AA

	

	MEMORY CHANNEL Power Cords

	Ireland, United Kingdom
	BN19A-2E

	Egypt, India
	BN19S-2E

	Central Europe

	BN19C-2E

	Israel


	BN18L-2E

	Switzerland
	BN19E-2E


	Italy
	BN19M-2E

	Denmark
	BN19K-2E

	Australia, New Zealand
	BN19H-2E

	

	Step 14 (Keyboards

	· Keyboards are not included with system and must be ordered separately

	
	Tru64 UNIX
	OpenVMS

	U.S./English keyboard
	SN-LKQ47-AA
	LK461-A2

	Belgian keyboard
	SN-LKQ47-AB
	LK461-AB

	Canadian/French keyboard
	SN-LKQ47-AC
	LK461-AC

	Danish keyboard
	SN-LKQ47-AD
	LK461-AD

	UK keyboard
	SN-LKQ47-AE
	-

	Finnish keyboard
	SN-LKQ47-AF
	LK461-AF

	German keyboard
	SN-LKQ47-AG
	LK461-AG

	Dutch keyboard
	SN-LKQ47-AH
	LK461-AH

	Italian keyboard
	SN-LKQ47-AI
	LK461-AI

	Swiss/French keyboard
	SN-LKQ47-AK
	LK461-AK

	Swiss/German keyboard
	-
	LK461-AL

	Swedish keyboard
	SN-LKQ47-AM
	LK461-AM

	Norwegian keyboard
	SN-LKQ47-AN
	LK461-AN

	French keyboard
	SN-LKQ47-AP
	LK461-AP

	Canadian/English keyboard
	-
	LK461-AQ

	Latin-American keyboard
	SN-LKQ47-AR
	-

	Spanish keyboard
	SN-LKQ47-AS
	LK461-AS

	Hebrew keyboard
	SN-LKQ47-AT
	LK461-AT

	Portuguese keyboard
	SN-LKQ47-AV
	LK461-AV

	BHCSY keyboard
	SN-LKQ47-AX
	-

	International keyboard
	SN-LKQ47-BA
	-

	Greek keyboard
	SN-LKQ47-BH
	LK461-BH

	Taiwanese keyboard
	SN-LKQ47-BI
	-

	Korean keyboard
	SN-LKQ47-BK
	-

	Romanian keyboard
	-
	LK461-BL

	Polish keyboard
	SN-LKQ47-BP
	LK461-BP

	Hungarian keyboard
	SN-LKQ47-BQ
	LK461-BQ

	Arabic keyboard
	SN-LKQ47-BR
	-

	Cyrillic keyboard
	SN-LKQ47-BT
	LK461-BT

	Turkish keyboard
	SN-LKQ47-BU
	LK461-BU

	Czech keyboard
	SN-LKQ47-BV
	LK461-BV

	Turkish/French keyboard
	-
	LK461-BW

	Yugoslavian keyboard
	-
	LK461-BY

	Thai keyboard
	SN-LKQ47-CB
	-

	S. Chinese keyboard
	SN-LKQ47-CV
	-

	Slovak keyboard
	SN-LKQ47-CZ
	LK461-CZ

	

	Keyboard or Mouse Extension Cable

	6 ft keyboard or mouse extension cable; to extend both keyboard and mouse order 2 cables
	17-04016-01

	

	Step 15 (Console Terminal ( Optional

	· Any terminal can be used as a console device

	· Third-party console devices require an EIA-232 25-pin DSUB connector

	VT510 video terminal; requires H857-x cable
	VT510-xx

	Console cable, female connector
	H857-A

	Console cable, male connector
	H857-D

	Console cable, male connector (IBM compatible)
	H897-E

	
	

	Step 16 ( Software ( Required selection of media and documentation for first system on site

	· Software Processor Code = E

	Tru64 UNIX

	· Tru64 UNIX systems include Tru64 UNIX Traditional Unlimited User license, Server Extension, Internet Access Software kit, and Netscape's Enterprise Server 3.0

	Tru64 UNIX media and online documentation on CD-ROM
	QA-MT4AA-H8

	Tru64 UNIX full hard copy documentation
	QA-MT4AA-GZ

	StorageWorks Software Package, includes licenses for Logical Storage Manager and AdvFS Utilities
	QB-5RXAE-AA

	TruCluster Available Server License
	QL-05SAE-AA

	Advanced Server for Tru64 UNIX, 25 client concurrent use license
	QL-5U29M-3D

	Advanced Server for Tru64 UNIX, 50 client concurrent use license
	QL-5U29M-3E

	Advanced Server for Tru64 UNIX, 100 client concurrent use license
	QL-5U29M-3F

	Advanced Server for Tru64 UNIX, 250 client concurrent use license
	QL-5U29M-3G

	Advanced Server for Tru64 UNIX, 500 client concurrent use license
	QL-5U29M-3H

	Layered products media and documentation for Tru64 UNIX on CD-ROM
	QA-054AA-H8

	DECnet/OSI end-system license for Tru64 UNIX
	QL-MTJAE-AA

	DECnet/OSI extended function license for Tru64 UNIX
	QL-MTKAE-AA

	

	OpenVMS 

	· OpenVMS systems include OpenVMS base license and Enterprise Integration Package V2.0

	· OpenVMS Concurrent Use Licenses are not specific to a single system and can be moved between systems at user discretion; OpenVMS Concurrent Use Licenses can also be shared in a mixed OpenVMS VAX and OpenVMS Alpha Cluster

	Concurrent Use 1-user license
	QL-MT3AA-3B

	Concurrent Use 2-user license
	QL-MT3AA-3C

	Concurrent Use 4-user license
	QL-MT3AA-3D

	Concurrent Use 8-user license
	QL-MT3AA-3E

	Concurrent Use 16-user license
	QL-MT3AA-3F

	Concurrent Use 32-user license
	QL-MT3AA-3G

	Concurrent Use 64-user license
	QL-MT3AA-3H

	Concurrent Use 128-user license
	QL-MT3AA-3J

	Concurrent Use 256-user license
	QL-MT3AA-3K

	Traditional unlimited user license
	QL-MT2AQ-AA

	OpenVMS media and documentation on CD-ROM
	QA-MT1AA-H8

	OpenVMS full hard copy documentation
	QA-001AA-GZ

	Layered products media and documentation for OpenVMS on CD-ROM, includes Enterprise Integration Server for OpenVMS media and documentation
	QA-03XAA-H8

	DIGITAL Enterprise Integration Server for OpenVMS media and documentation
	QA-5LVAA-H8

	DECnet extended function license for OpenVMS
	QL-MTGAE -AA

	DECnet end-system to extended function upgrade license for OpenVMS
	QL-MTHAE-AA

	OpenVMS Cluster license for Alpha systems
	QL-MUZAE-AA

	OpenVMS Volume Shadowing license
	QL-2A1AE-AA

	

	Step 17 ( Hardware and Software Supplemental Support Services

	· Systems include 3-year hardware warranty, on-site, with 5 x 9, 24-hour response time; select optional Hardware Supplemental Support Services if required

	· Options ordered as part of "Initial System Sale" take on warranty of the enclosure; options added after "Initial System Sale" take on warranty of enclosure until enclosure warranty ends

	· All unique AlphaServer DS20E options have a minimum of 1-year Return-to-Compaq warranty

	

	Hardware Supplemental Support (Americas and Asia Pacific only
	1-GB memory or less
	More than 1-GB memory

	5 x 9, 4-hour response time
	FM-TC4HR-36
	FM-TF4HR-36

	5 x 12, 4-hour response time
	FM-TC512-36
	FM-TF512-36

	6 x 16, 4-hour response time
	FM-TC616-36
	FM-TF616-36

	7 x 24, 4-hour response time
	FM-TC724-36
	FM-TF724-36

	Years 1-5, next day, onsite
	FM-TCXHW-60
	FM-TFXHW-60

	Years 1-5, 5 x 9, 4-hour response time
	FM-TC4HR-60
	FM-TF4HR-60

	Years 1-5, 5 x 12, 4-hour response time
	FM-TC512-60
	FM-TF512-60

	Years 1-5, 6 x 16, 4-hour response time
	FM-TC616-60
	FM-TF616-60

	Years 1-5, 7 x 24, 4-hour response time
	FM-TC724-60
	FM-TF724-60

	

	Installation Services

	Hardware product installation for DS20E
	FM-TC12X-IN

	Add-on memory installation for DS20E
	FM-TCMEM-IN

	Add-on CPU installation for DS20E
	FM-TCSMP-IN

	

	Software Supplemental Support— Americas and Asia Pacific only

	· Systems include 90-day Conformance to SPD and Telephone Advisory Support; select optional Software Supplemental Support Services if required

	· Software service upgrades for Tru64 UNIX include advisory and remedial software support with new version license rights for operating system for time period indicated

	· Software service upgrades for OpenVMS include advisory and remedial software support with new version license rights for operating system and Enterprise Integration Package for time period indicated

	· Software service upgrades for SMP upgrades running Tru64 UNIX or OpenVMS include new version license rights for Tru64 UNIX or OpenVMS SMP Upgrades for time period indicated; one required for each SMP upgrade

	

	
	Tru64 UNIX
	OpenVMS

	5X9 12-month Software Supplemental Support 
	FM-D20U9-12
	FM-D20V9-12

	5X9 36-month Software Supplemental Support 
	FM-D20U9–36
	FM-D20V9-36

	5X9 60-month Software Supplemental Support 
	FM-D20U9-60
	FM- D20V9-60

	7X24 12-month Software Supplemental Support 
	FM-D20US-12
	FM-D20VS-12

	7X24 36-month Software Supplemental Support 
	FM-D20US–36
	FM- D20VS-36

	7X24 60-month Software Supplemental Support 
	FM-D20US-60
	FM- D20VS-60

	

	12-month Software Supplemental Support ( DS20E SMP Upgrade
	FM-D20UP-12
	FM-D20VP-12

	36-month Software Supplemental Support ( DS20E SMP Upgrade
	FM-D20UP-36
	FM-D20VP-36

	60-month Software Supplemental Support( DS20E SMP Upgrade
	FM-D20UP-60
	FM-D20VP-60

	


	Pedestal
	
	Dimensions (HxWxD)
	18.5 x 8.85 x 27.5 in / 47.0 x 22.5 x 69.9 cm

	
	
	Shipping Dimensions
	24 x 26.25 x 40 in / 61.0 x 66.0 x 101.6 cm

	
	
	Weight
	

	
	
	Typical Configuration
	80 lbs/36 kg

	
	
	Maximum Configuration
	88 lbs/ 40 kg

	
	
	Shipping Weight
	

	
	
	Nominal
	100 lbs/ 45 kg

	
	
	Maximum
	110 lbs/ 50 kg

	
	
	Clearances
	Operating
	Service

	
	
	Front
	15 in/ 38.1 cm
	15 in/ 38.1 cm

	
	
	Rear
	6 in/15 cm
	29.5 in/75 cm

	
	
	Left Side
	None
	None

	
	
	Right Side
	None
	None

	
	
	
	

	Rackmount
	
	Dimensions (HxWxD)
	8.75 x 17.5 x 26 in/22.2 x 44.5 x 66.0 cm 
(5U)

	
	
	Shipping Dimensions
	24 x 26.25 x 40 in/61.0 x 66.0 x 101.6 cm

	
	
	Weight
	

	
	
	When lifting:
	

	
	
	Nominal
	80 lbs/36 kg

	
	
	Maximum
	86 lbs/39 kg

	
	
	Total added to cabinet (brackets, slides, cables):
	

	
	
	Nominal
	84 lbs/38 kg

	
	
	Maximum
	88 lbs/40 kg

	
	
	Shipping Weight
	

	
	
	Nominal
	100 lbs/45 kg

	
	
	Maximum
	110 lbs/50 kg

	
	
	Clearances
	Operating
	Service

	
	
	
	See specific cabinet requirements
	Minimum 4 ft / 121.9 cm, 
28 in / 71 cm withdrawal on rails

	
	
	
	
	

	Cabinet
	
	
	H9A10 M-Series
	H9A15 M-Series

	
	
	Dimensions (HxWxD)
	67 x 23.6 x 43.27 in/
170 x 60 x 110 cm
	79 x 23.6 x 35.4 in
200 x 60 x 90 cm

	
	
	Shipping Dimensions
	73 x 36 x 48 in /
185.5 x 91.5 x 122 cm
	85 x 36 x 48 in /
216 x 91.5 x 122 cm

	
	
	Weight
	Configuration dependent
	1000 lbs / 450 kg

	
	
	Shipping Weight
	Configuration dependent, maximum payload 1,000 lbs
	1056 lbs / 550 kg (normal)
1,408 lbs / 640 kg (maximum)

	
	
	
	

	System Unit
	
	Environmental
	

	
	
	Temperature
	

	
	
	Operating (Tower, Rackmount)1
	50( to 95(F/10( to 35(C

	
	
	Non-operating
	Not tested

	
	
	Storage (60 days)
	–40( to –151(F/–40 to –66(C

	
	
	Rate of change
	20(F/hr/11(C/hr

	
	
	Relative Humidity
	

	
	
	Operating
	20% to 80%

	
	
	Non-operating
	20% to 80%

	
	
	Storage (60 days)
	10% to 95%

	
	
	Rate of change
	20% hr

	
	
	Maximum Wet Bulb Temperature
	

	
	
	Operating
	82(F/28(C

	
	
	Storage (60 days)
	115(F/46(C

	
	
	Maximum Dew Point Temperature
	

	
	
	Operating
	36(F/2(C

	
	
	Storage (60 days)
	Not tested

	
	
	Heat Dissipation
	Nominal
	Maximum

	
	
	Tower 
	550W, 1,878 BTU/hr
	780W, 2,664 BTU/hr

	
	
	Pedestal
	
	

	
	
	H9A10/H9A15 Cabinets
	Configuration dependent
	

	
	
	Air Flow and Quality
	

	
	
	Intake location
	Front

	
	
	Exhaust location
	Rear Tower, Pedestal, Rack; Rear/top H9A10/H9A15

	
	
	Particle size
	N/A

	
	
	Concentration
	N/A

	
	
	Altitude
	

	
	
	Operating 2
	10,000 f/3,037m

	
	
	Non-operating
	40,000 ft/12,192m

	
	
	Acoustics (Declared values per ISO 9296 and ISO 7779)

	
	
	Acoustics 3
	LWAd3B
	LnAm5dBA (bystander position)

	
	
	Idle
	5.8
	44

	
	
	Operating
	6.3
	49

	
	
	Vibration
	

	
	
	Operating
	10-500 Hz .1 G peak

	
	
	Non-operating
	1.03 Grms 5-300 Hz

	System Unit (continued)
	
	Electrical (Power supplies are universal, PFC, auto ranging, 100/240 Vac)

	
	
	Nominal Voltage (Vac)
	100
	120
	200-240

	
	
	Voltage Range (Vac) (temporary condition)
	90-100
	110-128
	180-250

	
	
	Power Source Phase
	Single
	Single
	Single

	
	
	Nominal Frequency (Hz)
	50/60
	50/60
	50/60

	
	
	Frequency Range (Hz)
	49 to 51/59 to 61
	49 to 51/59 to 61
	49 to 51/59 to 61

	
	
	RMS Current (maximum steady state)
	
	
	

	
	
	Pedestal and Rackmount
	
	
	

	
	
	Single power cord
	6.6A
	5.5A
	3.0A

	
	
	Maximum VA
	780
	765
	730

	
	
	M-series Cabinet (configuration dependent)
	
	
	

	
	
	Nominal voltage (Vac)
	100
	120
	220-240

	
	
	Each power cord
	24A
	24A
	16A

	
	
	Power Cords
	
	

	
	
	Pedestal
	1 (75 in / 190 cm)
	IEC 320 C13 to NEMA 5-15 
(N. America) or IEC 320 C13 to country-specific

	
	
	Rackmount
	1 (14 ft 10 in / 452 cm)
	IEC 320 C13 to NEMA 5-15 
(N. America) or
IEC 320 C13 to IEC 320 C14 
(other countries)

	
	
	Cabinet
	2 (10 ft 10 in / 330 cm)
	120V non-removable NEMA 
L5-30P or 200-240V non-removable IEC 309

	
	
	Regulatory
	

	
	
	Agency Approvals
	UL: Listed to UL1950 (3rd edition)

	
	
	
	CSA: Certified to CAN/CSA-C22.2 No. 950-M95

	
	
	
	TUV: EN 60950/A4: 1997 VDE 0805 GS marked

	
	
	
	FCC: Part 15.B Class A

	
	
	
	CE:  EN55022, EN50082

	
	
	
	VCCI Class II ITE

	
	
	
	BCIQ: CISPR22, CNS13438

	
	
	
	C-Tick: CISPR22, AS/NZS 3548

	
	
	Reviewed to
	AS/NZ 3260:1993 Australian/New Zealand Standard

	
	
	
	EN 60950/A4: 1997 European Norm

	
	
	
	IEC 950 (2nd edition, 3rd amend)


1 Maximum operating temperature at sea level; reduce by 1.8(F (1(C) for each 600m (2,000 ft) above sea level.

2 Higher altitudes are possible if maximum operating temperature is reduced (see Temperature); other restrictions may apply such as maximum permissible altitude for hard drives.

3 Current values for specific configurations are available from Compaq, note that 1 B = 10 dBA.

Operator Control Panel


Removable media bay


Combination CD-ROM/floppy drive 


Hard disk drives


Power supplies


Ultra2 SCSI adapter


System motherboard 


CPU modules


Server Management Module








At A Glance


AlphaServer DS20E systems include:


Alpha 21264 processor, 500 MHz or 667 MHz (dual processor capability) 


4-MB level 2 writeback cache per 500 MHz processor or 8-MB L2 DDR cache per 667 MHz processor


ECC-protected memory bus and cache


256-MB, 512-MB, or 1-GB  SDRAM memory expandable to 4 GB


Five PCI and one shared PCI/ISA slot (six total slots)


Four hot swap hard disk drive bays


Internal mass storage capacity up to 72.8 GB


Integrated environmental server management module


Two 375-Watt hot pluggable power supplies with optional third for N+1 redundancy


Combo 24X CD-ROM Drive and 1.44-MB floppy disk drive


Protected by Compaq Services, including a three-year on-site limited warranty













































































































































































July 19, 1999


2nd  Review – 5/17/99


Due Back –  5/21/99Revision 2.4


March 28, 2000


Edited By:  

















1
3/1/99
World Wide – Servers



2
3/1/99
World Wide – Servers



1
DA-10445-01-001 — 11.15.1999


_986646668.doc
[image: image1.png]COMPAQ
“Your partner through time”

000
Ready







