TKMU Instructions

- Open all relevant Tait programs. Programming Software, Calibration Application and Tait Firmware Upgrade Tool.

- Read radios calibration and program data and save them to the desktop.

- Using the Tait Firmware Upgrade tool, click “Report Configuration” to check FW version currently in radio.

- Upgrade the radio to the STANDARD Tait FW matching the same version of FW as the “PATCHED” version. I.e. upgrade TP9400 to STANDARD FW v2.07 before loading in PATCHED FW v2.07. Same goes with the TP/TM9100, TP/TM9300 and the TM9400 etc.

- Once the radio has the standard version of FW in, re-calibrate and program using the already saved data files from beforehand.

- Now again using the Tait Firmware Upgrade Tool, load in the hacked or “PATCHED” FW.

- Once the PATCHED FW is in the radio, it should boot without showing any error. If it does show an error, then re-calibrate and program the radio from the same saved data files as beforehand.

- Locate the “Tait Key Management Utility” program and open it up. With the radio still attached to the PC, click the green “Report Configuration” tab.

- You should see all available SFE Keys appear as a list. Also, up top under “firmware” it will note the version of firmware in the radio and have “Modified” in brackets next to it.

- If activating all available SFE keys, simply click “enable all SFE”. Alternatively click on each listed SFE and then click activate.

- Once your various SFE Keys have been activated, close the TKMU program.

- Using the Tait Firmware Upgrade Tool, load back in the radios normal FW version again.

- Re-calibrate and program from the previous saved data just to be safe.

- After all the above is completed, you may wish to load the latest version of FW into the radio. This IS recommended.