 D-Link Support Publishing 							         June 1996


D-Link DE-650CT PCMCIA Ethernet Card Installation for OS/2 Warp Connect


Contents


Introduction


Requirements


Minimum Skill Level


Compatibility List


Getting Started


Support Options





Introduction


For convenience, the INSTALL.EXE provided on the driver disk. The A:\INSTALL.EXE will install all of the relative drivers. INSTALL.EXE requires a DOS based environment to do the installation process.


The ODI DOS driver, NDIS DOS driver, and NDIS OS/2 driver are the only network operating system drivers that support an On-Line Hot-Swap function. Hot-swap allows the card to be removed and re-inserted with out reloading the driver. Eventhough the driver supports Hot-Swap your NOS may not support hot-swap.


DCD16.OS2:


DCD16.OS2 and DCDOPEN.OS2 are client drivers for IBM OS/2 Warp.


Requirements


A PCMCIA compatible Laptop


A PCMCIA chipset that is PCIC compatible (Intel, Cirrus, Vadem, and VLSI).


A DE-650CT (our single media cards are not supported, 650T and 650C)


A knowledge of basic OS/2 commands and editing text files


Minimum Skill Level


A knowledge of basic OS/2 commands


A knowledge of editing text files


A knowledge of OS/2 basic file structure (CONFIG.SYS)


A knowledge of the network driver configuration being used


Please see Support Options if you are not comfortable will this skill level


Compatibility List


DE-650CT only (will not run with DE-650T or DE-650C)


Removal of any present PCMCIA drivers





For a laptop compatibility list please consult your local D-Link dealer


Getting Started


OS/2 Warp Connect Installation


Install OS/2 Warp Connect.  Select Advanced Installation.


At System Configuration screen, install Serial Device Support.  DO NOT select PCMCIA Support at this time.


When the option to install Networking Support appears, select No.


Restart your computer when prompted.


After restart, select OS/2 Warp Selective Install to return to System Configuration.  Double check that Serial Support is installed.  If so, select PCMCIA Support.


Select PCMCIA System for your computer.  If not sure which one to select, contact the computer manufacturer.  Click OK, then INSTALL.


If any errors during restart, you may need to change PCMCIA System selection.  If no errors, double click Warp Connect Selective Install for Networking.


Select Advanced Installation.  Select Other adapter… when given network adapter list.  Follow instructions on screen to install driver. 


After D-Link DE-650CT appears on list, select Settings…


Enter the Network adapter address.  This can be found on the back of the DE-650CT card (00 80 C8 ……).  Enter any other information (IP address if installing TCP/IP, etc.), then select Install.  Do not insert the DE-650CT yet.  You will get errors on restart.  Simply confirm each and continue.


Double-click MPTS.  Select Configure…


Highlight each protocol listed under D-Link DE-650CT, select Edit and enter network adapter address.  This must be completed for each protocol.  Click OK, Close, then Exit.


Before shutdown, you will need to edit the PROTOCOL.INI located in the C:\IBMCOM directory.  Example:


[PROT_MAN]


   DRIVERNAME = PROTMAN$





[IBMLXCFG]


   NETBEUI_nif = NETBEUI.NIF


   ODI2NDI_nif = ODI2NDI.NIF


   TCPIP_nif = TCPIP.NIF


   DE65SIBM_nif = DE65SIBM.NIF





[NETBIOS]


   DriverName = netbios$


   ADAPTER0 = netbeui$,0





[NETBEUI_nif]


   DriverName = netbeui$


   Bindings = DE65SIBM_nif


   NETADDRESS = "0080c8816d3c"


   ETHERAND_TYPE = "I"


   USEADDRREV = "YES"


   OS2TRACEMASK = 0x0


   SESSIONS = 130


   NCBS = 225


   NAMES = 21


   SELECTORS = 15


   USEMAXDATAGRAM = "NO"


   ADAPTRATE = 1000


   WINDOWERRORS = 0


   MAXDATARCV = 4168


   TI = 30000


   T1 = 1000


   T2 = 200


   MAXIN = 1


   MAXOUT = 1


   NETBIOSTIMEOUT = 2000


   NETBIOSRETRIES = 3


   NAMECACHE = 1000


   RNDOPTION = 1


   PIGGYBACKACKS = 1


   DATAGRAMPACKETS = 10


   PACKETS = 350


   LOOPPACKETS = 8


   PIPELINE = 5


   MAXTRANSMITS = 6


   MINTRANSMITS = 2


   DLCRETRIES = 10


   FCPRIORITY = 5


   NETFLAGS = 0x0





[ODI2NDI_nif]


   DriverName = odi2ndi$


   Bindings = DE65SIBM_nif


   NETADDRESS = "0080c8816d3c"


   TOKEN-RING = "no"


   TOKEN-RING_SNAP = "no"


   ETHERNET_802.3 = "yes"


   ETHERNET_802.2 = "yes"


   ETHERNET_II = "yes"


   ETHERNET_SNAP = "yes"


   TRACE = 0x0





[TCPIP_nif]


   DriverName = TCPIP$


   Bindings = DE65SIBM_nif





[DE65SIBM_nif]


   DriverName = DE65S$


   netaddress = "0080C8816D3C"


   interrupt = 11		( add this parameter 


   pcmciaslot = 2		( add this parameter





* Note: Check PCMCIA Plug and Play to see how slots are numbered.





Shutdown the computer, insert the DE-650CT, and restart.


 Below is an example CONFIG.SYS:


IFS=C:\OS2\HPFS.IFS /CACHE:64 /CRECL:4


PROTSHELL=C:\OS2\PMSHELL.EXE


SET USER_INI=C:\OS2\OS2.INI


SET SYSTEM_INI=C:\OS2\OS2SYS.INI


SET OS2_SHELL=C:\OS2\CMD.EXE


SET AUTOSTART=PROGRAMS,TASKLIST,FOLDERS,CONNECTIONS,LAUNCHPAD


SET RUNWORKPLACE=C:\OS2\PMSHELL.EXE


SET COMSPEC=C:\OS2\CMD.EXE


LIBPATH=C:\MPTN\DLL;C:\IBMCOM\DLL;.;C:\OS2\DLL;C:\OS2\MDOS;C:\;C:\OS2\AP	PS\DLL;C:\MMOS2\DLL;C:\GRPWARE;C:\NETWARE;C:\NETWARE\NLS\ENGLI	SH;L:\OS2;P:\OS2;SET PATH=C:\MPTN\BIN;C:\IBMCOM;C:\OS2;C:\OS2\SYSTEM;C:\OS2\MDOS\WINOS2;C	:\OS2\INSTALL;C:\;C:\OS2\MDOS;C:\OS2\APPS;C:\MMOS2;C:\NETWARE;L:\OS	2;P:\OS2;


SETDPATH=C:\IBMCOM;C:\OS2;C:\OS2\SYSTEM;C:\OS2\MDOS\WINOS2;C:\OS2\INSTALL;C:\;C:\OS2\BITMAP;C:\OS2\MDOS;C:\OS2\APPS;C:\MMOS2;C:\MMOS2\INSTALL;C:\GRPWARE;C:\NETWARE;C:\NETWARE\NLS;C:\NETWARE\NLS\ENGLISH;L:\NLS;P:\NLS;


BASEDEV=DETNE2.SYS


SET PROMPT=$i[$p]


SET HELP=C:\OS2\HELP;C:\OS2\HELP\TUTORIAL;C:\MMOS2\HELP;C:\NETWARE\NLS\ENGLISH;


SET GLOSSARY=C:\OS2\HELP\GLOSS;


SET IPF_KEYS=SBCS


PRIORITY_DISK_IO=YES


FILES=20


BASEDEV=IBMKBD.SYS


DEVICE=C:\IBMCOM\PROTOCOL\LANPDD.OS2


DEVICE=C:\IBMCOM\PROTOCOL\LANVDD.OS2


DEVICE=C:\IBMCOM\LANMSGDD.OS2 /I:C:\IBMCOM 


DEVICE=C:\IBMCOM\PROTMAN.OS2 /I:C:\IBMCOM


DEVICE=C:\OS2\BOOT\TESTCFG.SYS


DEVICE=C:\OS2\BOOT\DOS.SYS


DEVICE=C:\OS2\BOOT\PMDD.SYS


BUFFERS=90


IOPL=YES


DISKCACHE=D,LW,AC:C


MAXWAIT=3


MEMMAN=SWAP,PROTECT


SWAPPATH=C:\OS2\SYSTEM 2048 2048


BREAK=OFF


THREADS=256


PRINTMONBUFSIZE=134,134,134


COUNTRY=001,C:\OS2\SYSTEM\COUNTRY.SYS


SET KEYS=ON


SET BOOKSHELF=C:\OS2\BOOK;C:\MMOS2;


SET SOMIR=C:\OS2\ETC\SOM.IR;C:\OS2\ETC\WPSH.IR;C:\OS2\ETC\WPDSERV.IR


SET SOMDDIR=C:\OS2\ETC\DSOM


REM SET DELDIR=C:\DELETE,512;D:\DELETE,512;


BASEDEV=PRINT01.SYS


BASEDEV=IBM1FLPY.ADD


BASEDEV=IBM2FLPY.ADD


BASEDEV=IBM1S506.ADD


BASEDEV=XDFLOPPY.FLT


BASEDEV=IBMINT13.I13


BASEDEV=OS2DASD.DMD


SET EPMPATH=C:\OS2\APPS;


PROTECTONLY=NO


SHELL=C:\OS2\MDOS\COMMAND.COM C:\OS2\MDOS


FCBS=16,8


RMSIZE=640


DEVICE=C:\NETWARE\VIPX.SYS


DEVICE=C:\NETWARE\VSHELL.SYS GLOBAL


DEVICE=C:\OS2\MDOS\VEMM.SYS


DOS=LOW,NOUMB


DEVICE=C:\OS2\MDOS\VXMS.SYS /UMB


DEVICE=C:\OS2\MDOS\VDPMI.SYS


DEVICE=C:\OS2\MDOS\VDPX.SYS


DEVICE=C:\OS2\MDOS\VWIN.SYS


DEVICE=C:\OS2\MDOS\VW32S.SYS


DEVICE=C:\OS2\BOOT\OS2CDROM.DMD /Q


IFS=C:\OS2\BOOT\CDFS.IFS /Q


DEVICE=C:\OS2\MDOS\VCDROM.SYS


BASEDEV=OS2SCSI.DMD


BASEDEV=AHA154X.ADD


DEVICE=C:\OS2\MDOS\VMOUSE.SYS


DEVICE=C:\OS2\BOOT\POINTDD.SYS


DEVICE=C:\OS2\BOOT\MOUSE.SYS SERIAL=COM1


CODEPAGE=437,850


DEVINFO=KBD,US,C:\OS2\KEYBOARD.DCP


DEVINFO=SCR,VGA,C:\OS2\BOOT\VIOTBL.DCP


SET VIDEO_DEVICES=VIO_VGA


SET VIO_VGA=DEVICE(BVHVGA)


DEVICE=C:\OS2\MDOS\VSVGA.SYS


SET MMBASE=C:\MMOS2;


SET DSPPATH=C:\MMOS2\DSP;


SET NCDEBUG=4000


DEVICE=C:\MMOS2\SSMDD.SYS


DEVICE=C:\MMOS2\R0STUB.SYS





BASEDEV=PCMCIA.SYS /P					-----


DEVICE=C:\OS2\MDOS\VPCMCIA.SYS				     |	


DEVICE=C:\OS2\BOOT\COM.SYS					     |(PCMCIA 


DEVICE=C:\OS2\MDOS\VCOM.SYS					     |     Support


DEVICE=C:\OS2\BOOT\AUTODRV2.SYS C:\OS2\AUTODRV2.INI	     |	


BASEDEV=IBM2CMQ1.SYS /s0=2					-----





REM --- NetWare Requester statements BEGIN ---


SET NWLANGUAGE=ENGLISH


DEVICE=C:\NETWARE\LSL.SYS


RUN=C:\NETWARE\DDAEMON.EXE


DEVICE=C:\IBMCOM\PROTOCOL\ODI2NDI.OS2


REM -- ODI-Driver Files BEGIN --


rem DEVICE=C:\NETWARE\NTR2000.SYS


REM -- ODI-Driver Files END --


DEVICE=C:\NETWARE\ROUTE.SYS


DEVICE=C:\NETWARE\IPX.SYS


DEVICE=C:\NETWARE\SPX.SYS


RUN=C:\NETWARE\SPDAEMON.EXE


rem DEVICE=C:\NETWARE\NMPIPE.SYS


rem DEVICE=C:\NETWARE\NPSERVER.SYS


rem RUN=C:\NETWARE\NPDAEMON.EXE


DEVICE=C:\NETWARE\NWREQ.SYS


IFS=C:\NETWARE\NWIFS.IFS


RUN=C:\NETWARE\NWDAEMON.EXE


rem DEVICE=C:\NETWARE\NETBIOS.SYS


rem RUN=C:\NETWARE\NBDAEMON.EXE


DEVICE=C:\OS2\MDOS\LPTDD.SYS


REM --- NetWare Requester statements END ---


CALL=C:\IBMCOM\PROTOCOL\NETBIND.EXE


RUN=C:\IBMCOM\LANMSGEX.EXE


SET ETC=C:\MPTN\ETC


DEVICE=C:\MPTN\PROTOCOL\SOCKETS.SYS


DEVICE=C:\MPTN\PROTOCOL\AFOS2.SYS


DEVICE=C:\MPTN\PROTOCOL\AFINET.SYS


DEVICE=C:\MPTN\PROTOCOL\IFNDIS.SYS


RUN=C:\MPTN\BIN\CNTRL.EXE  


CALL=C:\OS2\CMD.EXE /Q /C C:\MPTN\BIN\MPTSTART.CMD


DEVICE=C:\IBMCOM\PROTOCOL\NETBEUI.OS2


DEVICE=C:\IBMCOM\PROTOCOL\NETBIOS.OS2


DEVICE=C:\IBMCOM\MACS\DE65S.OS2





Support Options


For technical support, you may contact your authorized D-Link distributor, dealer, or the following D-Link headquarters:


  In U.S.A.           Tel: +1(714)455-1688      Fax: +1(714)455-2521


                      BBS: +1(714)455-1779      BBS: +1(714)455-9616





  In Canada           Tel: +1(905)828-0260      Fax: +1(905)828-5669


                      BBS: +1(905)828-6673





  In U.K. & Europe    Tel: +44(181)235-5555     Fax: +44(181)235-5500


BBS: +44(181)235-5511


In Germany          Tel: +49(6196)643011      Fax: +49(6196)28049


BBS: +49(6196)28378


In Asia, Taiwan     Tel: +886(2)916-1600      Fax: +886(2)914-6299


BBS: +886(2)910-1835


  In France           Tel: +33(1)30238688       Fax: +33(1)3028689








  In Australia        Tel: +61(2)4177100        Fax: +61(2)4171077


                      BBS: +61(2)4173711





  In Nordic           Tel: +46(40)163330        Fax: +46(40)163340


                      BBS: +46(40)163343





  In Singapore        Tel: +657746233           Fax: +657746322


BBS: +657744787


  In China            Tel: +86(10)4934175       Fax: +86(10)4991648





  In Tokyo            Tel: +81(3)57512351       Fax: +81(3)57512352





  In India            Tel: +91(22)6172478       Fax: +91(22)6172476





The BBS communication setup information is as follows :


      Speed     :  2400-14400 baud                           - Germany


                   2400-9600 baud                            - Asia,Taiwan


                   14400-16800 baud                          - Canada


9600-14400 baud V32, V32bis, V42, V42bis  - U.S.A.


2400-19200 baud V24, V32, V32bis, V42bis  - U.K. & Europe


Parity    : none


Data bits : 8


Stop bit  : 1








Copyrights, Trademarks, and Disclaimers


A documents and DE-650 drivers are Copywritten to Copyright © 1996 by D-Link Corporation. All rights reserved.


All references to MS, Windows, Win95, Windows 95, Windows NT, WinNT are


rights reserved by Microsoft Corporation


All references to Nvl., ODI, Netware, NW, IPX, IPXODI, and LSL are rights


reserved by Novell Corporation


Further rights reserved to their respective corporations


