Huawei Certification Series Tutorials

HCIE-Datacom V1.0

LAB Environment Setup Guide

Huawei Technologies Co., Ltd.

Copyright Huawei Technologies Co., Ltd. 2021. All rights reserved. No part of this document may be reproduced or transmitted in any form or by any means without the prior written consent of Huawei Technologies Co., Ltd. Trademark Declaration and other Huawei trademarks are trademarks of Huawei Technologies Co., Ltd. All other trademarks and trade names mentioned in this document are the property of their respective holders. Note: The purchased products, services and features are stipulated by the contract made between Huawei and the customer. All or part of the products, services and features described in this document may not be within the purchase scope or the usage scope. Unless otherwise specified in the contract, all statements, information, and recommendations in this document are provided without warranties, guarantees, or representations of any kind, either express or implied. The information in this document is subject to change without notice. Every effort has been made in the preparation of this document to ensure accuracy of the contents, but all statements, information, and recommendations in this document do not constitute the warranty of any kind, express or implied.

| Huawei Technologies Co., Ltd. |  |
| --- | --- |
| Address: | Huawei Industrial Base Bantian, Longgang, Shenzhen 518129 |
| Website: | http://e.huawei.com |

Huawei Certificate System

Huawei Certification is an integral part of the company's "Platform + Ecosystem" strategy, and it supports the ICT infrastructure featuring "Cloud-Pipe-Device". It evolves to reflect the latest trends of ICT development. Huawei Certification consists of two categories: ICT Infrastructure Certification, and Cloud Service & Platform Certification, making it the most extensive technical certification program in the industry.

Huawei offers three levels of certification: Huawei Certified ICT Associate (HCIA), Huawei Certified ICT Professional (HCIP), and Huawei Certified ICT Expert (HCIE).

Huawei Certification covers all ICT fields and adapts to the industry trend of ICT convergence. With its leading talent development system and certification standards, it is committed to fostering new ICT talent in the digital era, and building a sound ICT talent ecosystem.

HCIE-Datacom V1.0 aims to train and certificate experts with professional knowledge and skills of multi-scenario convergence solutions in the datacom network field.

Passing the HCIE-Datacom V1.0 certification will indicate that you:

1) understand and master routing and switching advanced technologies, panoramic view of enterprise network architecture, typical architecture and technologies of campus network, planning and deployment of Huawei CloudCampus solution, typical architecture and technologies of WAN interconnection, planning and deployment of Huawei SD-WAN solution, typical architecture and technologies of bearer WAN, planning and deployment of Huawei CloudWAN solution, network automation technologies and practice.

2) will have a solid theoretical knowledge of multi-scenario convergence solutions for enterprise networks, and be able to use Huawei Datacom products and solutions to plan, deploy, maintain, and optimize enterprise campus networks, WAN interconnection, and bearer WAN. You will be competent for the expert positions of full-scenario enterprise networks (including account managers, project managers, pre-sales experts, post-sales experts, network architects, etc.).

| Version | Release Date | author | Applicable Time Range | readers |
| --- | --- | --- | --- | --- |
| 01 | 2021 - 11 - 30 | Talent Development Dept | 2021 - 11 - 30 - 2023 - 11 - 30 | HALP Instructor |

Table of Contents

foreword	4

Introduction	4

1 Equipment procurement	5

1.1 Purchase List	5

2 Cable Connection	6

2.1 Network topology	6

2.1.1 Overall network topology	6

2.1.2 Site HQ Internal Topology	7

2.1.3 Wide Area Network (MPLS) Topology	7

2.2 experimental planning	7

3 Precautions	10

3.1 Use of non-standard equipment	10

3.2 Other Precautions	10

4 Attachments	11

4.1 Related Annexes	11

foreword

Introduction

The HCIE-Datacom V1.0 LAB Environment Setup Guide specifies the experimental equipment, and generates the list of experimental equipment and physical connection diagram.

This document is intended to guide regions, suppliers, HALPs, and network institutes to purchase devices, install modules, connect cables, configure control devices, and build a topology diagram. Perform experiments based on the HCIE-Datacom LAB Guide V1.0.

Equipment procurement

Purchase List

For the equipment purchase list, see the attachment.

File name: HCIE-Datacom V1.0 Equipment List

Ensure that the equipment is purchased according to the lab requirements. If you have any questions, contact Certification@huawei.com.

Cable Connection

Network topology

Overall network topology

Site HQ Internal Topology

Wide Area Network (MPLS) Topology

experimental planning

The mapping between device names and models is as follows:

| Device Name | Device Model |
| --- | --- |
| AR1 | AR6121E |
| AR2 | AR6121E |
| AR3 | AR6121E |
| PE1 | NetEngine 8000 M6 |
| PE2 | NetEngine 8000 M6 |
| PE3 | NetEngine 8000 M6 |
| PE4 | NetEngine 8000 M6 |
| P1 | NetEngine 8000 M6 |
| P2 | NetEngine 8000 M6 |
| AR_Server_SW | S5720-52X-LI-AC |
| Border | S5731-H24T4XC |
| Edge_1 | S5731-H24T4XC |
| Edge_2 | S5731-H24T4XC |
| ACC_1 | S5731-H24T4XC |
| ACC_2 | S5731-H24P4XC |
| AP1 | AirEngine5760-1 |

After the device is connected, check whether the port connection is correct according to the following table.

| Device Name | Interface | Peer device |
| --- | --- | --- |
| PE1 | GE0/5/0 | PE2 |
|  | GE0/5/1 | P1 |
|  | GE0/5/2 | AR1 |
| PE2 | GE0/5/0 | PE1 |
|  | GE0/5/1 | P2 |
|  | GE0/5/2 | AR2 |
| PE3 | GE0/5/0 | PE4 |
|  | GE0/5/1 | P1 |
|  | GE0/5/2 | AR3 |
| PE4 | GE0/5/0 | PE3 |
|  | GE0/5/1 | P2 |
| P1 | GE0/5/0 | P2 |
|  | GE0/5/1 | PE1 |
|  | GE0/5/2 | PE3 |
| P2 | GE0/5/0 | P1 |
|  | GE0/5/1 | PE2 |
|  | GE0/5/2 | PE4 |
| AR1 | GE0/0/9 | AR_Server_SW |
|  | GE0/0/8 | PE1 |
|  | GE0/0/1 | PC4 |
| AR2 | GE0/0/9 | AR_Server_SW |
|  | GE0/0/8 | PE2 |
|  | GE0/0/1 | PC5 |
| AR3 | GE0/0/9 | AR_Server_SW |
|  | GE0/0/8 | PE3 |
|  | GE0/0/1 | Border |
| AR_Server_SW | GE0/0/1 | AR1 |
|  | GE0/0/2 | AR2 |
|  | GE0/0/3 | AR3 |
|  | GE0/0/24 | iMaster NCE-Campus |
| Border | GE0/0/1 | AR3 |
|  | GE0/0/23 | Edge_1 |
|  | GE0/0/24 | Edge_2 |
| Edge_1 | GE0/0/1 | Border |
|  | GE0/0/24 | ACC_1 |
| Edge_2 | GE0/0/1 | Border |
|  | GE0/0/24 | ACC_2 |
| ACC_1 | GE0/0/1 | Edge_1 |
|  | GE0/0/24 | PC1 |
| ACC_2 | GE0/0/1 | Edge_2 |
|  | GE0/0/23 | PC2 |
|  | GE0/0/24 | AP1 |
| AP1 | GE0/0/1 | ACC_2 |

Precautions

Use of non-standard equipment

Use the standard device list in the attachment when creating the experiment topology.

Non-standard devices may cause the following problems:

- The operation interface is inconsistent with that in the lab guide.

- Some experiment effects cannot be displayed.

- Not compatible with later versions of lab manuals and teaching materials

Other Precautions

It is recommended that the lecturer explain the teaching materials and students complete the teaching materials independently. If troubleshooting is required, the lecturer can participate in the teaching materials.

In addition, to avoid the impact of residual configurations on the experiment, students are required to clear the configuration information saved on the device after the experiment is complete and before the device is shut down. At the beginning of the experiment, ensure that the device starts from empty configurations. Otherwise, clear the configuration and restart the device.

In this experiment, the iMaster NCE physical server is connected to AR_Server_SW through GE0/0/24. In actual environments, the iMaster NCE physical server is installed in various modes, and the number of physical servers is not fixed. The actual connection mode between the physical server and AR_Server_SW depends on the purchased environment.

Attachments

Related Annexes