|  | HCIP-Datacom-SD-WAN Planning and Deployment Planning and Deployment Lab Setup Guide |  |  |  |
| --- | --- | --- | --- | --- |
|  | Applicable to version 1.0 |  |  |  |
|  | Huawei Technologies Co., Ltd. |  |  |  |

Copyright © Huawei Technologies Co., Ltd. 2021. All rights reserved. All contents of this book are protected by copyright laws. Huawei owns all copyrights, except the contents quoted from other parties. Without prior written permission from Huawei Technologies Co., Ltd., no person or organization shall copy, distribute, reproduce, store, or use any content in the information retrieval system in any manner, or use it for any other commercial purpose. All rights reserved. Trademark Declaration and other Huawei trademarks are trademarks of Huawei Technologies Co., Ltd. All other trademarks or registered trademarks mentioned in this document are the property of their respective owners.

| Enterprise Business Talent Development Dept, Huawei Technologies Co., Ltd. |  |
| --- | --- |
| Address: | Huawei Headquarters, Bantian, Longgang District, Shenzhen Postcode: 518129 |
| Website: | https://talent.huaweiuniversity.com/ |
| Customer service email address: | certification@huawei.com |

Update History

| Version | Date | Description |
| --- | --- | --- |
| 1.0 | 2021-01-18 | New version |

Document Information

| Prepared by | Applicable Time Range | Applicable to |
| --- | --- | --- |
| Talent Ecosystem Development Dept | 2021-01-31~... | All those who want to learn Huawei Datacom certification |

catalogue

1.	Preface	4

2.	HCIP-Datacom-SD-WAN Planning and Deployment logical topology	4

3.	HCIP-Datacom-SD-WAN Planning and Deployment physical topology	5

4.	Equipment procurement	5

5.	Cable connection	5

6.	Device Configuration	7

6.1	iMaster NCE-WAN Installation and Deployment	7

6.2	ISP/Service SW Configuration	7

The configuration of establishing an MPLS/Internet network.	7

Configure the branch/headquarter LAN as follows:	10

6.3	VM and Service Configuration	14

7.	Other Precautions	16

8.	Attachment	16

Preface

After the HCIP experiment specifications, the laboratory equipment is determined, and the list of the laboratory equipment and physical connection diagram are generated.

This document provides guidance for regional offices, suppliers, HALPs, and network academies to purchase devices, install modules, connect cables, configure control devices, and construct a topology view. Based on the HCIP-Datacom-SD-WAN Planning and Deployment experiment manual, complete the HCIP-Datacom-SD-WAN Planning and Deployment experiment.

HCIP-Datacom-SD-WAN Planning and Deployment logical topology

- HCIP-Datacom-SD-WAN Planning and Deployment Logical Topology

HCIP-Datacom-SD-WAN Planning and Deployment physical topology

- HCIP-Datacom-SD-WAN Planning and Deployment Physical Topology

: This topology is a physical topology that contains all devices in the device purchase list.

Equipment procurement

For details about the equipment purchase list, see the attachment.

File name: HCIP-Datacom-SD-WAN Planning and Deployment Experiment Device List V1.0.xlsx

Purchase equipment according to the test requirements. If you have any questions, contact the following personnel:learning@huawei.com.

Cable connection

Connections must follow Figure 2. Table 1-1 lists the mapping between device names and models.

Mapping between device names and models

| Device Name | Device Model |
| --- | --- |
| HQ-R1 | AR6120 |
| HQ-R2 | AR6120 |
| Branch-A-R1 | AR6120 |
| Branch-A-R2 | AR6120 |
| Branch-B-R1 | AR6120 |
| Branch-C-R1 | AR6120 |
| ISP/Service SW | S5731-H48T4XC |
| iMaster NCE-WAN | RH2288*3 |
| Service Server | RH1288 |

After the connection is complete, check the port connection according to the following table.

Interface connection table

| Local Device | Local Interface | Peer device | Peer Interface |
| --- | --- | --- | --- |
| HQ-R1 | G0/0/0 | ISP/Service SW | G0/0/21 |
|  | G0/0/8 | HQ-R2 | G0/0/8 |
|  | G0/0/9 | ISP/Service SW | G0/0/11 |
| HQ-R2 | G0/0/0 | ISP/Service SW | G0/0/22 |
|  | G0/0/8 | HQ-R1 | G0/0/8 |
|  | G0/0/9 | ISP/Service SW | G0/0/12 |
| Branch-A-R1 | G0/0/0 | ISP/Service SW | G0/0/23 |
|  | G0/0/8 | Branch-A-R2 | G0/0/8 |
|  | G0/0/9 | ISP/Service SW | G0/0/13 |
| Branch-A-R2 | G0/0/0 | ISP/Service SW | G0/0/24 |
|  | G0/0/8 | Branch-A-R1 | G0/0/8 |
|  | G0/0/9 | ISP/Service SW | G0/0/14 |
| Branch-B-R1 | G0/0/0 | ISP/Service SW | G0/0/25 |
|  | G0/0/8 | ISP/Service SW | G0/0/16 |
|  | G0/0/9 | ISP/Service SW | G0/0/15 |
| Branch-C-R1 | G0/0/0 | ISP/Service SW | G0/0/26 |
|  | G0/0/8 | ISP/Service SW | G0/0/18 |
|  | G0/0/9 | ISP/Service SW | G0/0/17 |
| RH2288 - 1 | Eth0 | ISP/Service SW | G0/0/1 |
|  | Eth1 | ISP/Service SW | G0/0/2 |
| RH2288 - 2 | Eth0 | ISP/Service SW | G0/0/3 |
|  | Eth1 | ISP/Service SW | G0/0/4 |
| RH2288 - 3 | Eth0 | ISP/Service SW | G0/0/5 |
|  | Eth1 | ISP/Service SW | G0/0/6 |
| RH1288 - 1 | Eth2 (10G interface) | ISP/Service SW | XG0/0/1 |
|  | Eth3 (10G interface) | ISP/Service SW | XG0/0/2 |

Note: The 10GE port of the RH1288 can be connected to a switch using a 10GE high-speed cable.

Device Configuration

  - iMaster NCE-WAN Installation and Deployment

For details about how to deploy the iMaster NCE-WAN controller, see "Software Installation" and "Hardware Installation" in iMaster NCE-WAN Product Documentation. The controller is deployed on three RH2288 servers.

Ensure that the southbound IP address of the controller is 172.21.16.198/24, which is related to the subsequent switch configuration.

The internal communication VLAN of the controller is 192, and the communication network segments are 192.168.2.0 and 24.

The southbound and northbound IP addresses of the controller can be deployed on the same network segment. The communication VLAN is 172 and the communication network segment is 172.21.16.0/24.

VMs can be managed by Fusion Computer (three or fewer hosts can be used free of charge).

  - ISP/Service SW Configuration

The switch is an important core device in the SD-WAN experiment and is used to set up MPLS/Internet networks and branch/headquarter LANs.

The configuration of establishing an MPLS/Internet network.

sysname ISP/Service SW

#

vlan batch 172 192

#

ip vpn-instance HQ-Video

ipv4-family

#

ip vpn-instance Internet

ipv4-family

#

Interface GigabitEthernet0/0/1

port link-type trunk

port trunk allow vlan 172 192

#

Interface GigabitEthernet0/0/2

port link-type trunk

port trunk allow vlan 172 192

#

Interface GigabitEthernet0/0/3

port link-type trunk

port trunk allow vlan 172 192

#

Interface GigabitEthernet0/0/4

port link-type trunk

port trunk allow vlan 172 192

#

Interface GigabitEthernet0/0/5

port link-type trunk

port trunk allow vlan 172 192

#

Interface GigabitEthernet0/0/6

port link-type trunk

port trunk allow vlan 172 192

#

interface GigabitEthernet0/0/11

undo portswitch

ip binding vpn-instance MPLS

ip address 100.1.1.2 255.255.255.252

#

interface GigabitEthernet0/0/12

undo portswitch

ip binding vpn-instance Internet

ip address 100.2.1.2 255.255.255.252

#

interface GigabitEthernet0/0/13

undo portswitch

ip binding vpn-instance MPLS

ip address 100.1.2.2 255.255.255.252

#

interface GigabitEthernet0/0/14

undo portswitch

ip binding vpn-instance Internet

ip address 100.2.2.2 255.255.255.252

#

interface GigabitEthernet0/0/15

undo portswitch

ip binding vpn-instance MPLS

ip address 100.1.3.2 255.255.255.252

#

interface GigabitEthernet0/0/16

undo portswitch

ip binding vpn-instance Internet

ip address 100.2.3.2 255.255.255.252

#

interface GigabitEthernet0/0/17

undo portswitch

ip binding vpn-instance MPLS

ip address 100.1.4.2 255.255.255.252

#

interface GigabitEthernet0/0/18

undo portswitch

ip binding vpn-instance Internet

ip address 100.2.4.2 255.255.255.252

#

interface Vlanif172

ip address 172.21.16.254 255.255.255.0

#

interface Vlanif192

ip address 192.168.2.254 255.255.255.0

#

ip route-static 100.1.1.0 255.255.255.252 vpn-instance MPLS 100.1.1.1

ip route-static 100.1.2.0 255.255.255.252 vpn-instance MPLS 100.1.2.1

ip route-static 100.1.3.0 255.255.255.252 vpn-instance MPLS 100.1.3.1

ip route-static 100.1.4.0 255.255.255.252 vpn-instance MPLS 100.1.4.1

ip route-static 100.2.1.0 255.255.255.252 vpn-instance Internet 100.2.1.1

ip route-static 100.2.2.0 255.255.255.252 vpn-instance Internet 100.2.2.1

ip route-static 100.2.3.0 255.255.255.252 vpn-instance Internet 100.2.3.1

ip route-static 100.2.4.0 255.255.255.252 vpn-instance Internet 100.2.4.1

ip route-static vpn-instance Internet 172.21.16.0 255.255.254.0 172.21.16.198 public

ip route-static vpn-instance MPLS 172.21.16.0 255.255.254.0 172.21.16.198 public

#

Configure the branch/headquarter LAN as follows:

#

vlan batch 11 12 21 22 31 32 41 42 110 111 120 121 130 131 140 141 150 151 160 161

#

ip vpn-instance Branch-A-Video

ipv4-family

#

ip vpn-instance Branch-B

ipv4-family

#

ip vpn-instance Branch-C

ipv4-family

#

ip vpn-instance HQ-FTP

ipv4-family

#

ip vpn-instance HQ-Video

ipv4-family

#

interface Vlanif11

ip binding vpn-instance HQ-Video

ip address 192.168.11.1 255.255.255.252

#

interface Vlanif12

ip binding vpn-instance HQ-FTP

ip address 192.168.12.1 255.255.255.252

#

interface Vlanif21

ip binding vpn-instance Branch-A-Video

ip address 192.168.21.1 255.255.255.252

#

interface Vlanif22

ip binding vpn-instance Branch-A-FTP

ip address 192.168.22.1 255.255.255.252

#

interface Vlanif31

ip binding vpn-instance Branch-B

ip address 192.168.31.1 255.255.255.252

#

interface Vlanif32

ip binding vpn-instance Branch-B

ip address 192.168.32.1 255.255.255.252

#

interface Vlanif41

ip binding vpn-instance Branch-C

ip address 192.168.41.1 255.255.255.252

#

interface Vlanif42

ip binding vpn-instance Branch-C

ip address 192.168.42.1 255.255.255.252

#

interface Vlanif110

ip binding vpn-instance HQ-Video

ip address 192.168.110.254 255.255.255.0

#

interface Vlanif120

ip binding vpn-instance HQ-FTP

ip address 192.168.120.254 255.255.255.0

#

interface Vlanif130

ip binding vpn-instance Branch-A-Video

ip address 192.168.130.254 255.255.255.0

#

interface Vlanif140

ip binding vpn-instance Branch-A-FTP

ip address 192.168.140.254 255.255.255.0

#

interface Vlanif150

ip binding vpn-instance Branch-B

ip address 192.168.150.254 255.255.255.0

#

interface Vlanif160

ip binding vpn-instance Branch-C

ip address 192.168.160.254 255.255.255.0

#

interface GigabitEthernet0/0/21

port link-type trunk

port trunk pvid vlan 111

port trunk allow-pass vlan 11 111

#

interface GigabitEthernet0/0/22

port link-type trunk

port trunk pvid vlan 121

port trunk allow-pass vlan 12 121

#

interface GigabitEthernet0/0/23

port link-type trunk

port trunk pvid vlan 131

port trunk allow-pass vlan 21 131

#

interface GigabitEthernet0/0/24

port link-type trunk

port trunk pvid vlan 141

port trunk allow-pass vlan 22 141

#

interface GigabitEthernet0/0/25

port link-type trunk

port trunk pvid vlan 151

port trunk allow-pass vlan 31 to 32 151

#

interface GigabitEthernet0/0/26

port link-type trunk

port trunk pvid vlan 161

port trunk allow-pass vlan 41 to 42 161

#

ospf 1 vpn-instance HQ-Video

area 0.0.0.0

network 0.0.0.0 255.255.255.255

#

ospf 2 vpn-instance HQ-FTP

area 0.0.0.0

network 0.0.0.0 255.255.255.255

#

ospf 3 vpn-instance Branch-A-Video

area 0.0.0.0

network 0.0.0.0 255.255.255.255

#

ospf 4 vpn-instance Branch-A-FTP

area 0.0.0.0

network 0.0.0.0 255.255.255.255

#

ospf 5 vpn-instance Branch-B

area 0.0.0.0

network 0.0.0.0 255.255.255.255

#

ospf 6 vpn-instance Branch-C

area 0.0.0.0

network 0.0.0.0 255.255.255.255

#

interface Eth-Trunk1

trunkport GigabitEthernet 0/0/1

trunkport GigabitEthernet 0/0/2

port link-type trunk

port trunk allow-pass vlan 172 192

#

interface Eth-Trunk2

trunkport GigabitEthernet 0/0/3

trunkport GigabitEthernet 0/0/4

port link-type trunk

port trunk allow-pass vlan 172 192

#

interface Eth-Trunk3

trunkport GigabitEthernet 0/0/5

trunkport GigabitEthernet 0/0/6

port link-type trunk

port trunk allow-pass vlan 172 192

#

interface Eth-Trunk4

trunkport XGigabitEthernet 0/0/1 to 0/0/2

port link-type trunk

port trunk allow-pass vlan 110 to 111 120 to 121 130 to 131 140 to 141 150 to 151 160 to 161 172

  - VM and Service Configuration

VMs can be managed by Fusion Computer (three or fewer hosts can be used free of charge).

Terminals at the headquarters and branch sites are deployed on VMs. The VMs are deployed on the RH1288. The RH1288 uses two 10GE interfaces to connect to switches. All service traffic is separated by VLANs.

A total of seven VMs are required. Six VMs function as terminals in the headquarters or branch, and one VM functions as the email server.

Six VMs are deployed. Each VM has three NICs. One is a service NIC, one is a deployment NIC, and the other is an email NIC. The VLAN and planning are as follows:

| Specifies the VM name. | Network adapter 1 (service network adapter) | Network adapter 2 (deployment network adapter) | Network adapter 3 (mail network adapter) |
| --- | --- | --- | --- |
| Video Server | IP:192.168.110.1/24 GW:192.168.110.254 VLAN:110 | IP:192.168.1.254/24 GW: none VLAN:111 | IP:172.21.16.232/24 GW: none VLAN:172 |
| FTP Server | IP:192.168.120.1/24 GW:192.168.120.254 VLAN:120 | IP:192.168.1.254/24 GW: none VLAN:121 | IP:172.21.16.233/24 GW: none VLAN:172 |
| Branch-A-1-PC | IP:192.168.130.1/24 GW:192.168.130.254 VLAN:130 | IP:192.168.1.254/24 GW: none VLAN:131 | IP:172.21.16.234/24 GW: none VLAN:172 |
| Branch-A-2-PC | IP:192.168.140.1/24 GW:192.168.140.254 DNS:192.168.120.1 VLAN:140 | IP:192.168.1.254/24 GW: none VLAN:141 | IP:172.21.16.235/24 GW: none VLAN:172 |
| Branch-B-PC | IP:192.168.150.1/24 GW:192.168.150.254 DNS:192.168.120.1 VLAN:150 | IP:192.168.1.254/24 GW: none VLAN:151 | IP:172.21.16.236/24 GW: none VLAN:172 |
| Branch-C-PC | IP:192.168.160.1/24 GW:192.168.160.254 DNS:192.168.120.1 VLAN:160 | IP:192.168.1.254/24 GW: none VLAN:161 | IP:172.21.16.237/24 GW: none VLAN:172 |

One VM functions as the mail server. The plan is as follows:

| Specifies the VM name. | Network adapter 1 (mail network adapter) |
| --- | --- |
| Mail Server | IP:172.21.16.240/24 GW: none VLAN:172 |

The Video Server uses the VLC software to set up a video server.

The FTP server uses the FTP server software to set up the FTP server and deploy the DNS service (AAAA record: www.internetsw.com<->100.2.1.2).

The web service is replaced by the web configuration page of the switch.

Use Winmail to deploy the mail service on the mail server and add users. The user plan is as follows:

| Email Address | User name/password |
| --- | --- |
| ztp@sdwan.com | admin/Huawei@123 |
| hq@sdwan.com | admin/Huawei@123 |
| branch-a@sdwan.com | admin/Huawei@123 |
| branch-b@sdwan.com | admin/Huawei@123 |
| branch-c@sdwan.com | admin/Huawei@123 |

Other Precautions

It is recommended that the trainer explain the experiment materials and the students complete the experiment materials separately. If troubleshooting is required, the trainer can participate in the experiment materials.

To prevent residual configurations from affecting the experiment, you are advised to clear the configuration information saved on the device before shutting down the device after the experiment is complete. At the beginning of the experiment, ensure that the device starts from the empty configuration. Otherwise, clear the configuration and restart the device.

Attachment