Huawei Certification Training

HCIP-Datacom-WAN

Planning and Deployment

Lab Setup Guide

Version:1.0

Huawei Technologies Co., Ltd.

Copyright Huawei Technologies Co., Ltd. 2021. All rights reserved. No part of this document may be reproduced or transmitted in any form or by any means without the prior written consent of Huawei Technologies Co., Ltd. Trademark Declaration and other Huawei trademarks are trademarks of Huawei Technologies Co., Ltd. All other trademarks and trade names mentioned in this document are the property of their respective holders. Note: The purchased products, services and features are stipulated by the contract made between Huawei and the customer. All or part of the products, services and features described in this document may not be within the purchase scope or the usage scope. Unless otherwise specified in the contract, all statements, information, and recommendations in this document are provided without warranties, guarantees, or representations of any kind, either express or implied. The information in this document is subject to change without notice. Every effort has been made in the preparation of this document to ensure accuracy of the contents, but all statements, information, and recommendations in this document do not constitute the warranty of any kind, express or implied.

| Huawei Technologies Co., Ltd. |  |
| --- | --- |
| Address: | Huawei Industrial Base Bantian, Longgang, Shenzhen 518129 |
| Website: | http://e.huawei.com |

Huawei Certificate System

Huawei certification is based on the "platform + ecosystem" strategy and the new ICT technology architecture of "cloud-pipe-device" collaboration. Huawei certification is a three-type ICT technology architecture certification, platform and service certification, and industry ICT certification. It is the only certification system that covers all ICT technologies in the industry. Based on the learning and advanced requirements of ICT practitioners, Huawei certification is classified into three levels: engineer level, senior engineer level, and expert level. Huawei certification covers all ICT domains and complies with the ICT convergence trend. It is committed to providing a leading talent cultivation system and certification standards, cultivating new ICT talents in the digital era, and building a healthy ICT talent ecosystem.

HCIP-Datacom-WAN Planning and Deployment V1.0 is designed to train senior engineers with professional knowledge and skills in WAN bearer network scenarios in the datacom network field.

After passing the HCIP-Datacom-WAN Planning and Deployment V1.0 certification, you have a systematic understanding and mastery of WAN bearer networks and solutions, WAN bearer network architecture and typical technical applications, WAN VPN, MPLS TE, SR, SRv6, and WAN domain controller network management. Knowledge and skills such as analysis, WAN traffic control, WAN bearer network O&M and troubleshooting, and WAN bearer network design and practice. You will be able to use Huawei datacom devices to plan, deploy, and maintain enterprise WAN bearer networks. You can be a senior engineer of enterprise WAN bearer networks. Positions (customer manager, project manager, pre-sales engineer, post-sales engineer, O&M engineer, etc.)

Huawei certification helps you open the window of the industry and open the door to change, and stand on the top of the trend in the datacom field.

| Version | Release Date | author | Applicable Time Range | readers |
| --- | --- | --- | --- | --- |
| 01 | 2021 - 06 - 03 | Talent Ecosystem Development Dept | 2021-06-03-2023-06-03 | HALP Instructor |

Contents

Foreword	5

Introduction	5

1 Device procurement	5

1.1 Procurement list	5

2 Cable Connection	6

2.1 Network topology	6

2.1.1 Physical topology of the service network	6

2.1.2 Managing Physical Network Topology	7

2.2 Lab planning	7

3 Precautions	9

3.1 Use of non-standard equipment	9

3.2 Other Precautions	9

3.3 Device Initialization	9

4 Attachments	11

4.1 Related Attachments	11

Foreword

Introduction

《HCIP-Datacom-WAN Planning and Deployment V1.0 Lab Setup Guide》defines the experimental equipment, generates the list of the determined experimental equipment, and the physical connection diagram.

This document is intended to guide regions, suppliers, HALPs, and network institutes to purchase devices, install modules, connect cables, configure control devices, and build a topology diagram. Then, complete experiments based on the HCIP-Datacom-WAN Planning and Deployment V1.0 Lab Manual.

Device procurement

Procurement list

For the equipment purchase list, see the attachment.

File name: HCIP-Datacom-WAN Planning and Deployment V1.0 Equipment List

When purchasing equipment, pay attention to the requirements of the lab. If you have any questions, contact the channel management personnel of the Training Qualification Mgmt Dept by email.

Cable Connection

Network topology

Physical topology of the service network

Managing Physical Network Topology

Lab planning

The mapping between device names and models is as follows:

| Device Name | Device Model |
| --- | --- |
| PE1 | NetEngine 8000 M6 |
| PE2 | NetEngine 8000 M6 |
| PE3 | NetEngine 8000 M6 |
| PE4 | NetEngine 8000 M6 |
| P1 | NetEngine 8000 M6 |
| P2 | NetEngine 8000 M6 |
| SW1 | S5731-H24T4XC |

After the device is connected, check whether the port connection is correct according to the following table.

| Device Name | Interface | Peer device |
| --- | --- | --- |
| PE1 | GE0/0/0 | SW1 |
|  | GE0/3/0 | PE2 |
|  | GE0/3/1 | P1 |
| PE2 | GE0/0/0 | SW1 |
|  | GE0/3/0 | PE1 |
|  | GE0/3/1 | P2 |
| PE3 | GE0/0/0 | SW1 |
|  | GE0/3/0 | PE4 |
|  | GE0/3/1 | P1 |
| PE4 | GE0/0/0 | SW1 |
|  | GE0/3/0 | PE3 |
|  | GE0/3/1 | P2 |
| P1 | GE0/0/0 | SW1 |
|  | GE0/3/0 | PE1 |
|  | GE0/3/2 | PE3 |
|  | GE0/3/4 | P2 |
| P2 | GE0/0/0 | SW1 |
|  | GE0/3/0 | PE2 |
|  | GE0/3/2 | PE4 |
|  | GE0/3/4 | P1 |
| SW1 | GE0/0/1 | PE1 |
|  | GE0/0/2 | PE2 |
|  | GE0/0/3 | PE3 |
|  | GE0/0/4 | PE4 |
|  | GE0/0/5 | P1 |
|  | GE0/0/6 | P2 |
|  | -(subject to the actual environment) | iMaster NCE physical server |

Precautions

Use of non-standard equipment

Be sure to use the standard equipment list shown in the attachment when building the lab topology.

The use of non-standard equipment may have but are not limited to the following problems:

The command is not supported or the port name is inconsistent with that in the lab manual.

Some experimental effects cannot be presented.

Not compatible with later versions of lab manuals and textbooks

Other Precautions

It is recommended that the lecturer explain the teaching materials and students complete the teaching materials independently. If troubleshooting is required, the lecturer can participate in the teaching materials.

In addition, to avoid the impact of residual configurations on the experiment, students are required to clear the configuration information saved on the device after the experiment is complete and before the device is shut down. At the beginning of the experiment, ensure that the device starts from empty configurations. Otherwise, clear the configuration and restart the device.

In this experiment, the iMaster NCE physical server is connected to SW1 through GE0/0/20. In actual environments, the iMaster NCE physical server can be installed in various ways, and the number of physical servers is not fixed. The actual connection mode between the physical server and SW1 depends on the purchased environment.

Device Initialization

In this lab, SW1 connects to the iMaster NCE physical server and management interfaces (GE0/0/0) of all NE routers to implement communication between the southbound management plane of the iMaster NCE and NEs. In this lab plan, the southbound management plane of the iMaster NCE and NE GE0/0/0 reside on the same network segment (172.21, 17.0, and 24). They can communicate with each other through Layer 2. Therefore, you only need to assign them to the same VLAN on SW1.

To solve this problem, perform the following pre-configuration on SW1:

Create VLAN 17 and assign the interfaces connected to PE1-4, P1-P2, and iMaster NCE physical servers to VLAN 17.

System-view

#

sysname SW1

#

vlan 17

#

interface GigabitEthernet0/0/1

port link-type access

port default vlan 17

#

interface GigabitEthernet0/0/2

port link-type access

port default vlan 17

#

interface GigabitEthernet0/0/3

port link-type access

port default vlan 17

#

interface GigabitEthernet0/0/4

port link-type access

port default vlan 17

#

interface GigabitEthernet0/0/5

port link-type access

port default vlan 17

#

interface GigabitEthernet0/0/6

port link-type access

port default vlan 17

#

interface GigabitEthernet0/0/20

port link-type access

port default vlan 17

Attachments

Related Attachments